[image: image1.jpg]


 SEQ CHAPTER \h \r 1
       Brenda Coley 

        Superintendent

August 13, 2018

To Whom It May Concern:

The Russell County School District invites you to bid on the upcoming School Bus Bid.  Bids will be received until 11:00 a.m. EST, Monday, August 27, 2018 at the Russell County Board of Education at 506 14th Street, Phenix City, Alabama 36867.

Envelopes containing bids must be sealed and marked in the lower left-hand corner: RCBOE SCHOOL BUS BID, August 27, 2018, 11:00 A.M. EST.  All bid prices must be submitted on the attached bid proposal form and signed by an authorized representative of the firm.  BIDS MUST BE MAILED OR HAND DELIVERED ONLY.

A contract for the project will be put into effect by means of a purchase order executed by the Board after tabulations are compiled.

Bid is to be inclusive price with no further charges made against the Russell County School District.  Full payment will be made to the successful vendor(s) upon satisfactory completion of the entire shipment and delivery of items awarded.

Any questions regarding the specifications or the bid process should be directed to: Cody Patterson, CSFO (334)468-5544 or Ward Dudley, Transportation Director (334)408-4926

Your interest in submitting a bid to our board is appreciated.

Sincerely,

Ward Dudley





Cody Patterson

Transportation Director



Chief School Financial Officer
School Bus Bid 
School Bus Specifications

2019 model or newer

72 Passenger 
The school bus furnished shall comply with all current State and Federal Motor Vehicle Safety Standards in effect at the time of bid.  Additionally, the Russell County Board Of Education is requiring the following additional specifications.  Any exceptions to these specifications should be concisely noted in the space provided.  Equivalency of any exceptions to the items requested will be solely at the discretion of school system representatives.  Please note that Russell County Schools will review & consider any and all below listed exceptions taken by participating vendors.
Please note that bidder must have a fully licensed in-state sales and service facility and must have bus (es) state inspected and Pre-Delivery Inspected (PDI) at this facility prior to delivery.  Buses shall be delivered to the County School Bus Shop with in the prescribed time allowed in the bid.  At the time of delivery driver orientation training must be conducted so that driver’s are fully prepared to operate the equipment. The Board will not accept factory direct deliveries.   Bids will only be accepted from established firms in Alabama.
DELIVERY MUST BE MADE WITHIN 150-180 DAYS OF RECEIPT OF PURCHASE ORDER.  
Bidders Name__________________________________________________________________________

Address______________________________________________________________________________

Make and Model of Bus__________________________________________________________________
Delivery Date__________________________________________________________________________

Company Representative, Sign_____________________________________________________________

Type C 72 Passenger (four or more)


Price Per Unit $____________________________

The Russell County Board of Education reserves the right to award this bid on factors other than low price including:  past performance of dealer, proven ability to support the product bid and parts compatibility with existing fleet. The Russell County Board of Education also reserves the right to add additional buses at a later date should the supplier agree to honor the same bid price.  In addition to all specifications listed below, buses must meet all State and Federal specifications. 
The Russell County Board of Education also reserves the right to use a Life Cycle Costing Analysis to determine the best overall value offered by each vendor. 
OVERALL DIMENSIONS TO BE AS FOLLOWS:

BIDDER EXCEPTIONS:

· Minimum interior height – 78”




______________________________

· Minimum 12” x 25” window openings



______________________________
· Acoustical Headliner                                                                                  ______________________________
BODY DATA
Please indicate below which type of electrical system is to be provided.

Fully Integrated Multiplex (Body & Chassis)
_____________

Partially Integrated Multiplex (Chassis Only)
_____________

Separate Multiplex Systems (Body & Chassis
_____________

Bus must be equipped with a Driver’s Pre-Trip Light Inspection System.  System shall systematically power all lights individually, while the driver performs his/her pre-trip inspection.  System shall also activate stop arm, crossing arm and horn.  This system is to be operated by a switch, conveniently located in the driver’s compartment.
Exception:________________________________________________________________________________

There shall be 4 applied rub rails on each bus, four on each side.   Rub rails shall be painted black.  Rub rails are to be sealed. Buses shall be lettered and numbered. This will be determined at bid award.
Exception:________________________________________________________________________________

Bodies shall be mounted in such a manner as to prevent shifting. At a minimum, two bolts in lieu of one shall be used at each mounting bracket to prevent a pivot point.  Rubber mounting pads reinforced with fiber shall be used to cushion the body on the chassis at every floor member and shall be held in place permanently with mechanical fasteners.

Exception: ________________________________________________________________________________

There shall be a
minimum of 2” thick blanket-type polyester insulation mounted in the ceiling.  
Exception: ________________________________________________________________________________

Bus shall be equipped with air stop sign and crossing arms.

Exception: ________________________________________________________________________________

The front heater shall be multi speed fresh air type, 90,000 BTU. There shall be two 50,000 BTU rear heaters. One in midpoint of bus and the third one in the rear RH corner. Heater motor and fan mechanism shall be serviceable without disturbing heater core.  Rear/Under-seat heaters must be equipped with a kick guard.
Exception: ________________________________________________________________________________
Rear view exterior mirrors shall be heated and remote type.  Cross over mirrors shall be heated. All mirror wiring shall be run through the inside of the mirror brackets in order to be hidden from view.
Exception: _________________________________________________________________________________

Driver’s fan must be located above the driver side window to the left of the driver.

Exception: _________________________________________________________________________________

Roof will be white.  All seat frames, heaters, and heater ducts shall have a baked-on black powder coating for scratch resistance.  Wheels will be black. 
Exception: ________________________________________________________________________________

Service door shall be manually operated. An exterior service door pull shall be provided to aid in opening and closing of door. Must have a positive latch vandal lock to keep door closed when bus is parked overnight.

Exception: ________________________________________________________________________________

There shall be an emergency equipment compartment located above the windshield. There shall be an additional glove storage box compartment mounted above driver’s window to hold route essential information and the recording system. Storage box shall be kept closed by use of a push button latch and lock.  The glove box door shall include a spring hold prop.
Exception: ________________________________________________________________________________





Floor seams shall be sealed with a waterproof sealer and covered with a 1” wide, 16 gauge metal stripping or molding.  The floor along the side walls shall be covered with molding so that dirt cannot readily work underneath the edges.  Sub floor shall be plywood, 5/8” marine grade (screwed down), for sound and temperature insulation.  Floor covering shall be black in color for a clean appearance.  There shall be an additional (RH) entrance door grab rail assit to aid passenger entry and exit.
Exception: ________________________________________________________________________________

Body shall be equipped with either rubber fenders or metal fenderettes.

Exception: ________________________________________________________________________________







All exterior lights with the exception of the headlight shall be LED. Reflectors shall be applied with mechanical fasteners.
Front and rear eight-way warning lights shall be serviceable lights with replaceable diodes to delay the need for complete light removal and replacement. Warning flashers must be Weldon 7000 or comparable with a 3-year warranty. Step well light shall be LED.
Exception: ________________________________________________________________________________

All interior dome lights are to be LED style.

Exception: ________________________________________________________________________________

Windows must be tinted with a maximum of 28% light transmission tint. Windows must include a 12” clear opening.

Exception: _________________________________________________________________________________
Driver’s seat shall be an air ride suspension type with adjustable seat back. Seat will recline and include adjustable lumbar support. Seat to be cloth.
Exception: ________________________________________________________________________________





All units shall be keyed a like.
Exception: ________________________________________________________________________________

All units shall have front and rear tow hooks.

Exception: ________________________________________________________________________________
Bus must be air-conditioned. Output must be a minimum of 130,000 BTUs utilizing two interior free blow evaporators, two condensers. Evaporators shall be mounted in the bulk head for rear unit. Front evaporator shall be side mount EM-1 and IW-10 in the rear or equivalent. Condensers mounted in the skirt is preferred but shall be mounted in such a way to obtain proper air flow and resist dirt, mud, and road splash from coming in contact with condenser. Condenser must be shrouded to the skirt. A reciprocating piston, transit-style air condition compressor in lieu of swashplate, automotive style compressor(s) is to be installed, TM-43 preferred. To maintain fleet uniformity, and keep our parts inventory to a minimum, system brand shall be ACC or an approved equal. Bus to be equipped with dash a/c and tied in with the front and rear air condition units. Must have electrical compartment mounted on LH side near battery compartment. Digital control switches are required.
CAMERA REQUIREMENT

Bus must be equipped with a Digital Camera System meeting state requirements. Seon, model Tropper 4 preferred. The recording device must be mounted in the compartment over the driver’s window. Under seat mount will not be acceptable. The camera wires must be ran behind panels where ever possible in order to keep exposed wires out passenger compartment. Camera heads are to be color and night vision.
Exception: ________________________________________________________________________________
Bus will be equipped with an internal PA system with a min. of 6 speakers. The PA mike will be a Plug in type mike with volume controlled by the radio. Bus will be installed with 2 way radios mounted in a position that is approved by the system.  Manufacturer will be responsible for the purchase and installation of the radios and insuring that they are tuned to the proper frequency.
Exception: __________________________________________________________________________________

CHASSIS DATA

Alternator to be Leece Neville or Delco 270 amps and shall be pad mounted.
Exception: _______________________________________________________________________________

Minimum 55 degree turning radius is desired.  Please list turning radius____________.

Exception: _______________________________________________________________________________

Bus is to be equipped with cruise control and fast idle switches.
Exception: _______________________________________________________________________________

Axles and suspension to be 10,000 lb. front and 21,000 lb. rear.  Front axle shall be equipped with oil-lubricated

bearings.  Buses to include rear air suspension. The suspension shall include (1) rubber bushed adjustable torque rod, and (2) heavy duty shock absorbers. The system shall incorporate dual load sensitive height control values to maintain a constant frame height and level the chassis side to side regardless of load. Rear axle ratio to be 5.22. For softer ride, front springs shall be parabolic type with a minimum of 2 (heavy-duty) shock absorbers.
Exception: _________________________________________________________________________________

Batteries Group 31 threaded studs, minimum of two (2) to be maintenance free and shall be a minimum 1900 CCA.  Batteries must be mounted on a (stainless steel) roller bearing slide out tray with spring loaded pin locks.  A battery cut-off switch shall be included, and shall be mounted in the battery box compartment. Group 8D or 4D batteries will not be accepted.
Exception: ________________________________________________________________________________



Air compressor preferred to be 18.7 CFM and shall be gear driven (belt driven compressors are not acceptable).  Brakes must be equipped with a heated air dryer with automatic purge system.  Petcock drain valves shall be mounted in the battery compartment for ease of access. Air tank capacity to be a minimum of 6,400 cubic inches. Air lines to be color coded and have quick release connectors.  Drum Brakes Only.

Exception: ________________________________________________________________________________




Buses to be equipped with extended life halogen headlights with amber daytime running lights.

Exception: ________________________________________________________________________________


Gauges to also include a tachometer, an hour meter, and trip meter and transmission temperature gauge.   Instrumentation panel should be able to provide diagnostic capabilities for body and chassis electrical system.  A digital clock shall also be installed in an area readily convenient for the driver.  
Exception: _________________________________________________________________________________
Engine to be in-line 6 cylinder, (V8 diesels are not acceptable), minimum 230 H.P. with a minimum of 660 ft. lbs. of torque.  It is preferred that the engine offered is designed to require a single turbocharger.  Engine will include an alarm system to monitor low oil pressure and high water temperature and will include check gauges and telltale light and buzzer. Engine shall come equipped with a fuel primer pump. Engine shall be equipped with an intake heater and block heater with an easy access plug mounted conveniently in the front bumper... Shall include fast idle and cruise control. Shall have exhaust brake system.  Cummins motor preferred.
Exception: _________________________________________________________________________________
Exhaust shall exit out the driver’s side at the very most rear portion of the unit.

Exception: ________________________________________________________________________________

Fuel tank size to be a minimum 100 gallons, and shall be mounted between the frame rails.  Fuel door shall include a latch lock and be located on the right side of the bus. The fuel/water separator will be heated and located in the engine compartment.
Exception: _________________________________________________________________________________

Tires are to be 11Rx22.5 14 ply standard tubeless radials with 18/32 tread depth.  Tires shall be Goodyear, Michelin or Bridgestone. Off brand tires are not acceptable.  Wheels to be 8.25” x 22.5” painted black. Front wheel bearings shall be oil bath type.
Exception: ___________________________________________________________________________________




Steering column shall tilt and telescope.
Exception: ________________________________________________________________________________
Bus dealer must provide an on-site service truck with an engine certified technician to perform on-site warranty work.

Exception: ________________________________________________________________________________
In addition to standard service, warranty and support, bidder shall provide on-line customer support for resource access for manuals, parts ordering and warranty, and service bulletins. Successful bidder shall provide one (1) complete set of technical manuals. These manuals shall be the ones published by the manufacturer of the components used in the production of the bus. These manuals shall cover engine, transmission, electrical schematics etc….and they shall cover diagnostic/repair procedures.
WARRANTY DATA




Preferred chassis warranty should be as follows and warranty certificates must be included with bid:
Engine - 5 years / 100,000 miles, must include electronic 

______________________________



Components (2 years towing)



______________________________
Driveline - 4 years / unlimited mileage



______________________________

Transmission - 7 years / unlimited mileage, must include

______________________________



Electronic components (5 year towing)


______________________________

Transmission warranty shall include Transynd Fluid


______________________________

at initial fill at factory.
Chassis – 3 years/50,000 miles bumper to bumper                                     ______________________________







      
Towing (other than engine and trans.) – 6 months


______________________________
Body extended warranty:

3 years/50,000 miles                                                                                          $______________________________

Extended Warranty engine:

     10 years/200,000 miles                                                                                 $______________________________

Extended transmission warranty:                                                                   
       7 years/Unlimited miles                                                                              $______________________________
Vendors, shall submit with their bid a complete price page menu, breaking out a complete set of diagnostic software and subscription prices, along with pricing for cables and data-link tools needed to hook to the bus for reading such software.  This submission shall be separate from the Bid Price Page.

Kenneth Barnes


Jerry Wayne Carpenter


Dillie Elliot


Keith Mitchell


Tommy Pugh


Eugenia Upshaw


Joseph Williams





RUSSELL COUNTY BOARD OF EDUCATION


506 14TH STREET


PO BOX 400


PHENIX CITY, AL 36868-0400


TELEPHONE: (706)321-2224      FAX: (334)448-8314


https://myrcsd.org/











