City of Lebanon

Request for Proposal

RFP # 18-LIGHTING 09/04
*City of Lebanon reserves the right to reject all or any part of any RFP*

The City of Lebanon is exempt from State Tax and Federal Exercise Tax.

NO PROPOSALS RECEIVED AFTER CLOSING WILL BE ACCEPTED.

The City of Lebanon invites sealed proposals for lighting, in accordance with the specifications and bidding instructions set forth in this Request for Proposals. Bids will be received by the Purchasing Department, 200 N. Castle Heights Avenue, Lebanon, Tennessee 37087, until 10:00 a.m. local time, September 4, 2018, at which time they will be opened and read aloud.

Please identify as a sealed proposal with stamped on the outside of the envelope RFP #18-LIGHTING 9/04”

Any questions in regard to this proposal should be directed to Lisa Lane, Purchasing Agent, (615) 443-2802.

A copy of the bid tabulation will be available upon written request to the Purchasing Department.

Any manufacturers’ names, trade names, brand names, or catalog numbers used in the specifications are for the purpose of describing and establishing general quality levels.  Such references are not intended to be restrictive.  Since the “City” does not wish to rule out other competition and equal brands or makes, please remember “ANY APPROVED EQUAL”.  HOWEVER, IF A PRODUCT OTHER THAN SPECIFIED IS BID, IT IS THE VENDORS RESPONSIBILITY TO PROVE TO THE “CITY” THAT SAID PRODUCT IS EQUAL TO OR EXCEEDS THE QUALITY OF THE SPECIFICATIONS LISTED FOR ANY ITEM.

The City of Lebanon is an equal opportunity entity and does not discriminate on the basis of age, race, sex, national origin, religion or disability in admission to, access to, or operations of its programs, services, activities, or in its awarding of such bids.
1. SCOPE AND CLASSIFICATION

1.1 Scope:  The intent of this Request for Proposals and resulting contract is to provide the City of Lebanon with information concerning lighting projects.

1.2 Classification:  The lighting specifications shall be of the following types either “equal to “or “better than”.

2. SPECIFICATIONS

PROJECT #1:  LOCATION:  WASTEWATER PLANT:
MATERIAL SPECIFICATIONS/LIGHT #1
· Quantity:  30-Valiant LED Fixtures
· Series:  AVPL2
· Performance Package: 30LEDE70 30 Chips, 700 mA Driver, 66 input watts
· Ballast & Lamp Included

· Voltage: MVolt Multi-volt 120-277V
· Color Temperature:  4K 400K
· Distribution:  R5 Type V
· Optics:   AY Acrylic (Prismatic)

· Options to Include:  Photocontrol:  (blank) 3 pin NEMA Photocontrol Receptacle (standard)

MISCELLANEOUS NOTES
· Colors:  Will specify color when ordering.
· Taller cupola cover (RCC) is required when used with ROAM or other similar wireless monitoring control systems
· Standard failure mode=”Fail On”
· Photocontrols supplied with ANSI Standard Turn-On levels
· XL option is required
· Must ship with unit, field installed and wired
· Required when using ROAM or other similar wireless monitoring control systems.
MATERIAL SPECIFICATIONS/LIGHT #2

· Quantity:   4-Autobahn LED Roadway Fixtures
· Series:   ATBM
· Performance Package:  16,600 lumens
· Ballast & Lamp Included if applicable
· Voltage:   Multi-volt, 120-277V
· Optics:  R3 Roadway Type III
· Mounting:  (blank) 2 bolt mounting
· Paint:  Will be chosen when ordering
MATERIAL SPECIFICATIONS/LIGHT #3

· Quantity:   8-Wallpack IV Glass LED

· Series:  W4GLED

· LED’s:  20C=20 LED (Two Engines)

· Drive Current:
  1000=1000mA (1A)

· Color Temperature:   40K=4000K
· Distribution:  T3M=Type III medium

· Voltage:  MVolt=Multivolt 120.208.240.277

· Control Options:  PER=NEMA Twist-Lock Receptacle Only (no controls)

· Finish:  GYSDP=Gray

· System Watts:  72Watt

· Lumens:  7027, B-1, U-3, G-4, LPW-97

· Ballast and lamp included

NOTES:

· MVOLT driver operates on any line voltage from 120-277V (50/60 Hz). Specify 120, 208, 240 or 277 options only when ordering with fusing (SF, DF options) or photocontrol (PE).
· Single Fuse (SF) requires 120, 277 or 347 voltage option.  Double fuse (DF) requires 208, 240 or 480 voltage option

· Casting should be pre-drilled for guard.  Wire Guard must ship with fixtures.

MATERIAL SPECIFICATIONS/LIGHT #4
· Quantity:   2 LED Wall Luminaires

· Series:   TWS LED

· Performance Package:  P1-1476 lumins

· Color Temperature:  50K=5000K1

· Voltage:  120 volt

· Control Options:  PE Photoelectric cell, button type

· Finish:  Dark Bronze

· Ballast and lamp included

· Width:   6-3/4”   Height:   10-7/8” Depth   5-5/16” Weight   3.19 lbs.

PROJECT #2:   LOCATION:  WATER PLANT

MATERIAL SPECIFICATIONS/LIGHT #1

· Quantity:  11-Valiant LED Fixtures

· Series:  AVPL2

· Performance Package: 30LEDE70 30 Chips, 700 mA Driver, 66 input watts

· Ballast & Lamp Included

· Voltage: MVolt Multi-volt 120-277V

· Color Temperature:  4K 400K

· Distribution:  R5 Type V

· Optics:   AY Acrylic (Prismatic)

· Options to Include:  Photocontrol:  (blank) 3 pin NEMA Photocontrol Receptacle (standard)

MISCELLANEOUS NOTES

· Colors:  Will specify color when ordering.
· Taller cupola cover (RCC) is required when used with ROAM or other similar wireless monitoring control systems
· Standard failure mode=”Fail On”
· Photocontrols supplied with ANSI Standard Turn-On levels
· XL option is required
· Must ship with unit, field installed and wired
· Required when using ROAM or other similar wireless monitoring control systems.
MATERIAL SPECIFICATIONS/LIGHT #2

· Quantity:  4-Wallpack IV Glass LED

· Series:  W4GLED

· LED’s:  20C=20 LED (Two Engines)

· Drive Current:
  1000=1000mA (1A)

· Color Temperature:   40K=4000K

· Distribution:  T3M=Type III medium

· Voltage:  MVolt=Multivolt 120.208.240.277

· Control Options:  PER=NEMA Twist-Lock Receptacle Only (no controls)

· Finish:  GYSDP=Gray

· System Watts:  72Watt

· Lumens:  7027, B-1, U-3, G-4, LPW-97

· Ballast and lamp included

NOTES:

· MVOLT driver operates on any line voltage from 120-277V (50/60 Hz). Specify 120, 208, 240 or 277 options only when ordering with fusing (SF, DF options) or photocontrol (PE).

· Single Fuse (SF) requires 120, 277 or 347 voltage option.  Double fuse (DF) requires 208, 240 or 480 voltage option

· Casting should be pre-drilled for guard.  Wire Guard must ship with fixtures.

PROJECT #3:   LOCATION:  SQUARE PROJECT


MATERIAL SPECIFICATIONS:

· Quantity:  4-    Holophane:GVP15H LMP CSC30 WDA22L6J17P05BK-PROVS FGIUL, FG, 50A RECEP (4) BB36 AB RFD173306
· NOTES:
· Max Wattage:  150Watt HPS

· GranVille Premier (GVP):  150 Watt High Pressure Sodium, Mogul Base, Multi-tap 120, 208, 240, 277V

· Factory Wired for 120 volt

· UL Only

· Modern Fluted Swing Open Design, Black, Symmetric

· No Ribs and Bands

· 2-Clear Acylic 3” Finial

· No Trim Finish and Clear or No Finial with Sylvania 150HP Clear Mogul Lamp

· Cincinnati Series Crossarm (CSC): CSC30 (Two at 180), CA, Black

· Wadsworth Aluminum Pole (WDA):  22 feet, L6J SiteLink Shaft 

· 17 inch Round Base for use with L6J Shaft

· 3x4 Tenon with 3 Receptacle Provisions

· Powder Coat Paint Finish, Black

· GFI Receptacle with Large In-Use Cover, Holophance Black

· GFI Receptacle with WP while closed cover, HLP Black 

· 50A 125/250V 3 wire (NEMA 10-50R) receptacle with weatherproof while closed cover, Holophane, Black 

· 1”x36” Anchor bolts Include
· Include bolt pattern with bolts and poles

· Sitelink break-away banner arms
· 36” Long

· 1.25” diameter pipe with half-sphere finial

· Tracnuts must be included, Holophane Black

Square Pedestals:

Milbank CP-16PadMount-Calt 16” Pad Mounted Base-Quantity-2
Millbank CP3B0211145EBKSP1 Commercial Pedestal-Quantity-2

MISCELLANEOUS PARTS FOR SQUARE PROJECT:

QUANTITY
DESCRIPTION
1000 ft.

SCH40 2” 10ft. PVC Conduit

1200 ft.

SS-THHN 3/0 BLACK 19STR CU 500R
600 ft.

SS THHN 2/0 Black 19STR CU 500R

600 ft.

SS THHN 4 Black 19STR CU 500R

4


HOM816M 200PFTRB HOM Outd 200A8/16ft LUG

4


B200 LDCNTR BLTON Hub 2”

1000ft.

SS THHN 2 Black 19str 2500R

500ft.

SS THHN 3 Black 19 STR CU 2500R

500 ft.

SS THHN 6 Green 19STR CU 500R

1


Miscellaneous Parts

QUANTITY
DESCRIPTION

UNIT COST

TOTAL
41 

Valiant LED Lights w/Ballast &



Lamps



$_____________
$_____________

4

Autobahn LED Roadway Fixtures &



Lamps



$_____________
$_____________

12

Wallpack IV LED Glass Fixtures &

Lamps

 

$_____________
$_____________
2

TWS LED Wall Luminaires &



Lamps



$_____________
$_____________

4

Holophane Lamps 150w HPS fixtures &



Lamps



$_____________
$_____________

4

22’ Aluminum Poles with bolts &



Patterns


$_____________
$_____________

4

Cincinnati Crossarms with bolts






$_____________
$_____________

4                      Round Bases 17” 

$_____________
$_____________

4

GFI Receptacles with large cover






$_____________
$_____________

4

GFI Receptacles with WP with cover






$_____________
$_____________

1000 ft.
SCH40 2” PVC Conduit
$_____________
$_____________

1200 ft.
SS THHN 3/0 Wire

$_____________
$_____________

600 ft.

SS THHN 2/0 Black Wire
$_____________
$_____________

600 ft.

SS THHN 4 black CU Wire
$_____________
$_____________

4

HOM Outdoor 16ft lug
$______________
$_____________

4

B200 LDCNTR Hub 2”
$______________
$______________

1000ft.

SS THHN 2 black 19 str
$_____________
$______________

500ft.

SS THHN 3 Black 19 str
$______________
$______________

500ft.

SS THHN 6 Green 19 str
$______________
$______________

1

Miscellaneous Parts

$______________
$______________



Freight
$________________________



Total

$________________________

3. WARRANTY 

Bidders shall state the warranty period on the bid pricing form.  

4. DELIVERY

The vendor must state their estimated time of delivery/pickup.  
_______________________________
5. EXCEPTIONS

The above are minimum specifications.  If there are any exceptions to these specifications, please make any notations to the right of that spec. and the exceptions will have to be approved by the department.  

6. BID PRICE

TOTAL BID $_______________________

     
6.2    Delivery    _____________________

“By submission of this bid, each bidder and each person signing on behalf of any bidder certifies, and in the case of a joint bid each party thereto certifies as to its own organization, under penalties of perjury, that to the best of its knowledge and belief that each bidder is not on the list created pursuant to §12-12-103.
Company Name and Authorized Representative

______________________________________________________________

Address and Contact Phone Number

PAGE  
2

