Addendum #1

Date: November 17, 2021

Rockwood School District – RFP No. RFP1221TRANSBUS

Title: New School Buses – Type C, 77 Passenger

The Request for Proposals (RFP) for the above-named project is amended as noted in this Addendum.

The requirement for a 273” wheelbase has been removed from the specifications. The district is interested in a true 77 passenger bus.

The requirement for a message display center has been removed from the specifications.

Questions/Answers

- Can the district clarify the requested engine specification along with the governed speed the expected revolutions per minute (RPM) at the requested 350 HP @368 lb. ft. Torque with a speed limit set at 75 mph? V-8 unleaded, standard 87 octane fuel @3900 RPM

- Will Rockwood accept the PSI 8.8 liter gasoline engine if pricing is competitive along with its proprietary specifications? Yes

- Nothing is stated about engine warranty coverage.  Please clarify engine warranty coverage? Minimum 5-year warranty

- Will Rockwood accept the school bus leading transmission, Allison 2500? Yes

- Nothing is stated with the transmission warranty coverage. Please clarify warranty coverage term? Minimum 5-year warranty

- Will 10,000-lb. front axle & suspension be acceptance? Yes

- Would 6.14 RAR be acceptable?  Yes

- Will a 210 AMP Leece Neville alternator be acceptable? No, due to AC a 325 Leece Neville alternator will be required.

- Will it be acceptable for the bus to use the standard setup with Cowl and headlight? Yes

- Please clarify a body size? True 77 passenger bus body , not a 71-passenger body with seating for 77.

- Will 78” headroom be acceptable? Yes

- Will a Safe Fleet roof hatches be acceptable? Yes, brand does not matter. Proposed hatches must be waterproof.

- Will a 7” clear LED backup be acceptable? Yes, the reverse light should be a minimum of 4”.

- Can the Kentucky Pole Test and Colorado Rack be exempt from this requirement? Proposed buses must meet or exceed all required Missouri state specifications.

- Does the district have a recommended alternator to assistance with additional electric need to power the added equipment? 320 AMP