Invitation to Bid

The City of Winchester is accepting sealed bids for the resurfacing of the of the indoor and outdoor swimming pools at the David R. Bean Swimplex, 211 Old Estill Springs Road, Winchester, TN 37398.  Specifications can be picked up at Winchester City Hall, 7 South High Street, Winchester, TN, Monday thru Friday from 8:00 am until 4:00 pm, or contact Darrell Jolley at darrelljolley@winchester-tn.com.

Sealed bid deadline is Tuesday, September 5, 2017 at 10:00 am with the bid opening immediately following at Winchester City Hall.  For more information contact Darrell Jolley at 931-581-1034 to schedule an appointment.

The City of Winchester reserves the right to refuse any and all bids.

OUTDOOR POOL

# Work:  Re-Surface –Outdoor Swimming Pool - The sequence of the work to be performed, herein listed, may not be in the order as listed below due to outside influences of weather, scheduling, etc.

# Drain pool as needed.

# Saw cut under perimeter tile and around permanent interior fixtures or fittings.

# Remove plaster from pool perimeter below tile and around all other interior fixtures

# Or fittings to create a 4” – 6” area to allow new plaster to roll with proper thickness onto

# all matching surfaces.

# Knock out “hollow spots” in existing plaster surface and repair.  Note:  Hollow spots cannot

# be properly identified and assessed until pool is drained.  Should hollow spots exceed 10%

# of the total area of the pool, Owner will be notified

# Pressure wash

# Acid clean to etch

# Rinse and neutralize wastewater as required.

# Apply polyurethane to interior fittings and fixtures, to provide additional security to areas from

# possible leakage.

# Apply “Scratch Kote” or (appropriate bonding agent depending on selection of finish) to pool interior as a means of additional bonding for the new plaster.

# surface.  Scratch Kote is a cemetitious agent.

# Install 2” black tile on steps and risers to health department regulations.

# Install new plaster

# Fill Pool

# Activate filtration system

# Balance water chemistry (Owner to pay for chemicals or provide chemicals)

# Repairs:  No Mechanical Repairs Included

# Materials:

# Contractor shall provide all materials, supplies, equipment and services necessary for the complete performance of the work herein described.  Any exclusions shall be noted in Section 8.

# Completion of Project:

# Prefer the work to be completed during the months of February -April 2018.

INDOOR POOL

# Work:  Re-Surface –Indoor Swimming Pool and Remove/Replace Tile for Six (6) Racing Lanes:  The sequence of the work to be performed, herein listed, may not be in the order as listed below due to outside influences of weather, scheduling, etc.

# Drain pool as needed.

# Saw cut under perimeter tile and around permanent interior fixtures or fittings.

# Remove plaster from pool perimeter below tile and around all other interior fixtures

# Or fittings to create a 4” – 6” area to allow new plaster to roll with proper thickness onto

# all matching surfaces.

# Chip out and remove old racing lane tiles for (6) racing lanes.

# Install new racing lane tile

# Knock out “hollow spots” in existing plaster surface and repair.  Note:  Hollow spots cannot

# be properly identified and assessed until pool is drained.  Should hollow spots exceed 10%

# of the total area of the pool, Owner will be notified

# Pressure wash

# Acid clean to etch

# Rinse and neutralize wastewater as required.

# Apply polyurethane to interior fittings and fixtures, to provide additional security to areas from

# possible leakage.

# Apply “Scratch Kote” or (appropriate bonding agent depending on selection of finish) to pool interior as a means of additional bonding for the new plaster surface. Scratch Kote is a cementitious agent.

# Install 2” black tile on steps and risers to health department regulations.

# Install new plaster

# Fill Pool

# Activate filtration system

# Balance water chemistry (Owner to pay for chemicals or provide chemicals)

# Repairs:  No Mechanical Repairs Included

# Materials:

# Contractor shall provide all materials, supplies, equipment and services necessary for the complete performance of the work herein described.  Any exclusions shall be noted in Section 8.

# Completion of Project:

# Prefer the work to be completed during the months of February -April 2018.