INVITATION TO BID
HANAHAN RECREATION AND PARKS 
PICNIC SHELTER
FOR THE CITY OF HANAHAN, SC

COH# - 22216

The City of Hanahan, SC is seeking bids from qualified vendors for the purchase of one (1)      20’ x 25’ picnic shelter kit.  The selected vendor shall supply all materials and shop drawings for the installation of the shelter.  Please see attached specification sheet for desired shelters.

See attached Specifications Sheet.

Bids will be received by the City of Hanahan, UNTIL 12:00 P.M. – March 1st, 2016. 
Submittals may be mailed, emailed, or hand-delivered to:

City of Hanahan Recreation & Parks
1255 Yeamans Hall Rd.
Hanahan, SC 29410

Attn. Picnic Shelter Bids
rmoneymaker@cityofhanahan.com

For information please contact:
Randy Moneymaker, Recreation and Parks Director
3100 Mabeline Rd.
Hanahan, SC 29410
(843) 266-0723
(843) 266-0725 Fax
rmoneymaker@cityofhanahan.com

SPECIFICATIONS

- Shelter design shall be (1) 20’ x 25’.

- Lumber shall be #1 grade kiln dried, with maximum 15% moisture content, southern yellow pine graded to meet the requirements of Standard Specifications for Structural Glued Laminated Timber.

- All treated components must be treated prior to gluing.  All treated components with ground contact must be treated prior to gluing, at .6lbs. per cubic foot with CCA.

- Adhesives shall be water proof.

- All columns and curved tapered beams shall be pressure treated.  Beams shall be fabricated with #1 grade kiln dried, with 15% moisture content, Southern Yellow Pine.

- The roof deck shall be two-inch #1 Grade, single tongue and groove with V-joint bottom face, kiln fried, o an average of 15% moisture content, southern yellow pine.  Deck shall be finger-jointed (glued) to full lengths required for spanning three or more supports.  2” x 6” fascia, southern yellow pine, #1 SPIB Grade, pressure treated in accordance with AITC 109.

- Standards. Galvanized 16d nails shall be provided.

- 30 Year architectural shingles with one layer of #30 felt.  Color shall be charcoal.  Galvanized roofing nails for shingles and felt should be provided.

- All steel and hardware for beam and column connections shall be provided.  Steel connections to be prime painted with rust inhibitor paint.  Hardware to be electrostatic zinc plated.

- Exposed faces of glulam members to receive one coat of factory-applied clear penetrating sealer.
Members shall be wrapped for shipping.

- The fabricator shall furnish a complete set of shop drawings showing the necessary details.