BID SPECIFICATIONS FOR GEM THEATRE DOOR REPLACEMENT

Remove (1) existing wood transom door frame and residential steel door leaf.

We will then furnish and install (1) new "Tubelite" aluminum entrance door and transom door frame.

"TUBELITE" Aluminum Entrance Door and Transom Door Frame

• Aluminum finish to be dark bronze anodized.

• Door frame will have a 2" face profile x 4-1/2" depth.

• Door to be narrow stile with 1 O" bottom rail and hardware will consist of (3) standard butt hinges,

(1) standard rim panic exit device, exterior keyed cylinder, 1" tubular pull, Delaney 8000 grade 1 door

closer w/ drop plate, standard perimeter weatherstripping, standard ADA 5-1/4" threshold, and a

brush door sweep.

• All hardware to be ADA compliant.

• Storefront Door to be glazed with 1" Bronze/Acid Etched Tempered Insulated Glass.

• Transom to be glazed with 1" clear annealed ES36 Low E insulated glass.

• Insulated glass to carry ten year manufacturer's seal failure material warranty

• Interior and exterior perimeters to be sealed with Tremco Spectrum 2 sealant.

Elevation "A" Single Door and Transom Door Frame Double Stack

Door custom width of 35-1/4"

City of Etowah

Gem Theatre Door Replacement

- Provide cost for door as described in this package..

- All bidders will include in their Bid Packet the following items:

  - Proof of Commercial General Liability -- $ 1,000,000.00 Limit

  - Proof of Worker’s Compensation – Statutory Limit

  - Automobile Liability --$ 1,000,000.00

  - Copy of Certifications for application of chemicals.

  - Listing of all staff and equipment that will be used for execution of said services.

  - Current business license

  - Listing of all contact information

  - Iran Divestment Documentation

  - Title VI Compliance Documentation

Failure to include each required document will result in the bid being disqualified.

- Must be able to begin and finish project no later than June 30, 2022.

- All property damage, public and private, caused by contractor or subcontractor shall be the responsibility of the contractor and must be dealt with quickly and professionally.  Any and all damage with public or private shall be reported to City Hall immediately.

- All work is to be with professional care and diligence.

- Billing shall be presented to City Hall upon completion of project. Contractor/Subcontractor shall be provided documentation for submittal of all invoices.

- Payment for services shall be made payable to Contractor/Subcontractor within 30 days of invoice.