BID SPECIFICATIONS FOR -- Etowah Carnegie Library Door Replacement

Remove one (1) existing wood door leaf, then furnish and install one (1) new aluminum door frame and aluminum entrance door inside of your existing wood door frame.

Existing transom framing and transom glass to remain.

City of Etowah to replace existing brick mold and repaint the existing door frame and new brick mold.

"TUBELITE" Aluminum Entrance Door

• Aluminum finish to be dark bronze anodized.

• Door frame will not be thermally broken.

• Door to be medium stile with 1 O" bottom rail and hardware will consist of (3) butt hinges per door leaf,

standard maximum-security lock with (1) keyed cylinder and thumb turn, 1" tubular push and pull,

Delaney 6000 grade 1 door closer, standard perimeter weatherstripping, threshold, & brush door sweep.

• All hardware to be ADA compliant.

• Door to be glazed with 1" Bronze Acid Etched Tempered Insulated Glass.

• Interior and exterior perimeters only to be sealed with Tremco Spectrum 2 sealant.

Elevation "A" Custom Size Door and Door Frame

• Include Tubelite standard rim exit device.

City of Etowah

Etowah Carnegie Library Door Replacement

- Provide cost for door as described in this package.

- All bidders will include in their Bid Packet the following items:

  - Proof of Commercial General Liability -- $ 1,000,000.00 Limit

  - Proof of Worker’s Compensation – Statutory Limit

  - Automobile Liability --$ 1,000,000.00

  - Copy of Certifications for application of chemicals.

  - Listing of all staff and equipment that will be used for execution of said services.

  - Current business license

  - Listing of all contact information

  - Iran Divestment Documentation

  - Title VI Compliance Documentation

Failure to include each required document will result in the bid being disqualified.

- Must be able to begin and finish project no later than June 30, 2022.

- All property damage, public and private, caused by contractor or subcontractor shall be the responsibility of the contractor and must be dealt with quickly and professionally.  Any and all damage with public or private shall be reported to City Hall immediately.

- All work is to be with professional care and diligence.

- Billing shall be presented to City Hall upon completion of project. Contractor/Subcontractor shall be provided documentation for submittal of all invoices.

- Payment for services shall be made payable to Contractor/Subcontractor within 30 days of invoice.