NARRATIVE AND INSTRUCTIONS

The Marshall County Emergency Management Agency desires to purchase a new Brush Truck.

All offerings made under this invitation shall meet or exceed the detailed requirements as set forth in the attached minimum specifications, drawings and plans, if any.  All values specified in the invitation shall be considered as minimums, unless otherwise specified.

INSTRUCTIONS to BIDDERS:

BID SUBMITTAL:

The bidder must submit his proposal on the “Bid Forms” provided with the “Invitation to Bid”.  The bid price must be written in blue or black ink, in numerals, and in words.  The words, unless obviously incorrect, will govern.  No qualifying letters or statements attached to the bid form will be considered. The bids will be received by Marshall County Government until the day, date and time as set forth in the “Bid Invitation”.  Local time shall prevail in all openings.  The bid due date and time will be strictly observed.  It is the responsibility of the bidder to ensure that his bid is delivered via mail, carrier, by hand or other method prior to the scheduled bid opening day, date and time.

Before submitting a bid or proposal each bidder shall carefully examine the specifications (if any) and inform himself fully concerning the existing conditions, and shall make his bid or proposal to provide the items covered at the lowest cost while offering the best service and quality.

BID WITHDRAWAL:

Any bid may be withdrawn prior to the day, date and time as set forth in the “Bid Invitation”.  Any bid may be withdrawn as a result of any authorized postponement by the Owner.

BID OPENING:

A bid opening will occur at the time and on the date as scheduled in the “Invitation to Bid”.  At the “Bid Opening”; an “Apparent Low Bidder” will be announced. A “Successful Bidder” will be announced after a review period in which all elements of the bid are taken into consideration to include, but not be limited to, alternate or option bids which the Owner may want to include in the Work and which were bid on as alternate or option items in the original bid request, delivery schedules, service factors, and other pertinent data.  The “Apparent Low Bidder” may or may not be the “Successful Bidder”.

Alternate bids and specifications are not acceptable unless specifically asked for and/or authorized in the Invitation to Bid.

Bids submitted will be immediately rejected when:

- The bidder fails to use the provided bid form,

- Bid form is not signed by an authorized representative of the bidding company,

- Conditions are placed upon the bid by the bidder,

- Unauthorized additions to the bid by the bidder,

- Lack of appropriate documents as required or requested in the Invitation to Bid,

A “Bid Tabulation” will be made available to the attendees at the “Bid Opening” showing all bids submitted.  Individual bids will not be available for inspection until after the review period has occurred as outlined in “Bid Inspection” below.

BID INSPECTION:

A record shall be kept, listing each bidder by name and address and indicating their bid amounts.  These records are to be open to public inspection after review and after the award to the successful bidder.

Evaluation Period:

  - The review period is for ten (10) business days following the bid opening, during which time the bids are closed to public inspection.

  - During the period when evaluation is being made, all bid analysis is confidential, thereby maintaining the integrity of the bidding system.

  - No County personnel in any office should discuss information pertinent to any bid during this period, other than at scheduled meetings of County personnel specifically intended for review with regard to the bid and/or project.

  - Violation of the confidentiality of bids pending award seriously compromises the County’s position in establishing contractual agreements.

During the bid review period, certain issues may be discussed with the potential “successful” Bidder.  Any failure to come to an agreement on the discussed issues may result in the rejection of that bidder’s bid and force the Owner to move to the next bidder in line, when deemed by the Owner as being in the best interest of the Owner.

AWARD of BUSINESS:

A bid shall be considered an offer subject to acceptance by Marshall County.   If a bidder fails to state the time within which a bid may be accepted, Marshall County shall have a minimum of sixty (60) days to accept. Any award of business shall be to the lowest and best bidder, taking into consideration the qualities of the articles or services to be supplied, their conformity with specifications, their suitability to the requirements of the Owner and the delivery terms.  Any or all bids may be rejected for good cause.

QUALITY:

All items offered under this invitation shall be of the highest quality, shall be in strict accordance with the manufacturer’s published specifications and shall be to the Owner’s satisfaction.  The bidder shall ensure that all items offered shall be of superior quality.

MINIMUM SPECIFICATIONS:

CHASSIS

2023 Ram 5500 (or will consider Ford F-550) 4WD 4 door crew cab (or will consider extended cab if crew cab is not available) for this build.  If both are available bid both ways.

10 speed automatic transmission

6.7 – 7.3-liter V8 gas or diesel engine (prefer gas) with ambulance prep package

White finish with clear coat

4-wheel ABS

Air conditioning

Variable intermittent wipers

Dual front and side airbags

Reclining front bucket seats or will consider 40/20/40 split bench

Gray interior preferably work grade vinyl seating

Tinted glass

Power windows and locks

Minimum 220-amp alternator

Tachometer

Platform heavy duty black powder coated running boards entire chassis length under doors

Electronic shift on the fly transfer case

Transfer case, oil pan and other appropriate skid plates

Power/heated trailer tow mirrors

Rear view camera

All weather floor mats

No carpet

Back up alarm

Utility lighting system

DOT approved LED stop, turn, corner, backup, and taillights

2 ½ inch lift kit with replacement wide fender modifications

Payload upgrades 19,500 GVWR minimum

20 x 10 forged aluminum polished aluminum rims

Single wheel conversion with differential gear change

After market Kevlar oversized off road tires – continental MPT81tires preferred – 335x80xR20 (40x12.5) with a weight rating of 6779 lbs. – mounted spare to be included

Heavy duty shocks

Speed correction for accurate speed

Custom cab console for siren, light boxes, remote monitor control and radio installation

Cup holders

The final console layout will be approved by the customer

Heavy duty front bumper with brush guard black in color

Front bumper mounted winch with minimum of 12,000 lb rating, 125-foot cable and handheld remote control

Class 5 rear receiver with tow hooks / chain attachments / and rated to 16,000 lbs.

BODY

Minimum 9’ aluminum flatbed body (max 12’) with 10-year warranty

Rhino lining on flat bed surfaces

Crosswalk minimum 20” and workstations with LED lighting

Flip up seat with harness

Rear license plate holder

Rear tire mud flaps

Pull out and drop-down steps for crosswalk and rear of body meeting NFPA standards

Reflectors installed in accordance with federal motor vehicle safety standards

Rear pump station LED work light

The body will be set on the chassis and fastened with grade 8 bolts and custom mounts. The body shall be separated from the chassis frame rails by a ¾” rubber separator for flex and to resist galvanic dissolution.

Storage

Side bed storage x 2 sides’ minimum 60Lx20Dx23T aluminum diamond plate construction with flip up doors with 2 T handle latches, adjustable shelving, and turtle tile or equivalent lining.  Roll up aluminum doors will be considered along with painted flip up doors with the same basecoat clear coat finish as the cab

Tray above drivers’ side compartment for 1 1/2” pre-connect hose lay capable of 150ft. of 1 ½ inch hose

Under body long storage compartment with rear access between frame rails of the body 7”x24”Wx84”L and removable slide out polypropylene tray with side handles – flip down latch handle on door

2 -24” W x 18”D x 18” T aluminum diamond plate underbody storage units mounted at front of body with t handle latch and flip down doors and turtle tile or equivalent lining

All compartment doors to have full length aluminum hinges

Polypropylene storage tray in crosswalk area to hold whip line

Skid Unit

WATER TANK

300-gallon polypropylene tank to be quoted with sight level gauge and LED panel and cab gauge

3” suction and 1.5” fill, 8”x8” fill tower, 3” overflow

¾” floor, ½ “walls, 3/8” baffles, tank bolt down tabs, lifting lugs, black in color with lifetime warranty

PUMP

200 gpm rated pump with bronze impeller – max 245gpm @175psiquote with both a 3-gallon fuel tank and plumb to chassis fuel system

Honda 20 hp v-twin engine or equivalent with high pressure piston pump

Choke and throttle control

Push button start with off/on switch

Quote with and without pump panel

Exhaust elbow kit

Exhaust prime for drafting with priming lever

3” NPT suction ball valve

2 1/2” discharge

Plumbed with fire rated nickel plated brass ¼ turn radius ball valves

Stainless steel manifold

1 inch booster reel discharge

1 ½ inch discharge for pre-connect with plug and chain

2 ½” tank to pump

2 ½” suction with plug and chain

Monitor discharge

1 inch whip line discharge connection – discharge will be in cross walk

1 inch recirculation fill line

FIREFIGHTING

1 inch x 10 ft. double jacketed whip line hose

1 gateable front turret package to include remote controlled monitor with selectable gallonage nozzle 15-120 gpm

Cab gateable joystick operator station

Hard coated aluminum alloy powder coated inside and out

Horizontal movement 270 degrees

Vertical movement 45 to 90 degrees

Hannay red painted steel booster reel with electric rewind and manual override – capable of 100’ of 1 inch hose

Chrome outriggers with 3-way rollers

100 feet of 1-inch Boostlite rubber forestry grade hose

2 – TFT Twister series nozzles with pistol grips

For whip line and booster reel

20 and 60 gpm flow settings at 100psi

1.5-inch NH swivel standard

Lightweight hard coat anodized aluminum

Quick-change rear valve seat, stainless steel shut-off valve

2- Nozzle brackets, weather and UV resistant, adjustable grip range of 3/8 inch to 3.25 inch that can be mounted horizontally and vertically for above nozzles

2 – 2.5-gallon water can type fire extinguishers mounted to rear of body behind side bed storage units

EMERGENCY LIGHTING AND SIREN

Whelan or equivalent 56” red/white LED light bar with takedowns and alley lights, 2 red/white front grill lights and 2 red front bumper lights, red front fender intersection lights to both sides, red side bed lights to both sides and 2 red/yellow rear bed lights meeting 360-degree visibility requirements

360-degree LED telescoping scene lighting with side mount pole to be mounted to back of cab or

crosswalk area

Electrical

20-amp battery charger with auto eject

GRAPHICS

NFPA dot chevrons on rear of body

Logo on front cab doors x 2

2 color striping to cab and body with station number on stripes at the cab both sides

Unit ID number on front fenders x 2

All graphics to be installed by 580 Graphics in Shelbyville, Tennessee

OPTION

Quote with and without Firecom wireless headset system

Quote with and without delivery

ANY MANUFACTORER SPECIFIC DEVIATIONS FROM BID SPECS WILL BE CONSIDERED; HOWEVER THEY MAY BE GROUNDS FOR BID REJECTION.