SPECIFICATIONS FOR INSTALLATION OF 1000 FEET OF 6 FOOT HIGH CHAIN LINK FENCING.

Requested bidders, please provide a “TURNKEY PRICE” for the Installation of the fencing and gates as per specifications.

- ALL BIDDERS ARE REQUIRED TO DO HAVE DONE A SITE EVALUATION AT 451 McMinnville Hwy at the Manchester Sports Park.

3.    BIDS ARE DUE ON Monday, October 15 AT 10:00AM.  THEY MUST BE SEALED WITH CONTRACTORS NAME, LICENSE NUMBER ON THE OUTSIDE.  IF MAILED OR DELIVERED THEY MUST BE RECEIVED BEFORE THE BID DEADLINE.  THEY CAN BE MAILED OR DELIVERED TO BRIDGET ANDERSON, FINANCE DIRECTOR CITY OF MANCHESTER, 200 WEST FORT STREET MANCHESTER TENNESSEE.  ALL QUESTIONS REGARDING THE BID MAY BE ADDRESSED TO BONNIE GAMBLE, DIRECTOR PARKS AND RECREATION, BGAMBLE@CITYOFMANCHESTERTN.COM OR 931-728-0273

4. Owner reserves right to accept or reject any or all Bids., reject a Bid not accompanied by a required      Bid Security or by other data required by Bidding Documents, reject a Bid which is in any way incomplete, illegible, unsigned, improperly signed or sealed, obscure, or reject a Bid which contains arithmetical errors, erasures, alterations, or irregularities of any kind.

Bids received after scheduled opening time will be returned to Bidder unopened.

5.  Bidders provide documentation demonstrating they have been in the fencing installation business for at least 5 years.

6. PLEASE REFER TO ENCLOSED DRAWING.

BASE BID

GENERAL

MATERIAL FOR FENCING

      - Approximately 1000 feet of black chain link fencing.

      - Height will be 6ft

      - Gauge is 6/9 core with Tension wire.

      - Fabric/wire will be Heavy Commercial Knuckle and Knuckle 9 gauge core with 6 gauge finish.

      - 3 (5ft) walk gates with end/gate post/corner equal to 3” 40 weight.

      - (2) 12 ft double drive gates

      - Line post equals 2 “ 40 weight

      - Top rail equals 1 5/8” 40 weight

      - Tension wire is Heavy Commercial 9gauge core, 6 gauge finish.

LAYOUT

The project will be composed of three areas.

            - Dog Pad Area is 15 ft x 15ft with  3 walk gates

            - Small Dog area with a 10ft double drive gate

            - Large Dog area with a 10ft double drive gate