Submit Proposal to:

Calhoun County Government

102 Courthouse Drive Ste. 112
St Matthews, SC 29135

Project:
Exterior Renovation

Contact:
Woody Rucker
803-837-0113

Storage Facility Details:

Sandy Run Tank I Calhoun County Water Department

Southern Corrosion Incorporated proposes the following scope of work: Scope of Work:

The exterior of the water tank will be pressure washed utilizing varying degrees of pressure to remove all caulk, contaminates, and loosely adhered paint, All localized corrosion will be addressed per SSPC-SP2 Hand Tool Cleaning and SSPC-SP3 Power Tool Cleaning surface preparation standards. All tooled localized and general corrosion will be spot primed. Then one (1) full coat of paint will be applied to the entire exterior of the structure and components.

All surface preparation and coating application will comply with fhe Coating Manufacturer's Production Information document.

Coating System:

Primer: Sherwin Williams Macropoxy 646 Epoxy.

Finish Coat: Sherwin Williams 1-ii-Sailds Polyurethane B65W. Owner to confirm the color of the finish coat.

identified Repairs:

1.	Inspect, operate, and make any needed repairs or maintenance to ensure the free movement of the level indicator components.

2.	No other repairs were identified during the inspection. Any other repairs identified during the renovation will be brought to the attention owner.

Insurance:

1. A certificate of insurance shall be provided for your records. Warranty:

1.	The Contractor shall warrant all material and craftsmanship to be free from defect in material and workmanship for period of one (1) year from the completion date.

2.	A NACE Level 1 Certificated Southern Corrosion representative shall return at the end of one year and along with the owner or owner's representative, perform an inspection of the work to identify any warranty issues. Any deficiencies noted during the warranty inspection will be addressed sixty (60) days of the inspection date.