Brand New 2018 or 2019 66,000 GVW Truck Chassis, Roll Off Hoist, and Tarper

All exceptions to these specifications must be noted and explained in terms of performance and cost.

All bids must be good for 30 days after bid opening.

Truck Chassis

- 11O” BBC Set Forward Axle

- Cab Construction, Steel or Aluminum, Specify

- 184” Clear C.A. for Roll Off Hoist

- Engine 13 Liter 410 HP 1650 Ft. Lbs. Torque GHG17 Certified

- After treatment Device – Locate Under Passenger Door

- After treatment Device – Auto Over the Road Regeneration

- Exhaust – Single Vertical Mounted to Cab “B” Post

- 3 Stage Jake Brake

- Double Frame, 3.2 Million R.B.M. 120,000 PSI Steel, 7/16” Outer – 1/4” Inner – Section Modulus – 31.0.

- Frame Fasteners – Truck Cab & Chassis, Grade 8 Bolts

- Block Heater with Plug Under Driver Door

- Jumpstart Post – Located on Frame, Under Hood, Near Starter

- PTO Fast Idle to 900

- Green House Gas (GHG) EPA Certified Engine

- 19.0 CFM Air Compressor

- Fan Clutch – Air, Automatic, Plus Dash Interior On and Off Switch

- Radiator – 1500 Sq Inch

- Antifreeze - 60° Below Zero

- Fuel Filter and Fuel Water Seperator

- Alternator – 160 Amp

- Batteries – 3 Each 12 Volt Galss MAT 2325 CCA Minimum

- Automatic Transmission with PTO with PTO Factor Switch

- Transmission Cooler – Water To Oil Heat Exchanger

- Tandem Axle Temperature Gauges – 2 Gauges – Bright Bezels

- Front Axle 20,000 Lb Rated

- Front Axle Wheel Cut - 45° Minimum

- Rear Axle 46,000 Lb Rated – Ratio 4:56

- Driver Control Tandem Axle Differential Lock, Both Axles Lock – Lighted Dash Switch with Buzzer

- Rear Suspension 46,000 Lb Hendrickson RT463 Bronze Bushings

- Drive Line – 1810 HD, Inter-axle 1710 HD

- Power Steering – Gear Driven Pump- TRW THP Power Steering with RCH Auxiliary Gear RHS. 4 Quart Reservoir

- Power Steering Cooler – Oil to Air

- Front Brakes – 16.5 x 6Q + Meritor and Auto Slack Adjusters – Dust Shields

- Rear Brakes – 16.5 x 8.62Q + Meritor Cast Spider Cam And Auto Slack Adjuster – Dust Shields

- Rear Brake Chamber Mounted In High Clear Position

- Front Tires – 315/80R22.5 20 Ply Radial

- Rear Tires – 11R 22.5 14 Ply

- Front Wheels – 22.5 x 9.00 Steel – 5 Hand Hole – Hub Pilot

  - Vendor White Color

- Rear Wheels – 22.5 x 8.25 2 Hand Hold HD Rear Wheels

  - Vendor White Color

- Air Dryer – Heated Bendix AD9 – Mounted Inboard Rails

- 18” Tilt & Telescoping Steering Wheel

- Gauges – Air Filter Restrictor, Engine Oil Temperature, Transmission Oil Temperature, In Addition to Standard Gauges All with Bright Bezels, Hour Meter

- Dashboard – Prefer All Metal and Hinged – Specify

- Cab Glass – All Factory Tint – 2 Piece Windshield, Window in Lower Passenger Door, with Fresnel Lens – RHS Power Window

- Fuel Tank – 100 Gallon Minimum

- D.E.F. Tank – 13 Gallon Minimum

- Clearance Lights – On Top of Cab Roof

- Factory HVAC, Heat and Defrost

- Seating

  - Driver Seat – High Back Air & Air Lumbar with 2 Seat Mounted Arm Rests

  - Passenger Seat – Mid Back Fixed

- Cab Mounts – Dual Air Suspension

- Grill – Bright, Radiator Mounted, Bugscreen Behind Grill

- Mirrors – West Coast Style Stainless Heated, plus 8” Diameter Stainless Left and Right Convex Mirrors, and RH Downview Mirror

- Hood - 90° Tilt Hood with Oil Filled Safety Closing Dampners

- 12 Volt Supply in Dash – In Addition to STD

- Automatic Circuit Breakers

- CB Power and Hard Point Mounting – Top of Dash with (1) CB/Antenna

- Color – Solid White Cab – Solid Black Chassis

- Radio – AM/FM, WB, CD Radio – with Bluetooth

- Electrical – Prefer Relays and Electrical Mounted Up High in the Dash Board

- Telematics – 2 Year Contract – Remote Diagnostics Service

  - Equal to Virtual Technician

Minimum Warranty

- Extended Warranty – Engine – 5 Year/200,000 miles

  - Zero Deductible Engine Warranty Minimum

- Engine Major Components – 5 Year/500,000 miles

- Frame, Cross Members, Cab Corrosion – 5 Years/Unlimited Miles

- Axles – 3 Years – Unlimited Miles

- Transmission Warranty – 5 Years/Unlimited Miles

- 12 Month Warranty on All Other Components

Attachment Your Warranty Parchments

Heavy Duty Roll-Off Hoist

- Type: Outside Rail

- Capacity: 60,000 Lbs.

- Hoist Main Frame: 4" x 8" x ½" A500 Grade C Tubing. One Piece Construction with No Splits or Welds

- Hoist Sub-frame: 3" x 2" x 3/16" A500 Grade C Tubing. One Piece Construction with No Splits or Welds

- 3" x 3/8" Tl Wear Strip on top of Main Frame

- 60" Long Rear Skid Plates - 3/8" Thick w ½" x 5" Full Gusset Support

- Outside rollers 3 ¾" Long x 4" Diameter w/Bronze Bushings, Grease Fittings , and Outside Support Brackets.

  - Minimum (5) Per Side, (10) Total

- Automatic Air-Actuated , Fold-up ICC Bumper

  - Note: Mechanical Linkage Type Fold-up ICC Bumpers that are

Prone to Bending and Mis-Alignment Are Not Acceptable

- ICC Bumper Shall be Designed to be Folded Towards Rear

- Heavy Duty Tandem Fenders to be 3/16" Steel Tread Plate for Maximum Strength and Durability. Smooth Steel Not Acceptable

- Automatic Gravity Activated Front Container Locks

- 1 " Thick Steel Rear Container Hold-Downs Required

- 10" Diameter Cable Sheaves w/Bronze Bushings and Grease Fittings

- Dump Angle to be Minimum 52°

- Hydraulic Pump: GPM @ 1500 RPM

- Hydraulic Tank: 50 Gallon Capacity w/Sight Gauge, 150 GPM Rated Return Line Filter Assembly with Replaceable Cartridges, 40 PSI Pressure Bypass, and Separate Air Breather Filter

- Operating Pressure: 1850 PSI Maximum

- Air Shift Hot Shift PTO for 4500 RDS Automatic Transmission

- Interchangeable Hoist & Winch Cylinders: Single Stage Type 6" Bore x 79" Stroke x 4" Hard Chrome Rods. Hoist Models Using Non-Interchangeable Hoist and winch Cylinders are Not Acceptable

- Lift Cylinders Shall be Pinned at Front of Hoist Frame for Maximum Lift Rated Capacity. Reverse Mounted Lift Cylinders are Not Acceptable

- Winch Cylinder Rods to be Protected from Falling Debris by 6" x ¼" Angled Steel  Covers

- Outside Controls for Hoist and Tarper Shall Use Integrated Heavy Duty (5) Spool Valve Bank w/Protective Steel Housing. Hoist Models That Do Not Integrate the Tarper Controls into the Hoist Control Are Not Acceptable

- Cable: 7/8" x 75' Long; 6 x 37 IWRC; 79,600# Rated Breaking Strength Minim um

- Cable End: 2" Thick Swivel-type Cable Hook on Swaged Steel Connection

- All Rear Lighting Including Back-up Lights as Required by ICC and DOT 108 to be LED Type and Flush Mounted into Rear Bumper

- 2 Additional Work Lights Mounted Low On Tarper Gantry

- All Electrical Wiring Shall Consist of a Plug-N-Play Factory Made Harness to Eliminate Wire Splicing and Need for Junction Boxes

- Conspicuity Type Markings as Required by DOT Regulations

- Electrical Back-up Alarm to be Self Adjusting Exterior Type

- Hoist Up Exterior Audible Alarm w/2" In-Cab LED Red Indicator Light

- Inside Air-Actuated Hoist and Winch Controls Shall be Conveniently Located in a Dedicated Fully Enclosed Control Console to Right of Drivers Seat. The Control Console shall also House Hoist PTO and Lighting Controls. Dash Mounted Controls Are Not Acceptable

- A Pioneer RP4500 SARG Auto Tarper with Hydraulic Adjustable Gantry And Tarp Arms Shall Be Provided w/All Tarper Controls Integrated Into the Same Valve Bank as Used by the Outside Hoist Controls

- A 48" x 20" x 16" Steel Tool Box that Opens from the Front is Required Tool Boxes that Open from the Top are Not Acceptable Because They Cannot be Opened with a Container On-board the Hoist Frame

- Hoist Shall be Primed and Painted Using a Baked-on Two-Part Epoxy Paint For Maximum Durability. Air-dry Paint Processes are Not Acceptable

- To Insure Performance and Reliability Standards are Met Hoist and Tarper Shall Be Factory Installed on Truck Chassis by the Hoist Manufacturer and Tested Prior to Delivery. Huck Bolts Shall Be Used to Secure Hoist Frame and Components to the Truck Frame

Warranty Requirements

- Hoist Frame: Lifetime Warranty

- Complete Hoist: I Year Warranty

- Hydraulic Cylinders, Pump, Valves: 2 Year Warranty

**Bidder Shall Provide a Contact List of Ten (10) Tennessee Based Counties Who Are Using the Exact Model Hoi s t Being Proposed