NARRATIVE AND INSTRUCTIONS

The Marshall County Ambulance Service desires to purchase an Ambulance Chassis Remount.

All offerings made under this invitation shall meet or exceed the detailed requirements as set forth in the attached minimum specifications, drawings and plans, if any.  All values specified in the invitation shall be considered as minimums, unless otherwise specified.

INSTRUCTIONS to BIDDERS:

BID SUBMITTAL:

The bidder must submit his proposal on the “Bid Forms” provided with the “Invitation to Bid”.  The bid price must be written in blue or black ink, in numerals, and in words.  The words, unless obviously incorrect, will govern.  No qualifying letters or statements attached to the bid form will be considered. The bids will be received by Marshall County Government until the day, date and time as set forth in the “Bid Invitation”.  Local time shall prevail in all openings.  The bid due date and time will be strictly observed.  It is the responsibility of the bidder to ensure that his bid is delivered via mail, carrier, by hand or other method prior to the scheduled bid opening day, date and time.

Before submitting a bid or proposal each bidder shall carefully examine the specifications (if any) and inform himself fully concerning the existing conditions, and shall make his bid or proposal to provide the items covered at the lowest cost while offering the best service and quality.

BID WITHDRAWAL:

Any bid may be withdrawn prior to the day, date and time as set forth in the “Bid Invitation”.  Any bid may be withdrawn as a result of any authorized postponement by the Owner.

BID OPENING:

A bid opening will occur at the time and on the date as scheduled in the “Invitation to Bid”.  At the “Bid Opening”; an “Apparent Low Bidder” will be announced. A “Successful Bidder” will be announced after a review period in which all elements of the bid are taken into consideration to include, but not be limited to, alternate or option bids which the Owner may want to include in the Work and which were bid on as alternate or option items in the original bid request, delivery schedules, service factors, and other pertinent data.  The “Apparent Low Bidder” may or may not be the “Successful Bidder”.

Alternate bids and specifications are not acceptable unless specifically asked for and/or authorized in the Invitation to Bid.

Bids submitted will be immediately rejected when:

- The bidder fails to use the provided bid form,

- Bid form is not signed by an authorized representative of the bidding company,

- Conditions are placed upon the bid by the bidder,

- Unauthorized additions to the bid by the bidder,

- Lack of appropriate documents as required or requested in the Invitation to Bid,

A “Bid Tabulation” will be made available to the attendees at the “Bid Opening” showing all bids submitted.  Individual bids will not be available for inspection until after the review period has occurred as outlined in “Bid Inspection” below.

BID INSPECTION:

A record shall be kept, listing each bidder by name and address and indicating their bid amounts.  These records are to be open to public inspection after review and after the award to the successful bidder.

Evaluation Period:

  - The review period is for ten (10) business days following the bid opening, during which time the bids are closed to public inspection.

  - During the period when evaluation is being made, all bid analysis is confidential, thereby maintaining the integrity of the bidding system.

  - No County personnel in any office should discuss information pertinent to any bid during this period, other than at scheduled meetings of County personnel specifically intended for review with regard to the bid and/or project.

  - Violation of the confidentiality of bids pending award seriously compromises the County’s position in establishing contractual agreements.

During the bid review period, certain issues may be discussed with the potential “successful” Bidder.  Any failure to come to an agreement on the discussed issues may result in the rejection of that bidder’s bid and force the Owner to move to the next bidder in line, when deemed by the Owner as being in the best interest of the Owner.

AWARD of BUSINESS:

A bid shall be considered an offer subject to acceptance by Marshall County.   If a bidder fails to state the time within which a bid may be accepted, Marshall County shall have a minimum of sixty (60) days to accept. Any award of business shall be to the lowest and best bidder, taking into consideration the qualities of the articles or services to be supplied, their conformity with specifications, their suitability to the requirements of the Owner and the delivery terms.  Any or all bids may be rejected for good cause.

QUALITY:

All items offered under this invitation shall be of the highest quality, shall be in strict accordance with the manufacturer’s published specifications and shall be to the Owner’s satisfaction.  The bidder shall ensure that all items offered shall be of superior quality.

MARSHALL COUNTY EMS

Lewisburg, TN

REMOUNT AMBULANCE SPECIFICATION

EMERGENCY MEDICAL VEHICLE

TYPE III CLASS 2 (4X2) DRW

TYPE III REMOUNT

DESIGNED TO MEET OR EXCEED

FEDERAL SPECIFICATION KKK-A-1822E,

AMD Standards 001 - 009

It is the intent of these specifications, and all requirements listed herein, to obtain bids on vehicles to meet the local needs of Marshall County, and to obtain bids that may be easily, and completely evaluated on an equal and competitive basis.

Because of the wide variances in types, configurations, options and models available in the industry, the purpose of these specifications is to set a standard by which all proposals received may be evaluated, studied and compared equitably. Vendors are required to return one (1) copy of these specifications, correctly and completely furnishing all information requested and answering all questions attached.  Any requested data literature, drawings, etc. and detailed written lists of exceptions taken must also be enclosed.

Since bids from a number of suppliers are anticipated, and Marshall County cannot be expected to be familiar with all various technical details of all vendor's products, the only adequate method of evaluation will be to compare in this format.  Vendors are cautioned that failure to submit proposals in format specified herein, will be grounds to reject vendors proposal and remove bid from consideration.

Any questions for clarifications, etc. should be directed in writing to the above, and responses will be furnished in writing to all prospective vendors.

Considerable time and effort have been invested to design these specifications to the specific needs of Marshall County EMS. Therefore, alternate bids will not be considered. No Exceptions to this will be allowed.

It is further understood that the vehicle described is for use in the provision of emergency services.

REMOUNT SPECIFICATIONS

EMERGENCY MEDICAL CARE VEHICLE

1.0	Scope, Purpose and Classification

1.1	Scope:  This specification covers the remounting and refurbishment of a commercially produced, surface medical care vehicle, hereinafter referred to as ambulance or vehicle.  A vehicle in compliance with this specification shall be defined as a standard ambulance.  This vehicle shall be in accordance with the Ambulance Design Criteria of the National Highway Traffic Administration, U.S. Department of Transportation, Washington D.C.  This bid specification is based on the Federal Ambulance Specification KKK-A-1822E.

1.2	Purpose:  The purpose of this document is to provide minimum specifications and test parameters for the remounting and refurbishment of an emergency medical care vehicle that meets the needs and desires of this agency.  It establishes essential criteria for the design, performance, equipment and appearance of the vehicle.  The object is to provide a vehicle that is in accordance with nationally recognized guidelines.   It is the intent to return the unit to a new or like new condition per the specification.

1.3	Classifications:  This specification calls for the following type of vehicle.  It is in accordance with KKK-A-1822.

Type III – Cutaway Van Chassis with integral modular ambulance body.

Class 2 - Two rear wheel driven (4x2)

Configuration A (ALS) - Elevating cot and squad bench (3.1.5.1)

1.4	General:  This is an engineer, design, construct and deliver type specification and it is not the intention of this agency to write out vendors of similar or equal equipment of the types specified.  It should be noted, however, that this specification is written around specific needs of this agency.  With the intent to standardize certain components, therefore, in numerous places we have named specific brands of components.  This has been done to establish a certain standard of quality.  Other brands will only be accepted providing the vendor provides documentation in the bid that the particular brand offered meets or exceeds the quality of the actual brand called for in the specification.

1.5	Materials:  The emergency medical care vehicle, chassis, ambulance body, equipment, devices, medical accessories and electronic equipment to be delivered under this contract shall be standard commercial products, which meet or exceed the requirements of this specification.  The ambulance shall comply with all Federal Motor Vehicle Safety Standards (FMVSS), the Federal regulations applicable or specified for the year of manufacture.  The chassis, components and optional items shall be as represented in the manufacturer's current technical data.  Materials used in the construction shall be new and not less than the quality conforming to current engineering and manufacturing practices.  Materials shall be free of defects and suitable for service intended.

1.6	Manufacturing Ability:  It is the intent of these specifications that the Remounter of these vehicles has the ability to remount and refurbish a complete ambulance within their plant facility, except for the chassis.   The Ambulance Remounter must have significant experience in the remounting and refurbishment of modular ambulance bodies and performed a minimum of 100 remounts in the last two years.  In addition, the Bidder/ manufacturer must include with this bid, proof of $5,000,000.00 product liability coverage.  NO EXCEPTIONS.

1.7	Exceptions to Specifications:  Any exceptions to these specifications indicated must be clearly pointed out.  Otherwise, it will be considered that items offered are in strict compliance with these specifications and the successful bidder will be held responsible for delivering a vehicle meeting these specifications.  Any exceptions must reference by Paragraph Number and Line and be explained in detail on a separate sheet marked "Exceptions".  Any bidder not complying shall not be considered as responsive.

1.8	Prices and Payments:  All bid prices shall be complete and include warranty and delivery of the completed vehicle to this agency.

Payment shall be made in accordance with these specifications and the Bid Proposal submitted

by the Bidder.  Payment will be made upon acceptance of the vehicle(s) and equipment specified under these specifications.

All bid prices and conditions must be specified on the Bid Proposal Form.  Bid prices shall be valid for at least 30 days from the date of the Bid Opening, or as otherwise specified on the Bid Proposal form.

Full payment will be made as each unit is received, inspected and found to comply with procurement specifications, free of damage and properly invoiced.

Successful vendor shall, at delivery, supply MARSHALL COUNTY EMS with MSO’s.

Does your bid comply EXACTLY as written?        Yes _____  No _____

1.9	Warranty:  The successful bidder shall provide remounter’s warranty coverage for the ambulance or rescue vehicle conversion, which coverage shall, at a minimum include:

- A Limited Warranty shall be provided on the ambulance remount conversion.  The remounter shall warrant to the original retail purchaser that, for a period of thirty six (36) months or thirty-six thousand (36,000) miles from the point of delivery, whichever first occurs, the ambulance remount shall be free of substantial defects in remount materials and remount workmanship, which are attributable to Warrantor remount work and which arise during the course of normal use and service.  There shall be a copy of the Warranty statement included with the bid documents.  NO EXCEPTIONS.

Does your bid comply EXACTLY as written?        Yes _____  No _____

(B)  There shall be a three (3) year / 36,000 mile warranty on the Ambulance Remount electrical wiring system effective starting the date this agency accepts delivery of the completed vehicle.  It is the intent of this requirement that items not manufactured or installed by the ambulance remounter be excluded.  There shall be a copy of the Warranty statement included as part of the bid documents.  NO EXCEPTIONS.

Does your bid comply EXACTLY as written?        Yes _____  No _____

(C)  There shall be a three (3) year / 36,000 mileage paint warranty covering the ambulance conversion and any painting done to the chassis.  There shall be a copy of the Warranty statement included with the bid.  NO EXCEPTIONS.

Does your bid comply EXACTLY as written?        Yes _____  No _____

(D) Warranty on the chassis portion of the completed ambulance or rescue vehicle shall be the responsibility of the chassis manufacturer, General Motors Company.  The terms of such warranty are set forth in the General Motors Warranty literature included in the owner’s manual.  The successful manufacturer’s sole responsibility with regard to the chassis shall be to provide reasonable assistance to the purchaser in obtaining warranty from the chassis manufacturer or its authorized service center.  Bidders failing to meet these requirements shall be considered non-responsive.

1.10	Anti-Collusion Statement:  By executing and submitting this bid, the Bidder certifies that his Bid is made without reference to any other bid and without any agreement, understanding, collusion or combination with any other person in reference to such bidding.

1.11	Ambulance Bids and Alternate Bids:  Considerable time and effort has been invested to design these specifications to the specific needs of MARSHALL COUNTY EMS.  Therefore, alternate bids not exactly to spec will not be considered.

1.12 	Warranty Surety:

To ensure quality, service, and full compliance to the above warranties the vehicle, with the exception of the chassis, must be constructed by the ambulance manufacturer.  Additional elements constructed and installed "in house" are required to ensure service and parts availability.  Subcontractors or lease/rental agreements to outside agencies will fail to meet this requirement.  NO EXCEPTIONS WILL BE ALLOWED.

- Does the ambulance manufacturer as the prime contractor remount the modular body?

Does your bid comply EXACTLY as written?        Yes _____  No _____

- Does the ambulance manufacturer as the prime contractor apply paint?

Does your bid comply EXACTLY as written?        Yes _____  No _____

- Are interior cabinets constructed and installed by the ambulance manufacturer as the prime contractor?

Does your bid comply EXACTLY as written?        Yes _____  No _____

- Are the wiring harnesses, circuit boards, and O2 systems  tested by the ambulance manufacturer as the prime contractor?

Does your bid comply EXACTLY as written?        Yes _____  No _____

- Is the upholstery for seat cushions, head pads, and backrests assembled and installed by the ambulance manufacturer as the prime contractor?

Does your bid comply EXACTLY as written?        Yes _____  No _____

1.13	Delivery:

Since delivery proposals by the bidder will weigh heavily in the determination of the bid award, the delivery schedules that are submitted by the bidders shall automatically become binding upon the successful bidder. Delivery delays due to component supply problems or chassis delivery problems to the manufacturer shall not penalize either the dealer or the manufacturer.  Delivery must be within one hundred fifty (120) days from arrival of the chassis from the manufacturer. Delivery mileage in excess of 100 miles shall be by common carrier or rail.   ABSOLUTELY NO EXCEPTIONS WILL BE ALLOWED TO THIS SECTION.

Does your bid comply EXACTLY as written?        Yes _____  No _____

1.14	Anti-Collusion Statement:

By signing this bid, the bidder agrees that his bid is made without any understanding, agreement, or connection with any other person, firm, or corporation making a bid for the same purpose and that his bid is in all respects fair and without collusion or fraud.

COMPANY NAME:

BIDDER SIGNATURE:

DATE:

1.15	   Service and Warranty Repair:

Successful vendor must have a comprehensive ambulance service and repair facility to administrate warranty repair as needed.  Vendor must have authorized factory trained service specialists and carry necessary inventory levels to affect prompt repair. NO EXCEPTIONS.

Bidders must list below their service facility and parts department profile to the purchaser.

FACILITY NAME:

ADDRESS:

PHONE NUMBER:

CONTACT NAME:

APPROX. MILES FROM PURCHASER:

YRS OF EXPERIENCE IN AMBULANCE REPAIR BUSINESS:

SQUARE FT. OF FACILITY:

NO. OF SERVICE BAYS:

NO. OF EMPLOYEES:

NO. OF FULL TIME AMBULANCE TECHNICIANS:

ARE TECHNICIANS FACTORY TRAINED?

TOTAL YEARS SHOP REPAIR EXPERIENCE:

VALUE OF SERVICE AND PARTS INVENTORY:

24-HR TURN AROUND TIME ON WARRANTY AND SERVICE?

NUMBER OF LIFT AREAS FOR SERVICE:

CAN SERVICE FACILITY PERFORM REMOUNTS?

CAN SERVICE FACILITY PERFORM ALUMINUM MODULE REPAIR?

IS FACILITY AND AUTHORIZED PPG COMMERCIAL FLEET PAINT

CENTER AND REPAIR FACILITY?

CAN SERVICE FACILITY DO SCOTCHLITE & LETTERING?

Does your bid comply EXACTLY as written?        Yes _____  No_____

1.16	Documentation of Remount Facilities:  This agency is very concerned with making bid award to the most

qualified remounter. In order to aid our decision, each bidder shall submit with their bid the following documentation:

- Photographs of physical plant including:

  - All bays & interior of remount facilities

  - Paint shop

  - Paint booth, spray facilities, and mixing room.

  - Hydraulic lifts

  - Frame machine

  - Stocks of remount inventory and remount chassis.

- Documentation of adequate Workman’s Compensation insurance, Product Liability Insurance, and Garage Keepers Liability Insurance.

- A current copy of their state issued Motor Vehicle Dealers License.

There are no exceptions allowed to this requirement. If requested documentation cannot be submitted with the bid, please do not go any further.  You will not qualify to bid.

Does your bid comply EXACTLY as written?   Yes_____No_____

  - Inspection Trip:

a.	There shall be a pre-build conference held at this agency’s facility prior to start of work.

- The successful bidder shall provide frequent status updates during construction.  Photos showing the progress shall be sent via email.

- Upon completion, two (2) Marshall County EMS representatives will meet at builder’s plant for the final inspection and acceptance.  Full payment will be made at this time.  The successful bidder shall pay all reasonable travel expenses (one-way).  Bidders more than 100 miles from Lewisburg, TN shall provide transportation, at no cost to Marshall County EMS, via commercial air carrier.  Meals, lodging, and fuel shall be included.  Bidders shall include the cost of these meetings and travel in with their bid.  These items are not to be “optioned” or “line itemed” in the bid.

Does your bid comply EXACTLY as written?        Yes _____  No _____  N/A_______

1.18	Marshall County EMS requires that the ambulance remounter provide “in house” repairs for structural repairs from any outside accidents, as well as warranty items that may arise as the result of the remount work.  Service facility shall be within 50 miles of Lewisburg, TN.

Does your bid comply EXACTLY as written?        Yes _____  No _____

1.19	References

This agency wishes to do business with an experienced, reliable, and honest ambulance contractor.  Contractor will provide a reference list of no less than twenty five (25) customers who have purchased similar remounts.  The reference listing shall include the name of service, contact person, and contact information.  Furthermore, the list shall include all new ambulance remount sales delivered by the bidding party for the last six (6) months.  The customers shall be identifiable in the reference listing as recent remount deliveries.  Marshall County EMS will sample the said references, along with any additional references from various sources, and compile the results.  The successful vendor will have a minimum 90% approval rate.

This agency is extremely concerned with awarding to the most qualified bidder.

There are NO EXCEPTIONS allowed to this requirement.  If you cannot, or choose not to comply, please do not

go any further.  You will not qualify to bid.

Does your bid comply EXACTLY as written?        Yes_____No_____

BID AWARD

BID AWARD CRITERIA: The agency is soliciting quality, state-of-the-art, and reliable emergency medical vehicles.  Acquisition price shall not be the only or the prevailing factor in deciding the successful bidder.

WARNING:

This agency will not tolerate Vendors who state compliance to specifications but deliver an incomplete product and/or sub-standard materials and workmanship.  Vendors who have made delivery of such an ambulance without making every reasonable effort to remedy the defects found at the time of delivery or within the warranty period will be notified that they are DEBARRED from submitting bids to this agency in the future.  This agency will not waste valuable time (more than once) trying to recover legal costs and deal with lost in-service time of new apparatus, working with vendors who are unresponsive to the needs of this agency.

Does your bid comply EXACTLY as written?        Yes _____  No _____

1.20	Exceptions to Specifications:  Any exceptions to these specifications indicated must be clearly pointed out.  Otherwise, it will be considered that items offered are in strict compliance with these specifications and the successful bidder will be held responsible for delivering a vehicle meeting these specifications.  Any exceptions must reference by Paragraph Number and Line and be explained in detail on a separate sheet marked "Exceptions".  Any bidder not complying shall not be considered as responsive.

Any proposal which indicates “No Exception” to any item on our proposal documents, and also includes a letter from the vendor taking major or total exception to our specifications or indicates they are quoting to provide their standard methods of construction without listing any differences in the manner described, will be immediately rejected as non-compliant and removed from further consideration.

Should a vendor offer a quotation in response to this proposal and not take explicit exceptions and attempt to offer a unit not in compliance with the exact specifications, the Purchaser reserves the sole right to issue a Purchase Order with specified penalties and/or invoke a Performance Bond.  Penalties for not providing vehicles(s) to the exact specifications, with approved exceptions or deviations specifically authorized by Purchaser, will be assessed.

2.0	TECHNICAL REQUIREMENTS - CHASSIS

Minimum technical specifications for customer providing all aluminum Type III Ambulance Module and subsequent remount to a 2024 or newer Ford E-450 4X2 DRW Cutaway Chassis.  Vehicle must comply with Federal and State laws and regulations and all safety regulations as applicable on date of delivery concerning automotive equipment and conditions and will be complete with standard equipment and all extra equipment as specified. Any reference to a particular brand is simply for specification purposes and is not intended to eliminate any brand of comparable quality, features, and design.

  - Manufacturer and Model Year: The chassis requirement for this specification is a 2023 or newer.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Model Type: The model type required is a Ford E-450 4X2 Dual Wheel cutaway chassis.

Does your bid comply EXACTLY as written?  	Yes____ No_____

2.3	Trim Level: Trim level shall be Preferred Equipment Group. Ford code 782A.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.4	Ambulance Prep Package: The chassis shall be equipped with the ambulance prep package, light/convenience group, Exterior upgrade and Interior upgrade packages. Ford codes 47A, 593, 18A, 18C.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.5	GVWR: The gross vehicle weight rating shall be 14,500 lb. Ford code 20F.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Axles and Suspension: The chassis provided shall be a 158.0" wheel-base, Heavy-duty front and rear springs with front stabilizer bar. Shock absorbers shall be heavy-duty. Rear axle is limited slip wide track with a ratio of 4.56:1.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Electrical: The vehicle shall be supplied with dual 12-volt batteries. They shall be standard OEM heavy duty batteries.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Electrical Generating System: The ambulance shall be equipped with OEM 225 amp alternator with internal voltage regulator.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Engine: Engine size shall be a 7.3L V-8 gas engine with heavy-duty engine.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Engine Block Heater: The chassis shall have an OEM engine block heater. This heater shall be wired to the 110V shoreline input on the modular body. Ford code 41H.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.11	Fuel Tank: The vehicle shall be equipped with a 55 gallon aft-axle fuel tank. Ford code 656.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.12	Tires and Rims: The vehicle shall be equipped with (7) LT225/75RX16E Radial black sidewall tires.  The OEM manufacturer's standard steel vented rims will be provided. Spare tire, wheel and tire changing tools will also be provided. Ford codes T67, 646.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.13	Transmission: The vehicle to be supplied with a 5-speed automatic transmission. Ford code 44T.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.14	Air Conditioning and Heater: The vehicle shall be equipped with factory air conditioning with heavy-duty cooling package and heavy-duty radiator. The heater shall be the manufacturer's standard.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.15	Glass: The chassis shall be equipped with the chassis manufacturer's standard tinted glass supplied as part of their Air Conditioning Cooling Package. The glass provided shall meet all the requirements of KKK-A-1822 and those of the Federal Motor Vehicle Safety Standards.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.16	Horns: The OEM manufacturer's dual electric horns shall be provided.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Mirrors: The vehicle's exterior mirrors shall be Ford OEM power trailer tow mirrors with built-in convex mirrors. 	Ford code 54E.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.18	Radio: The chassis manufacturer’s premium AM/FM/CD/Clock Audio-Stereo. Ford code 585.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.19	Seats: The Chassis shall be equipped with medium flint cloth Ford OEM Dual Cloth Captain’s Chairs with lap and shoulder belts for driver and passenger. Ford codes M, E, 782A, 18C.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.20	Speed Control and Steering Wheel: Cruise speed control and tilt steering to be supplied. Ford code 525.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.21	Convenience Packages: Chassis shall include convenience packages to include power door locks, power windows, interval windshield wipers, chrome grille/bumper, and composite headlights. Ford codes 782A, 593, 18A, 18C.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.22	Running Boards: Install new highly polished aluminum diamond plate running boards with integrated front splash 	shields. Running boards are to be properly reinforced to withstand many years of use without bowing or damaging 	the chassis body or ambulance module.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.23	Wheel liners: Install Phoenix stainless steel wheel covers.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Keyless Entry: Vehicle shall be equipped with a keyless entry/alarm key fob. Ford code 948.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Suspension: Suspension shall be modified as necessary to insure proper loading height.

Does your bid comply EXACTLY as written?	Yes____ No_____

  - Floor Covering: Vehicle to have Ford OEM vinyl floor covering.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.27 	Mud Flaps: The completed unit is to have custom mud flaps installed.

Does your bid comply EXACTLY as written?	Yes____ No_____

2.28	Undercoating: Professionally apply a petroleum based rustproofing type undercoating to chassis wheel well areas and apply where appropriate to bottom of modular body to soundproof and rustproof these areas.

Does your bid comply EXACTLY as written?       Yes_____No_____

3.0	REMOVAL AND TRANSFER OF MODULAR AMBULANCE BODY

3.1	Dismount: Disconnect all sub systems between chassis and module. Lift module to allow full inspection of understructure.

Does your bid comply EXACTLY as written?        Yes____No_____

3.2	Module Inspection: Inspect all exposed structure and framing for cracks, warpage, excessive wear and corrosion.  Any questionable areas shall be treated with dye penetrant material to expose cracks. Professionally repair as needed.

Does your bid comply EXACTLY as written?        Yes_____No_____

3.3	Type Changeover: Transfer the existing Medix module from Type III Ford to a Type III Ford. Modify suspension and bottom of module as necessary to properly accommodate the new chassis.

Does your bid comply EXACTLY as written?       Yes_____No_____

3.4	Module Remount: Mount down module squarely on chassis.  Remounter to use Ford OEM rubber isolators, mounting bolts and transfer body per OEM chassis body builder guidelines. The use of aftermarket or substandard components is strictly prohibited.

Does your bid comply EXACTLY as written?       Yes_____No_____

3.5	Cab Seal: Install a non-electrolytic black hard rubber seal between module and cab. Seal interface to eliminate air and water leaks. New stainless steel bolts will be used to fasten seal to cab and module body.

Does your bid comply EXACTLY as written?          Yes_____No_____

3.6	Trade-In: The existing Ford E-450 chassis will be traded-in and the final price shall include this credit allowance. This chassis currently has serious engine problems and will be towed to the successful vendor by Marshall County.

Does your bid comply EXACTLY as written?           Yes_____No_____

4.0	NEW CHASSIS PREPARATION

4.1	Springs: As necessary, level each side of modular body to insure proper handling and performance.

Does your bid comply EXACTLY as written?          Yes_____No_____

4.2	Exhaust Modification: Extend tailpipe to match module width.

Does your bid comply EXACTLY as written?          Yes_____No_____

4.3	Fuel Tank: Modify gas tank filler necks to fit module. Install new fuel filler hoses and copper plumbing per OEM requirements.

Does your bid comply EXACTLY as written?          Yes_____No_____

5.0	EXTERIOR BODY TRIM

5.1	Fuel Fill: Install new Cast Products fuel filler housing.

Does your bid comply EXACTLY as written?      Yes_____No_____

5.2	Diamond Plate: Install all new aluminum diamond plate complete on module body. This will include rub rails, running boards, and corner stone guards.

Does your bid comply EXACTLY as written?     Yes_____No_____

5.3	Rear Bumper: A new all aluminum rear bumper assembly, with center flip-up Grip-Strut, including new aluminum diamond plate rear kickplate will be installed. This bumper assembly shall be attached to the chassis frame rails only and shall in no way contact or be mounted to the module.

Does your bid comply EXACTLY as written?     Yes_____No_____

5.4	Fenderettes: New stainless steel fenderettes shall be installed.

Does your bid comply EXACTLY as written?     Yes_____No_____

5.5	Drip Rails: Install new polished satin finished drip rails above all patient entry doors and all exterior compartments.

Does your bid comply EXACTLY as written?     Yes_____No_____

5.6	Door Hinge: Polish existing stainless steel door hinges and reinstall.

Does your bid comply EXACTLY as written?    Yes_____No_____

5.7	Weather-strip: Install new patient entry and compartment door weather-strip with identical OEM type.

Does your bid comply EXACTLY as written?   Yes_____No_____

6.0	MODULE EXTERIOR PAINT FINISH AND LETTERING

6.1	Preparation: Prior to any paint and bodywork remove all exterior compartment and entry doors, all hardware, hinges, handles and emergency lighting.

Does your bid comply EXACTLY as written?     Yes_____No_____

  - Surface Refinished: In order to provide the highest quality paint surface preparation, and to evaluate the module for evidence of electrolysis, complete and total mechanical removal of all paint, oils, and contaminants from module surface utilizing a dry stripping system is required.

Does your bid comply EXACTLY as written?       Yes_____No_____

6.3	Electrolysis Inspection: Module will be inspected 100% for evidence of electrolysis.  Such areas will be specially treated to prevent any further damage and remove visible damage.  NO EXCEPTION.

Does your bid comply EXACTLY as written?      Yes_____No_____

6.4	Damage Repair: All minor body damage will be repaired in an industry standard professional manner to like new condition.  NO EXCEPTION.

Does your bid comply EXACTLY as written?     Yes_____No_____

6.5	Paint Process: The module body shall be repainted in PPG single-stage paint process. Color of module to be YZ matching the OEM chassis color exactly.  NO EXCEPTION.

Does your bid comply EXACTLY as written?      Yes_____No_____

6.6	Vinyl Striping: Orange reflective stripe shall be installed per TN and federal kkk requirements. This orange stripe shall be installed in a professional manner with no pinstripe needed to cover imperfections. This horizontal stripe shall be approximately 10” wide and transition into an approximately 6” wide Chevron pattern on the rear wall. No pinstripe will be allowed on this unit.

Does your bid comply EXACTLY as written?     Yes_____No_____

6.7	Lettering:  Provide and install customer specific lettering. This new lettering will be exactly the same as on our currents. Clarity and direction will be given to the successful vendor by this agency regarding exact style and placement. Install kkk, DOT, Ambulance and Star of Life decals per kkk regulations and existing layout. All custom lettering to have a ¼” white reflective border around the blue letters. The unit number for this ambulance will be #45.

Does your bid comply EXACTLY as written?  Yes____ No_____

6.8	Chevron:  The rear wall of the module shall have approx 6” orange reflective stripes, alternating with approx 6” white painted stripes, in a Chevron pattern installed. This reflective stripe shall be installed prior to any components being installed for a cleaner and more professional installation. This includes door handles, windows, lights, kickplate, door stops, etc. This pattern shall cover the entire rear of the module from the kickplate to the roofline drip-rail vertically and between both corner extrusions horizontally.

Does your bid comply EXACTLY as written?	Yes____ No_____

6.9	Window Film:  White perforated window privacy film shall be installed on all entry door windows to inhibit vision into the module. The rear windows will also have the 14” Star-of-Life decals installed on this window film.

Does your bid comply EXACTLY as written?	Yes____ No_____

7.0	ELECTRICAL SYSTEM:

7.1	Wiring Installation: All new power wiring provided shall be copper and conform to all the SAE J1292 requirements and shall have GXL or SXL high temperature thermoplastic or better insulation rated to 125 degrees centigrade and conforming to SAE J1127 and J1128

Does your bid comply EXACTLY as written?   Yes_____No_____

7.2	Wiring Labeling: All wiring shall be permanently color-coded and labeled for easy troubleshooting.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.3	Wiring Loom: The engine wiring shall be routed in conduit or high temperature looms with a rating of 300 degrees Fahrenheit.  All added wiring shall be in protective loom, located in accessible, enclosed and protected locations, and kept at least six inches away from the exhaust system components.  Wiring and components shall not terminate in the oxygen storage compartment except for the compartment light and door switch plunger.  Wiring necessarily passing through an oxygen compartment shall be routed in metal conduit.  All conduits, looms and wiring shall be secured to the body or frame with insulated metal straps in order to prevent sagging and movement which results in chafing, pinching, snagging, or any other damage.  All apertures on the vehicle shall be properly grommeted for passing wiring and conform to SAE 1292.

Does your bid comply EXACTLY as written?  Yes_____No_____

7.4	Power Disconnect: A solid-state, high-amperage power disconnect switch shall be installed to control power to the ambulance module and the related components. This disconnect shall be controlled by the chassis ignition circuit and shall in no way control any power to the OEM chassis or related circuits. This disconnect shall also be capable of monitoring amperage and recognize an abnormally high amperage. If a high amperage situation is seen by the power disconnect, it will automatically terminate power to the module to prevent damage to any components. Once the problem has been repaired, this disconnect shall be capable of being reset and returned to service. An adjustable timer shall be incorporated into this disconnect to allow power to continue to the module for a period of time after the chassis ignition is turned off. This timer shall be adjustable from 0-20 minutes and set per the customer’s desire. This does not currently exist but shall be incorporated into the design of the new chassis electrical system.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.5	Cab Wiring: Install new power distribution harness and battery cables.

Does your bid comply EXACTLY as written? 	Yes_____No_____

7.6	Engine Block Heater: Install OEM engine block heater to shoreline.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.7	Front Cab Console: Install a new contoured floor mount console to house switch panel, radios, and storage area.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.8	Siren Amplifier: The existing Whelen siren amplifier shall be tested for proper function. If this amplifier is faulty, it shall be brought to the attention of this agency so a decision may be made regarding its replacement.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.9	Back up alarm: Install a Velvac 97db OSHA backup alarm in a protected location.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.10	Spotlight: Install a 300,000 cp hand held spotlight with coiled cord and momentary switch.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.11 	Front Switch Panel: Install a new custom front switch panel complete with analog voltmeter and ammeter.

Does your bid comply EXACTLY as written? 	Yes_____No_____

7.12	Power Distribution System: Check for 100% complete operation.  Any compromised components will be replaced. The remounter shall provide a three (3) year / 36,000 mile electrical warranty.  NO EXCEPTIONS!

Does your bid comply EXACTLY as written?   Yes_____No_____

7.13	Taillights: The existing Whelen 700 series LED stop/tail/turn lights shall be reused if found to be functioning properly. If new lights or components are needed, it shall be brought to the attention of this agency so a decision may be made regarding replacement(s).

Does your bid comply EXACTLY as written?   Yes_____No_____

7.14	ICC Lights: Install new LED ICC lights.

Does your bid comply EXACTLY as written? 	Yes_____No_____

7.15	Scene Lights: Install four (4) Whelen #9SC0ENZR 900 Series LED Scene Lights.

Does your bid comply EXACTLY as written?  Yes_____No_____

7.16	Load Lights: Install two (2) Whelen #9SC0ENZR 900 Series LED Load Lights on rear of module.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.17	Intersection Lights: Provide and install Whelen 700 Series Red LED intersection lights with clear lenses and chrome flanges.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.18	Grille Lights: Provide and install two (2) new Whelen 500 Series LED red with clear lens grille lights in polished Cast grill housings.

Does your bid comply EXACTLY as written?  Yes_____No_____

7.19	Siren Speakers: Provide and install new Cast Products, # SAD/P 3801-0835-1 siren speakers recessed into the front bumper.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.20	Warning Lights: The existing warning lights shall be inspected for proper function and reused if found to be free of defects. Any broken or compromised lights or components shall be repaired or replaced as needed.

Does your bid comply EXACTLY as written?   Yes_____No____

7.21	Door Switches:  Install all new patient entry and compartment door switches.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.22	Inverter: Thoroughly inspect the inverter/charger for proper function. If a problem is found, it must be brought to the attention of this agency and a decision will be made regarding the replacement or repair of this inverter.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.23	Dome Lights: The existing dome lights shall be inspected for proper function and reused if found to be free of defects. Any broken or compromised lights or components shall be repaired or replaced as needed.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.24	Air Horns: The existing air horns and air horn system shall be transferred to the new chassis. This system shall operate from a 3-position console mounted switch activated by the OEM horn ring circuit. This switch shall have a horn, siren, and air horn position and shall be properly labeled. This entire system shall be installed in a professional manner. Any broken or compromised lights or components shall be repaired or replaced as needed.

Does your bid comply EXACTLY as written?   Yes_____No_____

7.25	Reverse/Rear View Camera:  The existing rear view camera and monitor system shall be transferred to the new chassis.This entire system shall be installed in a professional manner with all cables routed in a waterproof, durable, and secure manner. If a problem is found, it shall be brought to the attention of this agency so a decision may be made regarding replacement(s).

Does your bid comply EXACTLY as written?   Yes_____No_____

8.0  	EXTERIOR COMPARTMENTS

8.1	Weather-stripping: Replace all damaged exterior rubber door weather-stripping with original or equivalent seal.

Does your bid comply EXACTLY as written?   Yes_____No_____

8.2	Compartment Door: Compartment doors are to be closely inspected for condition and repaired as necessary.

a.	Checked and trued for alignment and closure

b.	All hold opens checked for operation and defective closures replaced

c.	Lubricate all door latches and locks for proper operation

d.	Replace any defective latches or locks to insure 100% operation

e.	Any damaged windows replaced with like OEM windows from OEM manufacturer

Does your bid comply EXACTLY as written?   Yes_____No_____

8.3	Compartment Interiors: Shall be repaired, refreshed, and cleaned as needed. All compartments shall be repaired, refinished, and repainted with Scorpion as needed. This shall be done in a professional manner using OEM materials.

Does your bid comply EXACTLY as written?   Yes_____No_____

PATIENT COMPARTMENT CABINETRY AND FIXTURE:

9.1	Cabinetry:  Cabinets will be checked for sharp edges, defective areas, and professionally repaired including the following items:

a.	Trim out interior to a like new condition

b.	Manually check cabinet mounting bolts and gas cylinders to insure stability, strength, and like new operation

c.	Check all shelves for security and make rattle proof

d.	Check all seatbelts and replace all defective ones

e.	Replace any damaged or missing trim.

f.	Replace any rusty or discolored screws or trim

g.	Provide high quality clean up and sanitation to all cabinetry, compartments, headliner, etc.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.2	Subfloor:  The existing sub-floor shall be reused.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.3	Flooring: The existing floor covering shall be reused.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.4	Interior trim: Trim out interior to new condition.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.5	Thresholds:  Install stainless steel side and rear thresholds to trim out floor at side and rear doors.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.6	Upholstery: All upholstery in the patient compartment shall be inspected and replaced as needed. This will include all seats, backrests, head-knockers, trims, etc. It is known that the EMT/captain’s chair needs a new seat bottom.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.7	Cot Mount: The existing Stryker PowerLoad cot mounting system shall be reused and properly  reinstalled.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.8	Formica Repair:  Replace any damaged Formica as needed. Materials and workmanship shall be of OEM quality matching in both texture and color.

Does your bid comply EXACTLY as written?   Yes_____No_____

9.9	Safety Net:  Inspect Safety net at the head of the squad bench and replace as needed.

Does your bid comply EXACTLY as written?   Yes_____No_____

10.0	HEATING AND AIR CONDITIONING

10.1	Heat/Cool Unit:  Install new Pro Air Super Core Heat/Cool unit with wall mounted thermostat.  System shall be complete with all-new hoses and fittings.

Does your bid comply EXACTLY as written?   Yes_____No_____

10.2	Heat/Cool Testing:  System shall be extensively tested for peak performance and readings shall be provided to customer.  Existing vent openings (patient compartment) shall be replaced as needed.

Does your bid comply EXACTLY as written?      Yes_____No_____

10.3	Power Vent:  Shall be tested and repaired or replaced to meet original equipment performance.

Does your bid comply EXACTLY as written?   Yes_____No_____

11.0	OXYGEN AND VACUUM SYSTEMS

11.1	Oxygen System:  Perform 4-hour nitrogen leak test.  Repair any leaks noted and re-test and tag system with certification.

Does your bid comply EXACTLY as written?   Yes_____No_____

  - Oxygen Outlets:  Install all new O2 outlets and perform leak test and seal as needed.

Does your bid comply EXACTLY as written?   	Yes_____No_____

  - Suction System:  Test vacuum lines, outlets for proper operation and repair or replace as necessary.

Does your bid comply EXACTLY as written?   Yes_____No____

##### Marshall County EMS

##### BID FORM

NAME OF BIDDER:

ADDRESS:

CITY/STATE/ZIP:

TELEPHONE:

PERSON TO CONTACT:

We herewith submit and bid as follows:

CHASSIS/MAKE:

MODEL/TYPE:

AMBULANCE BRAND:

F.O.B. POINT:

DELIVERY – CALENDAR DAYS:

TOTAL PRICE COMPLETE, AS SPECIFIED…………$

BID IS:		      AS PER SPECIFICATION, TAKING NO EXCEPTIONS

______      TAKING ONLY THOSE SPECIFICATION EXCEPTIONS

LISTED, ATTACHED AND REFERENCED TO PARAGRAPH

NUMBER SPECIFIED

It is agreed by the undersigned bidder that the signature and submission of this bid represents the bidder’s acceptance of all terms, conditions, and requirements of bid specifications and, if awarded, the bid will represent the agreement between the two parties.

SIGNED:____________________________________  DATE:__________________________________

NAME PRINTED:____________________________  TITLE:__________________________________

## EXCEPTIONS / CLARIFICATIONS

Each bidder may copy this form, as necessary to sufficiently list all exceptions and variations from specifications (Please list as shown, by page, item number, and check if vendor chooses not to supply, or is unavailable, or describe deviation or substitution in detail, if furnished).  Purchaser will be the sole judge of proposed substitution equivalency.

VENDOR NAME:

BIDDING:

EXCEPTION PAGE:		______________________     OF     ______________________

SPECIFICATION PAGE:        REFERENCE #	      NOT AVAILABLE:          EXPLANATION: