Pierce County

Board of Commissioners

REQUEST FOR PROPOSAL

CULVERT REPLACEMENT/CASON ROAD

BLACKSHEAR, GEORGIA

Prepared by:

Pierce County Board of Commissioners

312 Nichols Street

Blackshear, GA 31516

(912) 449-2022

- Request for Proposal

The Pierce County Board of Commissioners (COUNTY) is seeking proposals from qualified and appropriately licensed contractors to excavate and replace approximately eighteen (18) culvert pipes along Cason Road from the Ware County line to the Blackshear City limits. The recommended installation shall use Polymer Coated CMP pipes with Aluminized connecting bands. This project will work ahead of the proposed Georgia Department of Transportation Project #0016124 which was issued a Notice to Proceed on August 30, 2019.

All proposals are to be submitted in sealed envelopes marked “CULVERT REPLACEMENT/CASON ROAD—DO NOT OPEN” addressed to Pierce County Board of Commissioners. Attn: Mamie Jackson, P.O. Box 679, 312 Nichols Street, Blackshear, GA 31516 until 10:00 A.M. on Thursday, December 5, 2019 at which time they will be publicly opened. Copies of the Request for Proposal with specifications and forms can be obtained at the Pierce County Board of Commissioners Office.

- Property Description

The locations of the culvert pipes needing to be replaced are depicted in Exhibit “A” of this RFP. There are eighteen (18) pipes needing to be replaced throughout the entire length of Cason Road from the Blackshear City limits extending to the Pierce/Ware County line.

- Scope of Work

  - Current GDOT and FHWA specifications shall be met for all materials and equipment involved with this work unless otherwise specified.

  - Contractor will be responsible for working with Pierce County and East Coast Asphalt to ensure that all work and road closures will not inhibit the progress of the GDOT project nor compromise public safety.

  - Contractor will be responsible for all necessary flaggers, signs, barricades, and other safety devices necessary to control and direct traffic in accordance with the MUTCD manual and GDOT Best Management Practices.

  - The Contractor will be required to meet all laws, rules and regulations of the State of Georgia during the course of this project. “Georgia 811, Call Before You Dig” location service notification and requirements shall be the responsibility of the Contractor unless delegated to the County in writing by mutual agreement.

  - Erosion control shall be the responsibility of the Contractor.

  - Culverts shall be at least a Polymer Coated Corrugated Metal Pipe with Rolled Ends as applicable. Band shall be at a minimum 18” Aluminized Connecting Bands. Larger bands may be required as determined by the manufacturer or other specifications for each respective location. Culverts shall be installed to maintain positive drainage and shall match the flowline of the ditch or drainage basin. All pipes shall be properly spaced, spacing equal to the diameter of the pipe up to 36 inches. All pipes greater than 36 inches shall remain 36” apart unless engineering requirements determine otherwise and are provided to the Pierce County Road Superintendent in writing prior to installation for approval. A minimum of 12 inches of #5 stone shall be used for bedding under the pipe and for backfill up to ½ the diameter of the pipe. Pipes shall be set using compacted native material and a minimum of 12” of cover is required (2-foot preferred) unless approved by the Road Superintendent of Pierce County.

  - Culverts shall be sized in accordance with current Best Management Practices. A minimum pipe slope of 0.5% to allow for positive flow and self-cleaning shall be maintained.

  - Culvert pipes shall extend at least five (5) feet beyond the toe of road bank slopes.

  - Outlets shall be protected with rock, aprons, plunge pools, drains, geo-textiles, or discharge to stable vegetative areas.

- Additional Requirements

  - Bonds: Contractor will be required to provide a Payment Bond and Performance Bond no later than seven (7) days after receiving Notice of Contract Award. Both the Payment Bond and Performance Bond will be in the amount of one hundred percent (100%) of the Contract price.

  - Minimum Insurance Coverage: Contractor will be required to maintain in force a policy or policies of insurance written by one or more responsible insurance carriers licensed to do business in the State of Georgia that shall insure against liability for injury to and/or death of and/or damage to property of any person or persons, with policy limits of not less than the following: $1,000,000.00 combined single limit for injury to or death of any number of persons or for damage to property of others arising our of any one occurrence and $1,000,000.00 combined aggregate and umbrella coverage for any and all actions of Contractor based on the Operations to be provided and shall name, as additional insured and loss payee, the COUNTY.

  - Worker’s Compensation: Contractor will be required to provide proof of worker’s compensation insurance in coverage amounts required by the State of Georgia for the Operations to be provided that covers Contractor’s employees for any on-the-job injuries. Contractor must be willing and able to provide evidence of insurance not later than seven (7) days after receiving Notice of Contract Award.

  - Hold Harmless and Indemnification: Contractor agrees to indemnify, defend, and hold the COUNTY, its officers, employees, agents and assigns harmless from any judgements, liabilities and associated costs for bodily injury, death or damage to property resulting, in whole or part, from any and all acts or omissions of Contractor, its officers, employees, agents and assigns.

- Fee Basis

This project will be paid on a lump sum basis. The payment will be made in accordance with the terms and conditions of the contract at the end of the project once all work has been completed by the Contractor and approved by the COUNTY. A payment retainage of ten percent of the total project cost will be held by the COUNTY pending satisfactory completion of all project tasks and a final inspection of all sites.

- Submittal Requirements: Respondents are encouraged to have attended the optional pre-submittal meeting and site inspection as described in paragraph III.

  - Respondent submittals will include:

    - Respondent qualifications and experience to complete the project.

    - Letter confirming current bonding capabilities.

    - Copies of all current licenses required to complete the work.

    - Proposed work plan, level of effort, and timeline required to complete the project.

    - Identification of any work to be performed by sub-contractors. Include names of sub-contractors and copies of their current licenses.

    - Complete and signed “COUNTY RFP Submittal 19-11 Form”.

  - Number of submittal copies: Five (5)

- RFP submittal delivery method:  Registered, certified mail, overnight delivery service (UPS/FedEx), or hand delivery in a seal envelop marked “CULVERT REPLACEMENT/CASON ROAD—DO NOT OPEN”. Submittals using facsimile transmission and/or bids that are not sealed as described above will not be accepted.

- SUBMISSION DEADLINE: December 5, 2019 10:00 AM EDT. Any submittal received after the date and time specified herein shall not be considered.

- Submit documents to:

Pierce County Board of Commissioner

Attn: Mamie Jackson

PO Box 679

312 Nichols Street

Blackshear, GA 31516

- Questions/clarifications: Respondents may submit questions or requests for clarification up to the submission deadline. Answers to all questions will be emailed to all other contractors who have submitted a bid or have attended the optional pre-submittal meeting and site inspection that have provided their appropriate contact information.

Email: mamie.jackson@piercecountyga.gov

Phone: (912) 449-2022

Fax: (912) 449-2024

- Evaluation Criteria:

  - The following evaluation criteria will be used to select the successful Respondent:

    - Submittal details and completeness of information provided

    - Proposed plan and level of effort required to complete Project

    - Complete bid price

- Contractor Selection:

The COUNTY reserves the right to reject any or all proposals received, request additional information, use the information in any matter deemed to be in the best interest of the COUNTY, utilize outside assistance in evaluating proposals, waive informalities, and undertake the described work in a manner other than that set forth herein as applicable law allows.

- Additional Information:

  - This RFP is not intended as a formal offering for the award of a contract or for participation in any future solicitation.

  - All submissions shall become the property of the COUNTY and shall not be returned. Upon execution of a binding contract, all submission shall become a matter of public record and regarded as public records except for those parts of each submission defined by Respondent as business or trade secrets provided that said parts are submitted in a separate, sealed envelope and clearly marked as “trade secret” or “proprietary”.

  - The COUNTY shall not be liable for any costs incurred by any respondent in the preparation, submittal, presentation, or revision of its submission.

Pierce County Board of Commissioners

Request for Proposal: Culvert Replacement/Cason Road

(include in RFP Submittal Package)

Project: Replacement of eighteen (18) culvert pipes on Cason Road extending from the Blackshear City limits to the Ware County line.

Property locations: Pierce County (as depicted in Exhibit 1)

Blackshear, Georgia 31516

Submitting Firm:

Address:

Cost of Culvert Pipes and Material for Installation: $

Cost of Labor for Installation of Culvert Pipes: $

Total Project Cost: 					$

Authorize Signature:						Date:

Email:									Phone:

Firm Pricing

Submitted bid are firm through (minimum 60 days):

Date:

EXHIBIT 1:

See six (6) pages of attached images with approximate pinned locations of pipes