Tandem Dump Truck Specifications

ITB #2021-07-014 ROAD DEPARTMENT

|  | MODEL AND YEAR, MINIMUM SPECIFICATIONS |
| --- | --- |
| Notes to Bidder: | Dump trucks are to be used for transporting dirt and gravel for road constructions. |
|  | Truck, Conventional Cab and Chassis, Tandem Axle, Manual Transmission, with GVWR 54,320 minimum with dump body for this class truck chassis, |
|  | ALL SPECIFICATION CONSIDRED MINIMUM |
| Equipment | GENERAL Description |
| Year model | New or Used Model 2012 or Newer |
| Engine | Diesel 390 Hp Minimum |
| Aspiration | Turbo Charged with Air-to-Air intercoolers Filter Spin On Type |
| Engine retard | Not required |
| Transmission | MAN-8LL |
| Torque | Minimum 1100 pound |
| Clutch | Required |
| Front axle | 14,300 LB |
| Rear axle | 40,000 LB |
| Rear axle ratio | 4.11. Required OR Rated for this configuration |
| Rear axle spacing | Vendor shall work with body builder on mounting distance |
| Front springs | Air track system |
| Power Steering | Power Steering |
| Suspension rear | 40,000 LB |
| Frame | Truck Manufacturer and Body Builder Recommendation |
| Dump Truck Body | 16-yard capacity Steel construction Telescopic Hoist Asphalt lip on back Air latch tailgate Black in color |

CONTINUED

| Air dryer | Heated Air dryer. Unit(s) shall meet requirements of truck manufacturer for proper operation of air system, brake system, number of axles, and weight rating of truck." |
| --- | --- |
| Air compressor | Unit(s) shall meet requirements of truck manufacturer for proper operation of air system, brake system, number of axles, and weight rating of m truck. |
| Air tank | Unit(s) shall meet requirements of truck manufacturer for proper operation of air system, brake system, number of axles, and weight rating of with manual drain valve truck. |
| Brakes | Air Cam-Type Abs With Automatic Slack Adjusters |
| Parking brake | Cab Controlled |
| Cab | MFG Standard Conventional |
| Cab to include | Minimum (1) cup holder, dome light, MFG standard, 1 ea. grab handle on each side, steps - both sides |
| Cab trim | To include MFG standard cab trim panels, door trim panels required, headliner to be soft padded cloth or vinyl, flooring to be heavy duty vinyl or rubber, sun visors required driver and passenger padded vinyl |
| Seat driver | Air suspension, high back with integral headrest with fore, aft and height adjustments and recline. Cloth with dual armrest with 3-point seat belt. |
| Seat passenger | Non suspension, high back, with integral headrest, vinyl, fixed back, dual armrest with 3-point seat belt, lap and shoulder belt type. |
| Gauge cluster/ instrument panel/data center | Gauge cluster includes engine oil pressure (electronic) water temperature (electronic), fuel (electronic), tachometer (electronic), voltmeter, odometer display, miles, trip miles, engine hours, fault code readout: warning system low fuel, low oil pressure, high engine coolant temp, and low battery voltage (visual and audible): fuel economy, trip information, diagnostics, warning alarm will sound when headlights are left on |
| Mirrors | (2) Power Adjustable Rectangular |
| Air conditioning/ heat | Factory installed w/ integrated defrost |
| Grill/ hood | Conventional tilt |
| Exterior color (WHITE) | Fleet White |
| Steering column | Tilt |
| Exhaust | MFG Standard Vertical Tailpipe To Include Turn Out And Guard |
| Electrical | 12-Volt, Standard Equipment |

CONTINUED

| To include | Mfg. standard data link connector for vehicle programming and diagnostics in cab, hazard switch push on /push off, head light dimmer switch w/ turn signal lever, horn electric single, starter switch electric key operated, windshield wiper switch 2- speed with wash and intermittent with MFR standard wind shield wipers |
| --- | --- |
| Tires | 11r 24.5 Ply Steer Tires |
| Headlights | Sealed Beam |
| Marker lighting control | Not required |
| Clearance/marker | (4) truck lite amber factory installed on cab or sunshade |
| Stop, turn, tail & b/u | Flange mounted with two backup lights |
| Power outlet | n/a |
| Alternator | 12 Volt 130 Minimum |
| Batteries | 2500cca 2 batteries minimum |
| Jump start stud | On battery under step |
| Battery box | MFR standard |
| Radio | AM/FM with weather band, clock w/ two dual cone speakers in cab. |
| Cb | Power connections one pair on dash |
| Horn | Air Solenoid Operated Single or Dual Acceptable Please Consider Location Due To Cab Shield For Dump Body |
| Backup alarm | 87-112 decibels, adjusts automatically per OHSA |
| Engine block heater | Not required |
| Air cleaner | Dry type, single element |
| Fuel/water separator | Required |
| Fuel tank | Minimum 70-gallon aluminum tank |
| Def tank | Factory spec |
| Wheel’s front | 24.5 steel wheel, hub piloted |

END