Scope of Work

Matilija Middle School

Kitchen and Dining Project # 2020-1604

Paving at Kitchen / Dining Hall surround:

- Remove all asphalt per plan (Page 2 OH View).

- Cut and remove intruding surface roots to a depth of 12 inches as needed.  Note repairs needs at areas on driveway entrance where former tree wells have left soft areas and depressions.

- Regrade areas on West, North and East to allow proper drainage to the south per C 1.1

- Regrade areas to the West and East sides to allow proper drainage from the new asphalt to the Concrete at the southern edges.

- Fine grade, recompact existing, and install new subgrade where necessary to a minimum depth of 6 inches.

- Blend new paving to existing pavement. Apply tack oil at vertical surfaces of existing paving.

- Abut new paving at concrete edges.

- Lift any utility vaults to match new asphalt and install new vault covers.

- Pave areas between concrete pads at south side of Building C.

- Install 3 inches (after compaction) of hot mix asphalt pavement.

- Re-stripe the asphalt surface per new district provided layout using 2 coats of traffic paint.

- Comply with ADA signage per AS 1.0 /1,10,11, Path of Travel striped per AS 1.0/12, Truncated domes per plans (3 – 3’ x 5’) and specifications on AD 6.1

- Remove fencing at bike storage area fencing for paving and replace upon completion