SPECIFICATIONS FOR Auger Boring Machine & Augers

EXCEPTIONS: The Bidder must list any exceptions (major & minor) to the specifications. The Bidder must prove "an equal" in writing with literature. This information must be in the bid package and cannot be supplied at a later date. Failure to list all exceptions either in the space provided or on a separate page will be cause for rejection

DETAILS

Boring Range of 4" to 24" casing

Engine - minimum 31 HP Diesel, Air-cooled @ 2800 rpm

Transmission minimum 3 forward speeds and 1 reverse

Clutch - Operator presence clutch

Final drive - Dual planatery 2.5" Hex

Auger torque - minimum 8512 ft/lbs peak 1st gear

Forward thurst - 115,000 lb @ 3,000 psi

Dog plate - Hydraulically activated from control console

Hydraulic perameters - Gear pump max pressure 3000 psi x 14.5 gpm

Fuel tank capacity - 5.0 gallons minimum

Carriage 2900 lbs maximum

Auger flight - 12" x 20 foot on 2.25

Operator Station opposite side of spoil chute/ejector

12" Adapter Kit

Auger 12" OD 2-1/4" Hex W.C

Head, Rock 12" OD 2-1/4" Hex W.C

Head, Combo 12" OD 2-1/4" Hex W.C