SHIP HOUSING REPAIR

ESCAMBIA COUNTY

	OWNER:
	Leslie E. Gilmore
	
	JOB #:
	S-R-2019-5

	ADDRESS:
	620 W. Hernandez Street
	
	DATE PREPARED:
	01/28/2019

	
	Pensacola, Florida 32501
	
	OPENING DATE:
	02/08/2019

	PHONE: 
	850-332-7858
	
	CLOSING DATE:
	02/22/2019

	
	
	
	CLOSING TIME:
	12:00 noon


I/(we) certify that I/(we) have carefully examined the Housing Repair Specifications and Contractor Requirements related to the above referenced job, as furnished by Escambia County, and I/(we) have also examined the site on which the proposed work is to be performed. On the basis of these examinations, I (we) propose to furnish all materials, tools, machinery and labor necessary to complete the work in a professional, workmanship manner and to complete the work at the prices listed.

	$
	
	

	TOTAL JOB COST
	
	CONTRACTOR’S SIGNATURE

	
	
	
	

	
	
	
	TITLE

	
	
	
	

	BID OPENING DATE:
	
	
	FIRM

	 FORMCHECKBOX 
 ACCEPTED
	 FORMCHECKBOX 
 REJECTED
	
	

	
	
	
	FIRM PHONE NUMBER

	
	
	
	

	BID COMMITTEE REPRESENTATIVE
	
	


REQUIRED PERMITS AND INSPECTIONS:                                                                                          
WINDOWS, DOORS, FRAMING, MECHANICAL, GAS and PLUMBING
	Type of Construction:
	Wood frame
	

	Home was built in 1942

	Hallway:
	17'6”x7'x8'
	Bath:
	5’x7'x8'


All measurements are for reference only and should be confirmed by the bidder

Mandatory On Site Pre-Bid Conference: February 15, 2019 at 8:30 a.m.
INSTRUCTIONS TO BIDDERS
Submit COVER SHEET AND ALL NUMBERED PAGES OF SPECIFICATIONS with signature of Contractor and bid price in ink. Specifications in pencil will be rejected.  Bids must be in a sealed envelope and hand-delivered to Neighborhood Enterprise Division Office, 221 Palafox Place (Ernie Lee Magaha Government Building), Suite 200, Pensacola, FL 32502 office by 12:00 noon at the closing date specified. NO email or fax submissions will be accepted. 

The bids will be opened by the Bid Committee and bid will be awarded based upon the lowest eligible bidder with the most reasonable combined bid for the work specified. Itemized pricing may be used for negotiation in the event of a change in the scope of work as described within the Specifications.

Labor, overhead, permits, insurance, and profit must be included into each itemized price and not listed as a separate itemized price or listed under “miscellaneous.” Non-compliance will result in rejection of bid.

All repair work performed must be inspected (rough and final) and conform to County Ordinances, State Laws, and Florida Building Code.

The Contractor is responsible for proper reattachment/hook-up of any/all of the following that apply: appliances, existing coaxial cable, telephone and interface, electric, water, and sewage.

HOMEOWNER’S RESPONSIBILITIES AND OBLIGATIONS

The Homeowner is responsible for maintaining existing utilities for the Contractor’s use during the rehabilitation period.

The Homeowner is responsible for the removal of all belongings/furnishings from the designated work areas prior to the start date.  The Contractor is not responsible for removal of Homeowner’s trash and/or discarded belongings or furnishings.

PAYMENT SCHEDULE

Payment will be lump sum at project completion for bids $15,000.00 and under.  For bids over $15,000.00, contractor will be eligible for a 40% draw once all required permit applications are in place and 50% of the work is completed as determined by the itemized costs for the job.
The Contractor will be eligible for Final Payment after the following conditions have been fulfilled:


Completion of Write-Up



Inspection/Sign-off by Building Inspections and/or Health Department



Acceptance by the Homeowner (Homeowner’s Release & Warranty) 



Warranty packet for all covered items provided to NED Staff 



Premises free from all construction debris



Submission of RRP Checklist Form & Contractor’s Final Affidavit


Original Invoice from Contractor
                       Surety’s Consent to Final Payment if applicable

COMPLETION DATE

There is a Thirty (30) day time limit on each rehabilitation job. For every day worked in excess of the Thirty day contract period, a FIFTY DOLLAR ($50.00) per day penalty will be assessed.

ESCAMBIA COUNTY NEIGHBORHOOD ENTERPRISE DIVISION

HOUSING REPAIR PROGRAMS

CONTRACTOR REQUIREMENTS

If your firm is awarded the bid for housing repair (SHIP or HUD programs), the following items must be current and on file in order to issue a Purchase Order. 


*Current W9 (less than one year old).

*County Vendor Information sheet (less than one year old).

*Worker’s Compensation as required by State Law OR exemption form.


*Commercial General Liability ($1,000,000 coverage) listing Escambia County as 
Certificate Holder and additional insured. 

*Automobile Liability ($1,000,000 coverage) listing Escambia County as Certificate 
Holder and additional insured. 

GL and Auto Liability Certificates must state the following under the Additional Interest: 

Escambia County

C/O Neighborhood Enterprise Division

221 Palafox Place 

Suite 200

Pensacola, FL 32502-5844
All correspondence should be directed to Neighborhood Enterprise Division, 221 Palafox Place, Suite 200, Pensacola, FL 32502 for review.






*For jobs valued at $25,000 or higher, 100% Performance and Payment Bonds will be required. Performance and Payment Bond format will be provided. 

Contractor will be responsible for the recording of all bonds with Pam Childers, Clerk of The Circuit Court of Escambia County Florida.
Refer to BCC Office of Purchasing Risk Management Guidelines Procedure No: PP-180 (Performance and Payment Bonds) and PP-185 (Risk Management Guidelines) for further details. [http://www.myescambia.com/business/purchasing-policies-and-procedures]
Additionally, Neighborhood Enterprise Division will maintain the following info in the Contractor’s file:


*Contractor may not be federally debarred from participating in programs as per 
www.sam.gov 

*State registered or State Certified Contractor License

*Escambia County Competency Board License

*Escambia County Business/Occupational License 

*EPA RRP Renovator and Firm Certifications

Firms bidding on Lead Based Paint Abatement jobs must also submit proof of Lead Abatement firm certification.    

All information must be current at time of award. IF INFORMATION CANNOT BE PRODUCED IN A TIMELY MANNER, THE COUNTY RESERVES THE RIGHT TO AWARD THE BID TO THE NEXT MOST RESPONSIVE BIDDER.
                                                                                                                                                SHIP Housing Repair Program Specifications
Leslie E. Gilmore
620 W. Hernandez Street
Pensacola, FL 32501
PAGE 1
GENERAL:
▪ The subject Housing Unit was built prior to 1978 and therefore is a “targeted property” under the EPA’s RRP Rule, as some painted surfaces may be disturbed. Therefore the implementation of “safe work practices” is required along with submittal by the RRP Contractor of “Renovation Record keeping Checklist” form.
▪ There is no Federal Funding on this project.
The Homeowner will have the following sample choices, where applicable:
VCT Floor                                                                                1 choice
MECHANICAL:
Build new heater closet to meet Escambia County Code. Install drywall, door unit and trim to match existing as close possible.

Install new gas HVAC system with air handler including new duct work, registers and grills.  

Return air grill is to be removable filter type with filter installed.                                                                                               

Gas line is to meet current Escambia County Code.                                                                                                                                   

Install disconnects at condenser and air handler as required by Escambia County Code.
Prep and paint hall walls, ceiling and wood work (painted surfaces) to match existing as close as possible.
Mechanical total $_______________
HALL BATH:
Remove existing ceramic walls and floor. Remove window, install framing, insulate exterior wall where wall and window removed. Install siding and related trim on exterior where window removed.
Reframe floor as required, install subfloor, vaper barrier and underlayment to bring to existing grade.

Install green board on walls to match existing thickness of walls, finish and texture walls.
Install 1 new comfort height American Standard 1.28-GPF toilet model Champion 4 in white or Delta 1.28-GPF toilet model Riosa in white or equivalent with seat. 
Install new pedestal sink Aqua Source AP122-00 or Glacier Bay model Shelbourne or equivalent, reset existing faucet.                                                                                                                                

Detach and reset tub/shower faucet and trim.                                                                           

Install new drain assemblies from new fixtures to waste.
Install all new ¼ turn brass valves with braided supply lines at toilet and bath sink.
Install 1 new Sterling tub surround model 61044100 in white or Style Selections model SS01603059 or equivalent in white with corresponding tub.
Prep floor and install VCT. Install New wood base and shoe moulding.

Reinstall shower rod, install new toilet paper holder and towel bar.

Prep and paint walls, ceiling and wood work to match existing as close as possible.
Hall bath total $______________
                                                                                                                                                   SHIP Housing Repair Program Specifications
Leslie E. Gilmore
620 W. Hernandez Street
Pensacola, FL 32501
PAGE 2

HALLWAY:
Remove existing floor furnace, install framing as required to install subfloor and repair hardwood floor. Finish new hardwood to match existing as close as possible. 
Hallway total $______________
WINDOWS AND EXTERIOR DOORS:
There are thirteen (13) windows that are to be replaced with new retrofit custom windows (windows that are built specifically to fit existing openings).                               
The new windows are to be vinyl framed, insulated (double glazed) single hung window units with low E glazing impact rated glazing fitted with screens. Detach and reset window dressings as required.
Windows in the bedrooms are to meet egress requirements as per Escambia County Code. 
Replace two (2) entry door units with metal clad insulated 6 panel colonial door units

with thresholds, weather stripping and interior and exterior trim on all units. 

Both door units shall be hurricane rated.
Install new dead bolt and lock sets that are keyed alike on all new exterior door units. 
Existing locks may be reset if contractor and homeowner are in agreement.

Install peep hole on the front entry door.

Install new Storm door on front entry Anderson 2500 Series or Larson Savannah Series or equivalent with hardware.
Paint new door units to match existing as close as possible inside and out.
Window and door total $_____________
ROOF:

Remove existing brick chimney, using area on roof to vent new heater.

Remove old gas water heater vent.

Repair roof decking with like, kind and quality, repair roof to match existing roof as close as possible.
TOTAL JOB COST $__________________________

                                   (TO FRONT COVER)                                                                        
