[image: image1.jpg]Y

USP262-

Valley Center Schools

Embracing Today. Preparing for Tomorrow.




October 12, 2022
Valley Center USD 262 is seeking bids for the following work:
Refurbish the main and auxiliary gym floors at Valley Center Intermediate School.
 
Repair/replace any flooring boards due to damage as needed. Approval for this work must be gained prior to work.
 

Sanding: The sanding of the floor shall be done in such a manner as to remove all existing gym finish, paints, soils and stains. The sanding shall smooth the floor and remove any uneven areas from the floor. Edges and corners shall be hand edged so all exposed areas of the floor will be uniform in color.
The floor shall be sanded with an industrial drum sanding machine equipped with a dust collection system.  All cuts shall be in accordance with standards established by the Maple Flooring Manufactures Association, and the original floor manufacturer. A minimum of four cuts shall be made, each being a finer grade. Each cut shall remove the coarser marks of the previous cut. After the final cut of the drum sander the entire floor shall be rotary sanded with 100 grit paper. The final sanding of the floor shall be by rotary machine equipped with 120 or 150 grit screen
 
Clean Up: The contractor shall vacuum the floor with a heavy-duty commercial vacuum, to remove all dust from the floor and cracks between the boards, upon completion of job.  
 
Sealing: The floor shall be sealed with two coats high-quality gym seal that is approved and labeled as meeting the Maple Flooring Manufactures Association's standards. 
 
Painting: After the second coat of seal has been allowed to dry, screen the seal to 120 to180 grit and rinsed. The     game lines shall then be taped to make ready for painting. All paint is to be of the highest quality oil-based enamel in the customer's choice of colors.  Paint is compatible with urethane seal and finish. All game lines will be applied per Kansas State High School Activities Association regulations and per the local Unified School District. Lines for pickle ball courts may be added in the auxiliary gym. After the paint has dried completely, it shall be abraded and cleaned with waterless cleaner and allowed to dry before finishing may begin.
 

Finishing: The floor shall be coated with  two coats of MFMA approved oil modified urethane gym finish. Finish contains at least 45% non-volatile solids.
The work is to be performed between May 22, 2023, and July 31, 2023.
To arrange an opportunity to inspect the floors, please contact Dan Gentry by email at dan.gentry@usd262.net . 

All bids can be submitted either electronically, or by mail and must include your signature. Please submit your bids by 1:30 P.M. (CST) on November 18, 2022, to Mike Bonner, Assistant Superintendent, 143 S. Meridian, Valley Center Kansas, 67147, or via email at mike.bonner@usd262.net . 
USD 262 Valley Center retains the right to accept or reject any or all bids.

Please feel free to contact me should you have any question. I can be reached at 755-7000 or by email at mike.bonner@usd262.net .

Sincerely,

[image: image2.emf]
Mike Bonner

Asst. Superintendent, Valley Center Schools[image: image3.png]



