Specifications for  
New Model Knuckle Boom Loader
FY 23-02

Unit proposed must be a Kenworth T300 Series Day Conventional Cab with a PACMAC KB20HJ TKB1824 body and loader or an ACCEPTABLE ALTERNATE: Multiple units may be proposed and evaluated.

Minimum Specifications/Requirements for Chassis
-Low oil, Engine oil shut off.
-Air Brakes.
-Auto Transmission with Transmission Cooler.
-Idle Speed Control Switch.
-Power Steering.
-Exhaust to Exit under Truck.
-Rear Single Tandem Axle.
-Locking Rear Differential.
-Steel Wheels, 11-R-22.5 Radial Tires.
-Rear Bumper.
-Top of Cab Mounted Amber Strobe Light.
-Front Tow Hooks.
-Mud Flaps.
-AC/Heat.
-Air Horn.
-Dual 12V Battery System.
-Air Ride Drivers Seat.
-Passenger Seat.
-West Coast Style Side Mounted Mirrors.
-White Paint.
-All Factory Standards.

Minimum Specifications/Requirements for Body/Loader
-Joystick Controls.
-Minimum Hoist Rating 15 Tons.
-10 Gauge Steel Walls.
-Minimum 3/16” with an Option for 1/4” Single Sheet Steel Floor.
-18’ Length Body.
-LED Lights.
-Rear Mounted Strobe Lights, Top of Each Corner.
-Minimum 24 Cubic Yard Body.
-Barn Type Swinging Doors-Must Swing Flat Against Side.
-Engine Programmed with Proper RPM Level Activated by a Heavy Duty Switch at Operator’s Station.
-Must Have Auto Disengagement When in Gear, Auto Re-engagement When in Neutral.
-Bucket Must Have Bolt on Replacement Cutting Edge.
-Transmission Coupled Hydraulic Pump; No Drive Shaft.
-Boom Length 16’ with 4’ Telescopic Section, Total Length 20’.
-Tarp System to be Included.
-Option for Electric Tarp System.
-All Factory Standards.

The Following is for Both Components
-Specify Manufacturers Standard Warranty YR/HR.
-Specify How Many Miles Service Location is From Waycross.
-Vendor Shall Respond by Telephone Within 3 Hours of Service Call.
-Specify Length of Time to Respond to Service Call.
-Specify Interim Replacement Equipment Availability for Extended Downtime During Warranty Period.
-While Under Standard Warranty it Will be the Vendor’s Responsibility to Transport Equipment to and From Service Center.
-List 5 Business References with Comparable Machines Within a 200 Mile Radius of Waycross.
-List Top 5 Features of Equipment that Would be the Most Beneficial to the City of Waycross.
-Complete Color Brochure with all of the Specifications on the Proposed Machine Must be Included in Proposal.
-List Any Other Warranties; Including Extended Warranties That May be Available on Proposed Unit.
-Proposed Unit Must be Available for Demo if Requested.