ESCAMBIA COUNTY WASTE SERVICES

4 X 4 OFF-ROAD UTILITY VEHICLE

SPECIFICATIONS

Approximate Size:  Length 128.4”, Width 66.5”, Height 82.8” with a Wheelbase 77.4”

Cargo Box:  Width 52”, Length 46.7” Depth 11.4”, Load Capacity 1102 lbs.

Diesel Engine:  3 cylinder, 68.5 cu.in., 24.8 horsepower, 75-amp alternator

Transmission:  Variable Hydraulic

Hydraulics:  Hydrostatic power steering.  Hydraulic cargo bed lift.

Fluid Capacity:  Fuel tank 7.9 gal, Cooling 5.5 qts, Crankcase 4.0 qts, Transmission 3.2 gal

Tires and Wheels;  Heavy-duty worksite 25 x 10-12, 6 ply

Operating Features:  Independent front suspension.  Semi-independent rear suspension.  Fully-sealed, 4-wheel wet disc brakes.

Safety & Convenience

Unitized cab and chassis frame. Fully enclosed factory cab with roll-down windows.  Standard a/c, heater and defroster.  Two-point retractable seat belts.  Dash-mounted parking brake.  Horn.  Spark arrestor muffler.  2” hitch receiver, front and rear.  Front guard.

Required Additional Options

External rearview mirror.  Turn signal/hazard light kit.  Rear halogen work light.  Trailer hitch ball mount kit.  Strobe light mount kit.