CONTRACT SPECIFICATIONS

Section I

For this project, the following specifications and special provisions shall govern and control the work executed or performed under this Contract, and shall become a part of the Contract.

  - GENERAL:

This section is a compilation of general project related information primarily regarding the bidding of this project. Additional information regarding several of these items listed below can be located in other sections attached hereto. Bidders shall familiarize themselves with all project documentation.

  - GENERAL ITEMS:

    - Project Scope of Work: The intent of this project is to replace steel gas mains and services with plastic. The work will be throughout the Daphne Utilities natural gas distribution system. Daphne Utilities currently plans to perform work equally approximately $200,000 during the 2021/2022 fiscal year. However, the  Contractor  is not guaranteed this amount of  work.

    - Bid Opening Date and Time: October 27, 2021 at 9:00 am, Local Time.

    - Bid Opening Location: Central Services Building located at 8301Well Road, Daphne, Alabama.

    - Pre-Bid Conference: No pre-bid meeting will be held. Any questions should be directed to Alex Godfrey, Daphne Utilities Gas Manager alex@daphneutilities.com or to Bobby Purvis, Daphne Utilities Operations Manager bobby@daphneutilities.com. Questions about bid procedures should be directed to Marinda Turner, Daphne Utilities Buyer marinda@daphneutilities.com.

    - Bids must be submitted upon the standard forms, furnished by the Utilities Board of the City of Daphne, Alabama, included in the Project Specifications.

    - A Guarantee will be required with each bid for at least 5% of the amount of bid not to exceed

$10,000 filed in the form of a certified check or Bid Bond payable to the Utilities Board of the City of Daphne, Alabama. Bid Bonds shall include certification that the bonding company is listed in Circular 570 of the U.S. Treasury Department. The name, address, telephone number and contact person for the bonding company shall also be included.

    - The Contractor shall also furnish both a Contract Bond and a Labor and Material Bond. All bonds shall be for 100% of the Contract price. The Contractor may elect to provide the required Contract Bond and Labor and Material Bond as specified or with each work order. If Contractor selects to provide with each work order, bonds shall be secured within the timeframe stipulated for each work order. No additional time shall be granted for obtaining these items. All other applicable parts of these documents shall remain effective and enforced.

    - The Owner reserves the right to reject any and all bids and to waive any informalities in bids received.

    - Only bids from qualified contractors licensed by the Alabama State licensing  Board  for General Contractors will be accepted. Licensing information can be found in Title 34, Chapter 8, of the Code of Alabama, 1975 and any amendments related to the licensing of General Contractors.

    - Envelopes containing bids must be sealed, marked, addressed as follows, and delivered to the Buyer, Utilities Board of the City of Daphne, Alabama, 8301 Well Road, Daphne, Alabama 36526: "Bid 2021-2022 Annual Contract for Steel Gas Main and Service Renewals", to be opened at 9:00 a.m., Local Time, October 27, 2021. The Bidder's Alabama State Contractor's license Number shall be included on the envelope  containing the submitted  proposal.

CONTRACT SPECIFICATIONS

SECTION 2

For this project, the following specifications and special provisions shall govern and control the work executed or performed under this Contract, and shall become a part of the Contract.

  - SPECIFICATIONS:

All work under this Contract shall be done in accordance with the Utilities Board of the City of Daphne, Alabama (Daphne Utilities) Standard Specifications latest revisions, and as amended herein, and with the Alabama Department of Transportation Standard Specifications for Highway Construction, Latest Edition, and the standard practices, procedures, and ordinances of Daphne Utilities. The Standard Specifications may be obtained from Daphne Utilities' web site: www.daphneutilities.com.

    - These specifications shall govern and control the work executed or performed as if said specifications were included in detail and at length herein, in addition to the following:

      - Where supplementary specifications such as Federal, ASTM, ANSI, AASHTO, AWWA, etc., are referenced in these specifications, such references shall be the latest edition of such supplementary specification.

      - Any wording in the above noted Alabama Department of Transportation Standard Specifications as may refer to the Governor, State, State Highway Department, Director, Owner, etc., shall be deemed to read the Utilities Board of the City of Daphne, Alabama, acting through their officers or duly authorized representative.

      - Any wording in the above noted standard specification as may refer to "Engineer" shall be deemed to read the designated representative of Daphne Utilities.

    - All abandoned material designated by the Owner to be salvaged shall be removed and located by the Contractor at a location designated by Daphne Utilities as directed by the Owner.

  - SAFETY AND HEALTH REGULATIONS FOR CONSTRUCTION:

The Contractor shall comply with the Department of Labor Safety and Health Regulations for construction promulgated under the Occupational Safety and Health Act of 1970 (PL 91-596) and under Section 107 of the Contract Work Hours and Safety Standards Act (PL 91-54).

The Contractor alone shall be responsible for the safety, efficiency and adequacy of his plant, appliances, and methods of construction; and for any damage which may result from their failure or their improper construction, maintenance or operations. The Contractor shall supervise and direct the work and be solely responsible for the means, methods, techniques,

sequences and procedures of construction, as well as project site safety and the safety of its personnel, third parties, and members of the public.

Daphne Utilities will not accept liability for accidents or injury to personnel, damage to equipment or theft of property involving contractors, vendors, suppliers, or delivery companies.

To conduct business with Daphne Utilities, contractors, vendors, suppliers, or delivery companies must certify in writing that they have in force a drug and alcohol policy at least equal to that established for the Daphne Utilities.

  - PROJECT SIGN:

A project sign is not required for this Contract.

  - SUMMARY OF THE WORK:

    - Location: This project is located throughout the Daphne Utilities natural gas distribution system.  There is no guarantee that the individual work orders on a work order  listing

will be from the same area. Work orders may be issued for additional areas within Daphne Utilities natural gas distribution system than those listed above.

    - Description of Work: The intent of this project is to replace existing steel gas mains and services with plastic pipe.  The Owner shall periodically provide work orders and maps

that identify mains and services to be renewed.  The Contractor shall have a total of   ten

(10) calendar days to begin work on each work order, including notifying appropriate parties/City permit notifications. There may be months when no work order is provided. In case of an emergency renewal, the Contractor shall have forty-eight (48) hours to begin the service renewal. Clearing and grubbing with heavy equipment is not a part of this contract. The description of work includes the following:

- Obtain all necessary permits/licenses. The Owner shall obtain any necessary ALDOT permits required to perform the work. The Contractor shall comply with the requirements of said permit. The Contractor shall be responsible for obtaining City and County permits. The Contractor's effort for coordinating City and County permits including but not limited to completing all necessary forms, drawings, and maps shall be submitted under the appropriate pay item. Fees from City/County permits shall be initially paid by the Contractor. Following payment by the Contractor, the Contractor shall submit a paid invoice from the entity to the Owner with a payment reimbursement request. The Owner will reimburse the Contractor for the actual permit cost with no mark up. Any other necessary permits or licenses including City business licensee is the Contractor's responsibility and a subsidiary obligation of the various Contract bid items.

For the City of Daphne permit, the City Right of Way Ordinance 2017-22 shall be the standard for pavement repairs unless details noted herein are more stringent. Also, there is a preference for the City's permit to be filed electronically.

- Verify the exact locations and conditions of existing utilities by requesting line locates through Alabama One Call and by contacting utilities directly who are not members of the Alabama One Call system. This is a subsidiary obligation of the various Contract bid items.

- Take pre-construction video or pictures of the project site to document all site conditions as specified herein. This is a subsidiary obligation of the various Contract bid items.

- Provide notifications per the Project Specifications.

- Install and maintain traffic control when needed.  For all two-lane roads, this is  a subsidiary obligation of the various contract bid items.

- Install and maintain erosion control measures as necessary to perform the work in accordance with the Board's Standard Specifications and as specified herein.

Renew existing steel gas mains and services with plastic pipe in accordance with the Project Specifications and Daphne Utilities Standard Specifications which are located on DU web site at www.daphneutilities.com. Install fittings. Make hot taps and line stops. Contact the Owner to have the meter swapped over to the new service. Retire steel mains and services.

Per DU Standard Specifications, all gas main work shall conform to the applicable requirements of the U.S. Department of Transportation, Part 192, Transportation of Natural and Other Gas by Pipeline: Minimum Federal Safety Standards, most recent edition. Any Contractor employee who will perform gas work shall meet the "Qualifications of Pipeline Personnel - Subpart N". Contractor's employees who fuse plastic pipe shall be certified in fusion approved by Alabama Public Service Commission Office of Pipeline Safety. Contractor's employees who connect to hot gas mains or install live taps shall be qualified in that covered task per the Daphne Utilities Operator Qualification Program. (Ref: Energyworld.net, Computer based training and hands on training.) Documentation of compliance of these requirements shall be provided to Daphne Utilities.

- Complete restoration of disturbed easements and right-of-way areas to pre­ construction condition or better. Unless specifically stated otherwise and a pay item is established for the needed item, this is a subsidiary obligation of the various Contract bid items.

The Contractor's required work includes: Verifying exact locations and conditions of existing utilities, light clearing and grubbing with hand tools, installing necessary materials to complete the work in accordance with Daphne Utilities' Standard Specifications, restoration of disturbed areas to pre-construction condition or better, maintenance of construction areas including erosion control, traffic control, and safety planning and implementation, and all other related items in accordance with the project plans and specifications in addition to applicable permits and regulations. Unless specifically stated otherwise, these items are a subsidiary obligation of the various unit prices bid.

The Contractor shall provide all labor, equipment, permits, and bonds necessary to perform the required work in strict accordance with the project plans, specifications, and schedules.

The work orders may include information obtained from the Owner's records regarding existing pipe materials and locations. Mains and Services designated in the work order for renewal may be revised as directed by the Owner to accommodate actual materials and/or conditions discovered during construction.

The Quantities for this contract have been established for the purpose of evaluating bids and does not guarantee the Contractor will be issued this work in work order listings. Payment will only be made for the work performed and accepted. No payment will be made for damages to the existing system caused by the Contractor.

The Owner reserves the right to delete, reduce or increase, as necessary, the quantity of any bid item or item of work. No adjustments shall be made to the other bid item prices due to these revisions. Any changes shall be directed to the Contractor by the Owner in writing.

Where within these documents the terms or basis of payment is for removing and replacement, it shall be understood that no measurement or payment shall be made for removal of any material, regardless of the nature or character of the material removed. Measurement and payment shall be made only for that item replaced, as shown on the plans, required in these documents or directed by the Owner. The Owner reserves the right to direct the Contractor not to replace paved areas.

Costs associated with locating and time for the adjustment of existing utilities conflicting with the proposed construction shall be included in the contract unit bid price. No direct payment will be made for verifying locations or performing the relocation of water, power, telephone, gas, television cable, or any other existing utility required to perform the required construction.

    - Sequence of Construction: The Contractor shall provide sufficient crews to complete the work within the time specified.

If requested by the Owner, prior to the start of construction the Contractor shall prepare and submit for approval to the Owner a copy of the proposed sequence of construction operations for the required work for the work included herein. If requested, a sequence of operations must be approved by the Owner prior to the beginning of construction work. All Work shall be completed within the specified time including submittals.

The work shall be completed expeditiously. Area cleanup shall be performed daily in order to properly maintain all areas. Provisions shall be made by the Contractor to allow for the safe occupancy of vehicles and normal operations.

Required work in roadway rights-of-way shall be performed in a manner to allow a minimum of one lane of traffic for access to the adjacent areas, where practicable. Should limited access to isolated areas be required, the Contractor shall provide   the

property owners in the affected access area a minimum of one-day notice. This notice shall also include the anticipated limited access duration. Periodic access shall be provided to normally accessible areas should construction extend more than four (4) hours in these areas. In any event, the City shall be informed of changed traffic conditions.

    - Schedules: The Contractor shall be given ten (10) calendar days to begin work on each work order listing after the date on which said listing is issued to the Contractor. Actual contract time requirements will be prorated based on the dollar amount of work included in each work order listing. The Contractor shall complete the listed work not later than one (1) calendar day for each $2,000.00 of work required in the work order listing, rounded up to the nearest whole day.

For example: The Contractor is given a work order listing with $50,000.00 worth of work to perform. This work shall be completed in fifty-five (55) calendar days (30 days to submit permit applications and begin work plus one (1) day per $2,000.00 ($50,000/$2,000 = 25)).

Any requests for time extensions must be made to the Owner at the time of the delay. The Contractor shall provide two or more crews when required to complete the work provided in each listing in the above timeframe. Should the Contractor not respond in the time required the Owner may elect to terminate the contract or have the work performed by others.

The required work shall be issued to the Contractor from the Owner periodically as listings of work orders identifying specific steel gas mains and services to be renewed along with natural gas system maps identifying the locations of the main and service lines. Generally, the value of work required per listing shall be no less than $50,000 as based on the unit prices on the Contractor's Schedule of Prices. However, the Owner may also provide listings requiring less than $50,000 of work. There may be months when no listing is provided.

Commencement and prosecution of work shall be in accordance with this section in lieu of those applicable portions of section 8.02 "Notice To Proceed" and 8.04 "Prosecution of Work" in the General Conditions.

    - Project Documentation: Project documentation is a subsidiary obligation of the various Contract bid items.

      - General: Prior to start of construction, work on private property or within easements shall be documented by the use of photographs or video. At least one station marker shall be visible for identification purposes and station markers shall be set by the Contractor. During the course of the documentation, any features or items of interest or importance which may be encountered shall be photographed or videotaped. Each photograph shall clearly identify the date,  time, and location the photograph was taken. Photos and videos shall be delivered to the Owner within one week after they are made. There shall be no separate payment for photographic and/or video work.

    - Reports, Records and Data: The Contractor and each of his subcontractors shall submit to the Owner such schedules of quantities and costs, progress schedules, payrolls, reports, estimates, records and other data as the Owner may request concerning work performed or work to be performed under this Contract.

    - Permits, Certificates, Laws, Regulations and Ordinances: The Contractor shall, at his own expense unless specifically stated otherwise herein, procure all permits, certificates and licenses required of him by law for the execution of his work.

The Contractor shall comply with all federal, state, and local laws, ordinances and rules and regulations relating to the performance of the Work. A copy of all required permits shall be maintained at the project site at all times. If the Contractor observes that the Specifications are at variance with any laws, ordinances, rules and regulations applicable to the Work, he shall provide the Owner written notice thereof prior to submitting proposals. Any Work performed by the Contractor which is contrary to such laws, ordinances, rules and regulations and without written notice to the Owner shall be either dismantled and rebuilt or modified, with approval of the Owner to comply with said laws, ordinances, rules and regulations. Costs arising from such shall be borne by the Contractor.

In accordance with the Contract, the Contractor and all sub-contractors shall be required to demonstrate proof of enrollment in the E-Verify system. A copy of the Memorandum of Understanding (MOU) entered into between the Contractor and the Department of Homeland Security to use the E-Verify system is one example of sufficient documentation of enrollment.

  - WAGE RATES:

The construction of said Project shall in all respects conform to all applicable requirements of federal, state and local laws and ordinances.

  - WEATHER CONDITIONS:

In the event of temporary suspension of work or during inclement weather, or whenever the Owner shall direct, the Contractor shall, and shall cause his subcontractor to protect carefully his and their work and materials against damage or injury from weather. If any work or materials are damaged by reason of failure on the part of the Contractor or any of his subcontractors to protect his work, such materials shall be removed and replaced at the expense of the Contractor.

  - EXTENSION OF CONTRACT TIME:

Unless specifically waived by the Owner, when an extension of Contract time is granted by the Owner to the Contractor to complete the Work under this Contract, the Contractor shall pay any additional engineering cost or other cost accrued to the Owner as a result of the time extension granted.

  - CONTRACTOR REQUIREMENTS FOR CONFINED SPACE ENTRY:

Contractor shall comply with all applicable Occupational Safety and Health Administration (OSHA) Federal Regulations and the State of Alabama regulations.

  - CLEAN-UP:

The job shall be kept clean at all times. The Owner reserves the right to demand that the Contractor's forces be diverted to this clean-up at any time that site conditions warrant such diversion. Such diversion of Contractor's forces shall not entitle the Contractor to any extension of time or additional compensation.

  - BRANDS OF EQUIPMENT AND MATERIALS:

When referenced, the name of a certain brand, make or manufacturer is to denote the quality standard of the equipment or material, and is to convey the general style, type, character and quality of the equipment or material. Whenever a material or article required is specified or shown on the Plans by using the proprietary product or a particular manufacturer or vendor and is followed by "An Approved Equal" clause, any material or article which will meet the design criteria and is equal in function and durability as determined solely by the Owner shall be considered.

  - AFFIDAVIT OF COMPLIANCE:

Suppliers of materials shall furnish if requested, in triplicate to the Owner, an affidavit stating that all materials furnished under this Contract conform to the requirements as set forth in these Specifications. Unless specified otherwise, all materials shall be new, unused and without defect.

  - MANUFACTURER'S CERTIFICATION:

To assure that manufacturers and suppliers are aware of the use to which their equipment and products will be subjected, the Contractor shall require the manufacturer or manufacturer's representative to place the following certification on submitted data transmittals:

"This is to certify that we have examined the Plans and Specifications for this Project and have ascertained that this equipment or material is suitable for the purpose and use intended.”

Authorized Signature

  - BASIS OF AWARD: Owner intends to award the project to the lowest responsible and responsive bidder based on available funding.

CONTRACT SPECIFICATIONS

SECTION 3

  - GENERAL:

# The unit price bid for the various items shall be compensation in full for furnishing all labor, equipment, and incidentals for the item in place complete in every detail. There will be no direct payment for clearing, excavating, dewatering, bracing, clean-up and restoration of property, testing or other items of work necessary for the installation of the item unless noted otherwise herein, with the exception of removal and replacement of pavement as paid for under separate items.

All items shall be in accordance with Daphne Utilities Standard Specifications latest revisions and as amended herein. Daphne Utilities Standard Specifications are located at www.daphneutilities.com.

  - UNIT BID ITEMS:

Bid items are numbered according to the following classifications: Backfill (BF); Erosion Control (EC); General Construction Items (GC); Pavement Items (PA); Gas Facilities (GF).

They shall be measured for payment as follows:

BF-1    Select Sand or Sand-Clay Backfill: The unit price bid for this item shall be compensation   in full for providing all labor, materials, equipment, and incidentals necessary to remove, install, and compact one (1) cubic yard in place (CYIP) of select sand or sand-clay used for backfill complete in place. This item will be measured in place, meaning the length authorized for installation will be multiplied by the width and the depth authorized for installation and then converted to cubic yards. Backfill shall be placed and then prepared and compacted for installation of base material.  Surplus material will not be paid for and must be hauled away from the site at no additional cost to the Owner.

EC-1    Silt Fence (Type Al: The unit price bid for this item shall be compensation in full for all labor, material, equipment, incidentals, and appurtenances necessary for complete installation of one (1) linear foot (LF) of silt fence installed for temporary erosion control as shown, specified, or directed by the Owner.  Measurement for this item shall be the actual length of Silt Fence (Type A) installed as shown, specified, or directed by the Owner. The unit price bid shall also include the necessary removal of this item once the site is stabilized and the work completed.

EC-2    Wattles: The unit price bid for this item shall be compensation in full for all labor, material, equipment, incidentals, and appurtenances necessary for complete installation of one (1) linear foot (LF) of Wattle installed for temporary erosion control as shown, specified, or directed by the Owner. Measurement for this item shall be the actual length of Wattles installed as shown, specified, or directed by the Owner.  The unit price bid shall also include the necessary removal of this item once the site is stabilized and the work completed.

EC-3   Final Grassing for Erosion Control: The unit price bid for this item shall be compensation in full for all labor, materials, equipment, incidentals, and appurtenances necessary for one (1) acre (AC) of grassing for erosion control complete in place including, but not limited to ground preparation, fertilizing, seeding, and maintenance. Measurement for this item will be made to the nearest tenth of an acre of the area actually grassed as shown, specified, or directed by the Owner. No payment will be made for grassing areas outside of the construction limits as shown, specified, or directed by the Owner.

EC-4     Mulching: The unit price bid shall be compensation in full for all labor, materials,  equipment, incidentals,  and appurtenances  necessary  for  complete  installation  of one

    - acre (AC) of mulching complete in place. Measurement will be made to the nearest tenth of an acre of the actual area mulched as shown, specified, or directed by the Owner. No payment will be made for mulching of areas outside of the construction limits as shown, specified, or directed by the Owner.

EC-5     Solid Sod: The unit price bid for this item shall be compensation in full for furnishing all  labor, materials, equipment and incidentals necessary to install one (1) square yard in place (SYIP) of solid sod, complete in place, as directed by the Owner.

EC-6   Topsoil: The unit price bid for this item shall be compensation in full for furnishing all labor, materials, equipment, and incidentals necessary to install one (1) cubic yard in place (CYIP) of topsoil, complete in place, as directed by the Owner in accordance with Section 650 Top Soil of the Alabama Department of Transportation's Standard Specifications for Highway Construction, latest edition. The required topsoil for this item shall be material obtained from an offsite source. Topsoil shall be workable, friable, loamy soil free from hard lumps, stiff clay, gravel, noxious weeds, brush, and other deleterious materials. Lime shall be added to reduce the possibility of odor. Topsoil shall be placed in areas disturbed by the proposed work and as directed by the Owner, prior to grassing. The topsoil shall be placed to the compacted depth specified by the Owner, but shall not, in any case, be placed at a compacted thickness of less than two (2) inches.

It is intended that the grassing operation shall follow immediately after the placing of topsoil in which case such grassing operation would require satisfactory compaction in order to prevent erosion. In the event that grassing operations are delayed, the layer of topsoil shall be compacted until satisfactory. The Contractor shall maintain the topsoil  that has been placed, without extra compensation, in connection with any seeding, sodding, planting, or other work,  until  final  completion  of  the  project.  Maintenance shall consist of preserving, protecting, and other work as may be necessary to keep the site in a satisfactory condition.

GC-1   Permit Coordination: The unit price bid for this item shall be compensation in full for furnishing, all labor, material, equipment and incidentals for coordination and obtaining one each (EA) City or County permit as necessary. It is intended that permit coordination will only be paid once per work order listing issued regardless of the individual number of work orders in the listing. The Contractor's effort for coordinating said City and County permits shall include but is not limited to completing all necessary permit applications, providing all necessary drawings, and maps. Typically, the maps provided in the work order and standard drawings provided in the Project Specifications may be used as a template with any necessary specific details added by the Contractor for   each   area.   Fees from  City/County permits shall be initially paid by the Contractor. Following payment by the Contractor, the Contractor shall submit a paid invoice from the entity to the Owner with a payment reimbursement request. The Owner will reimburse the Contractor for the actual permit cost with no mark up. The Owner will obtain any necessary ALDOT permit and no payment will be allowed under this pay item for the Contractor since the coordination effort will be performed by others for the ALDOT permit. The Contractor shall be required to comply with all conditions of the ALDOT permit.

For the City of Daphne permit, the City Right of Way Ordinance 2017-22 shall be the standard for pavement repairs unless details noted in these Project Specifications or Daphne Utilities Standard Specifications are more stringent.

GC-2  Removing and Replacing Concrete Curb and Gutter: The unit price bid for this item shall be compensation in full for furnishing all labor, materials, equipment and incidentals necessary to remove and replace one (1) linear foot (LF) of concrete curb and gutter, concrete valley gutter or concrete curb. Curb, gutter, and curb and gutter shall be of the same type and thickness as that removed, with a minimum 28-day compressive strength of 3,000 psi. Existing curb and gutter shall be removed to the nearest construction joint and saw cut. In measuring this item for payment, the actual length installed will be used to calculate the quantity for payment. Construction shall be in accordance with the City's Ordinances, the County's requirements, or the State's requirements; in whichever Right-of-Way the Contractor is performing work.

GC-3    Remove and Reset Fencing: The unit price bid shall be compensation in full for removing and resetting one (1) linear foot (LF) of fence of the various types encountered. Measurement will be made for the length authorized to be removed and reset. Posts, fencing or other materials lost, damaged, or destroyed by the Contractor's operations shall be placed with new material of the same type that existed prior to removal at the Contractor's expense. No payment shall be made for fences removed by the Contractor that are not authorized by the Owner.

GC-4    Crushed Stone Foundation w/ Geotextile Fabric (6-lnch Lifts): The unit price bid for this item shall be compensation in full for all labor, materials, equipment, tools, incidentals, and appurtenances for furnishing, spreading, and compacting crushed stone and geotextile fabric for the full width of trench under the pipe for a minimum six (6) inch thickness and up to the spring line of the pipe for one (1) linear foot (LF) of trench for the foundation of the pipe in accordance with the Project Plans and Specifications or as directed by the Owner. Where the depth is greater than six (6) inches, the quantity to be paid for shall be increased proportionally. Therefore, when the existing soils require 18 inches of stone under the pipe then this pay item would be multiplied by three. No payment will be made for this item where the Contractor elects to utilize crushed stone with geotextile fabric for convenience or to fill overcuts. The geotextile fabric shall be Mirafi 140NL, or Owner approved equal.

PA-lA    Removal and Preparation of Base for Flexible Pavement: The unit price bid for this item  shall be compensation in full for providing all labor, materials,  equipment,  tools, incidentals, and appurtenances necessary to remove one (1) square yard in place (SYIP) of flexible wearing surface and base and prepare and install dense graded crushed stone aggregate base (ALDOT Type B as per ALDOT Standard Specification Section 825.03, 6- inch minimum thickness) complete in place in accordance with the Project Plans and Specifications. Payment for this item shall include removal of the existing base, wearing surface, and any intermediate pavement layers, regardless of the thickness.

Measurement for this item will be the calculated area of removal and preparation of base for flexible pavement removed and replaced as shown, specified, or directed by the Owner. No payment will be made for removal and preparation of base for flexible pavement outside of the areas shown or directed by the Owner. For the City of Daphne, the City Right of Way Ordinance 2017-22 shall be the standard for pavement repairs unless details noted in these Project Specifications or Daphne Utilities Standard Specifications are more stringent.

PA-lB   Removing and Preparing Base for Flexible Pavement (Crushed Stone Base): The unit price bid for this item shall be compensation in full for furnishing all equipment, labor, materials and incidentals necessary for removing one (1) square yard in place (SYIP) of flexible wearing surface and base and the preparation and placement of new No.4 crushed stone base (6" minimum thickness) complete in place. The limits of the removal of the existing pavement and base shall be saw cut. In measuring this item for payment, the actual length and width of the base removed will be used to calculate the quantity for payment. The additional cut-backs made for the wearing surface will not be included in payment for this item. Construction shall be in accordance with the City's Ordinances, the County's requirements, or the State's requirements; in whichever Right­ of-Way the Contractor is performing work. For the City of Daphne permit, the City Right of Way Ordinance 2017-22 shall be the standard for pavement repairs unless details noted herein are more stringent.

PA-2     Bituminous Wearing Surface for Patch Surfacing: The unit price bid for this item shall be compensation in full for providing all labor, materials, equipment, tools, incidentals, and appurtenances necessary to remove and replace one (1) square yard in place (SYIP) of asphalt patch (ALDOT Standard Specification 429A, 2006 Edition, Y,-inch max. aggregate size mix) (424 A for City of Daphne) in  accordance  with  the  pavement  replacement detail for City, County, and State Right-of-Way provided in the appendices, including leveling, tack coat, binder layer if necessary, striping, handicap markings and incidentals necessary for completion of the wearing surface. Measurement for this item will be the calculated area of bituminous wearing surface for patch surfacing removed and replaced as shown, specified, or directed by the Owner. No payment will be made for bituminous  wearing surface for  patch surfacing outside of the areas shown or  directed  by the Owner. For the City of Daphne, the City Right of Way Ordinance 2017-22 shall be the standard for pavement repairs unless details noted herein are more stringent.

PA-3    Temporary Asphalt Patch: The unit price bid shall be compensation in full for one (1) square yard in place (SYIP) of temporary asphalt patch complete in place including maintenance and removal prior to the final patch covered under other pay items. The Contractor shall allow a 30-day weathering period of the temporary patch prior to placement of the final patch. Construction shall be in accordance with the City's Ordinances, the County's requirements, or the State's requirements; in whichever Right­ of-Way the Contractor is performing work. For the City of Daphne, the City Right of Way Ordinance 2017-22 shall be the standard for pavement repairs unless details noted herein are more stringent.

PA-4     Removing and Replacing Concrete Driveways, Sidewalks or Ditch Paving:  The unit price  bid for this item will be compensation in  full  for  furnishing  all  material,  labor,  equipment, and incidentals to remove and replace one square yard in place (SYIP) of concrete driveway, sidewalk or ditch paving. The unit price bid for this item shall also include base course, saw cuts, expansion and construction joints, formwork, reinforcing steel, pavement  markings, and incidentals  for  a complete removal and installation  of the

concrete pavement. The thickness of the pavement patch installed shall match the thickness of the existing driveway, sidewalk, or ditch with a minimum of 5 inches. Measurement for this item will be the calculated area of concrete sidewalks, driveways, or ditch paving removed and replaced as shown, specified, or directed by the Owner. No payment will be made for replacement of concrete sidewalks, driveways, or ditch paving outside of the areas shown or directed by the Owner. Construction shall be in accordance with the City's Ordinances, the County's requirements, or the State's requirements; in whichever Right-of-Way the Contractor is performing the work.

TC-1     Provide Traffic Control for Construction  in  Roadway  Greater  Than  2 Lanes:  The unit  price bid per one (1) day (DAY) for this item shall be compensation in full for furnishing, installing, maintaining  and removing all required  traffic control items  for  construction  in  a roadways greater than 2 lanes.  The Contractor will be responsible for ensuring that the traffic control provided meets the requirements of the City, County, or State Right­ of-Way, whichever the construction is being performed in. The traffic control shall be in place before construction begins, maintained for the duration of the construction and then promptly removed. Travel control on two lane roads is a subsidiary obligation of the various contract bid items.

TC-2    Traffic Control for Complete Road Closure: The unit price bid per one (1) day (DAY) for this item shall be compensation in full for furnishing, installing, maintaining and removing all required traffic control items for construction associated with a complete road closure. The Contractor shall not close any roads prior to obtaining the Owner's and appropriate Authorities' approval.

GF-1    Install and Tap Plastic Service on Steel Main - Open Cut 50 Feet or Less: The unit price bid for this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to install one (1) each (EA) W' polyethylene gas service up to fifty (50) feet in length via open cut. Payment shall include installing the service tee, EFV, service riser, Y." pipe, and appurtenances required for properly connecting the new service line to the existing steel gas main. The Contractor will also be responsible for tapping the service tee, testing and purging the new service, and retiring the existing steel service including purging with air and capping the stub. All material is Owner supplied.

GF-2    Additional  Footage  for  Open Cut (each linear foot  over  50):  The unit price bid for  this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to extend the plastic service line described in Pay Item GF-1, "Replace Existing Steel Gas Service with Plastic - Open Cut 50 Feet or Less," and GF-11, "Install and Tap Plastic Service on Plastic Gas Main - Open Cut 50 Feet or Less," for one (1) linear foot  (LF) to the length required to complete the service installation.  This pay item is in addition to GF-1 or GF-11. Payment for all other items required to complete the service installation shall be paid in accordance with their unit prices. All material is Owner supplied.

GF-3     Install and Tap Plastic Service on Steel Main - Bore 50 Feet or Less: The unit price bid for this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to install one (1) each (EA) Y." polyethylene gas service up to fifty

(50) feet in length via directional bore. Payment shall include installing the service tee, EFV, service riser, Y." pipe, and appurtenances required for properly connecting the new service line to the existing steel gas main. The Contractor will also be responsible for tapping the service tee, testing  and  purging the  new  service,  and retiring the  existing

steel service including purging with air and capping the stub. All material is Owner supplied.

GF-4     Additional Footage for  Bore (each linear  foot  over 50):  The unit price bid for  this item  shall be compensation in full for furnishing all labor, equipment,  and  incidentals  necessary to extend the plastic service line described in Pay Items GF-3,  "Replace Existing Steel Gas Service with Plastic - Bore 50 Feet or Less," and GF-12, "Install and Tap Plastic Service on Plastic Gas Main - Bore 50 Feet or Less." for one (1) linear foot  (LF) to the required length to complete the service installation.  This pay item is in addition to GF-3 or GF-12. Payment for all other items required to complete the service installation shall be paid in accordance with their unit prices. All material is Owner supplied.

GF-5    Insert Existing Steel Gas Service with Plastic -  100 Feet or  Less:  The unit price bid for   this item shall be compensation in full  for  furnishing  all  labor,  equipment,  and incidentals necessary to insert one (1) each (EA) existing steel gas service with one (1) W', 5/8", or W' polyethylene gas service up to  one  hundred  (100)  feet  in  length. Payment shall include installing the service tee, EFV, service riser, service pipe, and appurtenances required for properly connecting the new service line to  the  existing steel gas main. The Contractor will also be  responsible  for  tapping the  service  tee, testing and purging the new service, and retiring the existing steel  service  including purging with air and capping the stub.  All material is Owner   supplied.

GF-6		Additional Footage for  Inserting (each linear  foot  over  100):   The unit price  bid  for this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to extend the plastic service line described in Pay Item GF-5, "Insert Existing Steel Gas Service  with Plastic	100 Feet or Less."  for  one (1)  linear foot  (LF)  to  the required length to complete the service installation. This pay item is in addition to GF-5. Payment for all other items required to complete the service installation shall be paid in accordance with their unit prices.  All material is Owner   supplied.

GF-7    Retire Steel Gas Service: The unit price bid for this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to retire  one (1) each  (EA)  steel gas service. The Contractor will be responsible for  purging the retired service with air and capping the stub.  All material is owner  supplied.

GF-8  Install 2" MDPE Gas Main - Open Cut: The unit price bid for this item  shall  be compensation in full for furnishing all labor, equipment, and incidentals necessary  to  install via open cut one (1) linear foot (LF) of 2" MDPE gas main. All material is Owner supplied.

GF-9     Install 2" MDPE Gas Main -  Bore:  The unit price bid for this item shall be compensation  in full for furnishing all labor, equipment, and incidentals necessary to install  via  directional bore one (1) linear foot (LF) of 2" MDPE gas main. All material is Owner supplied.

GF-10A Install 2" Stopper Fitting: The unit  price  bid for  this item shall be compensation  in full for furnishing all labor, equipment, and incidentals necessary to install one (1) each (EA) stopper  fitting, including the tap and line stop.   All material is Owner  supplied.

GF-lOB Install 2" Bypass (Bottom Out) Stopper Fitting: The unit price bid for this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to install one (1) each (EA) bottom out stopper fitting, including the tap and line stop. All material is Owner supplied.

GF-11   Install and Tap Plastic Service on Plastic Gas Main - Open Cut 50 Feet or Less: The unit price bid for this item shall be compensation in full for furnishing  all labor,  equipment,  and incidentals necessary to install one (1) each (EA) %'' polyethylene gas service up to fifty (50) feet in length via open cut.  Payment shall include installing the  electrofusion  tap tee, EFV, service riser, %" pipe, and appurtenances required for properly connecting the new service line to the plastic gas main. The Contractor will also be responsible for tapping the service tee and testing and purging the new service. All material is Owner supplied.

GF-12   Install and Tap Plastic Service on Plastic Gas Main - Bore 50 Feet or Less:  The unit price bid for this item shall be compensation in full for furnishing all labor, equipment, and incidentals necessary to install one (1) each (EA) %" polyethylene gas service up to fifty (SO) feet in length via directional bore. Payment shall include installing the electrofusion tap tee, EFV, service riser, %" pipe, and appurtenances required for properly connecting the new service line to the plastic gas main. The Contractor will also be responsible for tapping the service tee and testing and purging the new service. All material is Owner supplied.

CONTRACT SPECIFICATIONS

SECTION 4

  - DESCRIPTION

    - Coordinate the Work with OWNER, utility owners, and owners of affected private properties and provide the required notices.

  - NOTICES TO OWNER

    - Notify the OWNER in writing at least 2 business days in advance of beginning any Work on Site.

    - Notify appropriate City of Daphne personnel or other appropriate agency personnel of the closing of any street. Information as requested by the City of Daphne or other agency shall be provided in accordance with the permit conditions.

  - NOTICES TO  OTHER  UTILITIES

    - Notify owners of adjacent utilities when prosecution of the Work may affect them.

  - NOTICES TO PROPERTY OWNERS

    - Notify all property owners whose properties are affected by the Work between 2 to 4 days prior to beginning work by distributing fliers to the homes and business connected to the sanitary sewer mains being inspected. The flier should state the CONTRACTOR 24-hour emergency contact phone number and contact name along with the nature of the work. The Notification template to the customer shall be pre-approved by the Owner prior to use.

  - SUBMITTALS

    - Submit final notice of each of the above notices to OWNER for review/approval before delivery.