HO-6B

REHABILITATION WORK WRITE-UP ESTIMATE

Owner: ___Michael Haywood____________________ Date: ___11-24-2015____

Physical Address: ___1220 School Street________________________________

City __Columbia______________ Zip Code ___38401__ Grant Year __2012

Administrator: ___Maggie Fincher_____________ Grantee: City of Columbia

Year built: 1940 – Lead Levels Not Elevated

Formatted in conjunction with the Uniform Physical Condition Standards Checklist

SITE				    Rehabilitation     Lead Based Paint

GROUNDS

Erosion/Rutting/Overgrown Vegetation/Ponding/Site Drainage

Trim tree limbs off roof at front and remove vegetation that is touching

Or growing on the house.

$                   		$_________

WALKWAYS/ STEPS

Broken/Cracks/Settlement/Damaged/Missing or Damaged Handrails

Remove rear concrete steps and replace with wood steps.

Wood Exposed to Weathering Elements:  All exterior wood shall have a minimum preservative retention rate of 25 percent for above ground applications and a minimum preservative retention rate of 40 percent for all wood in contact with the ground.

Wood Decking:  Exterior wood decking shall consist of composite, natural durable wood or pressure treated wood material.

Wood Stairs and Handrails:  Exterior wood stairs shall be constructed with properly treated dimensional lumber. Handrails shall be constructed of composite, natural durable wood or pressure treated wood material.  Wood: All exterior exposed wood shall have a minimum preservative retention rate of 25 percent for above ground applications and a minimum preservative retention rate of 40 percent for all wood in contact with the ground. All structural posts below grade shall have the factory treated end of the post below grade. Use of CCA Treated lumber shall be prohibited.

Posts and Columns: All posts columns and guardrails at deck level and above shall be factory made and finished.

Handrails: Rails shall be smooth, splinter-free, painted or factory finished, withstanding weather, and resisting checking and splintering.

$_______               	$_________

BUILDING EXTERIOR

FOUNDATIONS

Damaged/Cracking/ Crawl Space Access/Vapor Barrier/Insulation/Debris/Vermin

728s.f. floor insulation and vapor barrier

All crawl spaces shall contain cavity floor insulation equal to or greater than R-19.

Install new vent screen in foundation wall on North side of house

$_______		$_________

ELECTRICAL

Broken/Missing/Fixtures/Bulbs/Receptacles/ Covers

Replace light fixture at front. See Building Systems		   $__________	$_________

ROOFS

Damaged/Soffit/Fascia/Vents/Drains/Membrane/Shingles/Gutters/Downspout/Ponding

Remove existing shingles and install new. 8.5 square

Shingles: 215 lb. seal tab type over 30 lb. felt with metal drip edge,  with minimum 25-year product warranty.

Remove approx. 15lf of damaged fascia and 15sf of damaged soffit.

Replace fascia with new wood and soffit with new vinyl soffit. Paint new fascia wood.

Wood Exposed to Weathering Elements:  All exterior wood shall have a minimum preservative retention rate of 25 percent for above ground applications and a minimum preservative retention rate of 40 percent for all wood in contact with the ground

Vinyl: 0.42” minimum thickness, UV protected.

$_______		$_________

WALLS

Approx 1,140 sf

Damaged/Missing/Cracks/Gaps/Holes/Spalling/Peeling/Caulking/Chimneys

Pressure wash and paint exterior

Stabilize chimney footer to repair and prevent future chimney lean. Possibly

With a helical pier. Caulk intersection of wall and chimney with silicone.													      $_______		$_________

WINDOWS

Damaged/Missing/Broken/Frames/Panes/Sills/Lintels/Trim/Screens/Caulking/Paint

Remove existing windows, repair jambs and sills, install new windows.

All window frames must be solid vinyl. Double hung type to have both upper and lower sashes able to slide with a removable bug screen for lower section.All glazing shall be double paned with low E with Argon. The vapor seal on the glazing must have a minimum ten-year  warranty.All windows shall have a minimum one-year warranty on the operation of the window. All windows shall have a National Fenestration Rating meeting minimum energy code requirements for Zone 4. All windows shall have a U-Factor equal to or less than 0.40 and an SHGC rating equal to or less than 0.40.  8 windows					   		     $_______		$_________

BUILDING SYSTEMS

ELECTRICAL SYSTEMS

Access/Damaged/Missing/Breakers/Covers/Wires/Corrosion/Evidence Water Leaks

Allowance to remove existing abandoned wiring, install covers on open junction boxes and install new 200 amp breaker panel in crawlspace and repair electrical items in individual rooms. See individual room electrical notes.

$_______		$_________

WATER HEATER

Inoperative/Leaking/Rust/Corrosion/Missing/TPRV/Vent/Chimney/Combustion Air

Install TPRV discharge tube.

Water heaters located in any interior space shall have a metal 2” deep overflow pan with discharge pipe indirectly plumbed into DWV receptor, floor drain or to the exterior. Discharge pipe end shall have a bug screen.

$_______		$_________

SANITARY SYSTEMS

Broken/Leaking/Clogged/Missing/Cleanout/Septic System

Note: Assure Sump pump functions properly

$_________ 		$_________

HVAC

Inoperative/Noisy/Vibrating/Leaking/Rust/Corrosion/Missing/Vent/Chimney

Install new HVAC system     750s.f.  Package heat pump may be the most economical.

All units shall have a central HVAC system comprised of a split system heat pump with emergency heat strips for heating and cooling using high- efficiency equipment. Units with gas shall be a split system air conditioner and furnace. Heat pump/ AC systems shall have a minimum SEER (Seasonal Energy Efficiency Rating) rating of 14.5 with a minimum HSPF (Heating Seasonal Performance Factor) rating of 8.5. Fuel oil, gas fired furnaces and boilers shall have an Annual Fuel Utilization Efficiency (AFUE) equal to or greater than ninety-two percent (92%). Electric- resistance-only heat systems are prohibited. Alternative HVAC systems may be approved by THDA. Prefer direct vent on gas furnace or sealed combustion.

Duct system can be a mix of; hard duct main trunk lines (sheet metal) from the air handlers with flex duct branching out to the individual vents. All duct must be insulated with a minimum R-5, unconditioned space R-8 and installed per SMACNA standards. All joints and connections shall be sealed tight compliant with UL 181 and pass pressure testing. Exhaust vent piping shall be insulated. All ducts shall be installed per manufactures requirements and free of restrictions. Vent louvers shall be white adjustable type. Dryer vent shall be through wall weather type with damper. All supply ducts shall have individual dampers enabling supply air adjustments at each register grill and repositioning damper. All HVAC systems shall be Tested and Balanced within 20% of the ACCA Manual J, S and D calculations.

All individual HVAC systems shall be controlled by a digital thermostat.

$_______		$_________

BATHROOM

BATHROOM

Cabinets/Top/Sink/Plumbing/Tub/Shower/Ventilation/Water Closet/Accessories

Repair loose and leaking plumbing pipes in shower wall with ½” copper pipe; install new tub/shower and faucets.

Tub/ shower surrounds shall be 3 piece integral panels of plastic, fiberglass, or cultured marble minimum 72” above flood rim.

Potable water lines shall be copper or PEX material. Installation in exterior walls must be center in the insulation. Hose bibs must be of the freeze proof kind. All piping located in attic or crawl space shall be insulated. All hot water lines shall be insulated equal to or greater than R-4 pipe wrap

$_______	$_________

CEILING

Approx 45 sf

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair cracks and paint.	Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$____		$_________

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/ Paint

Install new door, hardware and paint

Solid Wood, Composite or hollow core panel doors. Interior doors shall be a minimum of 1 3⁄8” thick.  a. Interior Hardware:  All doors, except bi-fold and bypass type doors, shall be equipped with passage lever-handle hardware. All bedrooms and bathrooms shall be equipped with privacy locks.  Primed once, with two-coat semi-gloss finish on all sides and faces. No VOC.

$_____		$_______

ELECTRICAL SYSTEM

Inoperative/GFCI/ Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Add GFCI outlet	See Building Systems

$________		$_________

FLOORS

Approx 45 sf

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Remove and replace approx. 12sf of rotted subfloor with new ¾” subflooring and install new vinyl flooring in entire Bathroom. Sheet Vinyl: Shall be Armstrong or equivalent minimum 10 mil wear layer. Provide product adhesive and underlayment as recommended by the manufacturer. All surfaces shall be clean, dry, and appropriate temperature during installation. Vinyl sheet flooring shall conform to the requirements of ASTM F 1303, Type I.

Vinyl Composition Tile (VCT): Shall be Armstrong or other approved equal, 1/8” x 12” x 12”. Provide product adhesive and underlayment as recommended by the manufacturer. All surfaces shall be clean, dry, and appropriate temperature during installation. Follow manufacture’s recommendation for pattern layout.

$_______	$_________

WALLS

Approx 224 sf

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/ Mildew

Paint walls. 	Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC.

$_______		$_________

KITCHEN

KITCHEN

## Damaged/Missing/Cabinets/Countertop/Sink/Faucet/Plumbing/Appliances

Install approximately 8lf of cabinets and counter with sink and faucets

Cabinet fronts shall be made of solid wood (not particleboard); doors, draws and fronts shall be factory finished. Cabinet ends shall be finished with appropriate veneer. All cabinets shall be Kitchen Cabinet Manufacturers Association (KCMA) approved.

Tops shall be post form plastic laminate, bullnose front edge, rolled backsplash, finished exposed ends and sealed at the cut out for sink. Other appropriate materials may be used such as solid surface, and cultured marble for bathroom vanities. Any other types of countertops must be approved by THDA.

Sinks shall be a minimum eight inches (8”) deep, 33 x 20 stainless steel double bowl. For ADA compliance when applicable, roll under sink depth shall be shallower as required in mobility impaired dwelling units

$_______		$_________

CEILING

Approx 156 sf

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair cracks and paint. 	Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$_______		$_________

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Repair frame and hardware at rear door. Paint door.

Primed once, with two-coat semi-gloss finish on all sides and faces. No VOC.

$_____                        $______

ELECTRICAL SYSTEM

Inoperative/GFCI/Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Install GFCI outlet. See Building Exterior

$_______		 $_________

FLOORS

Approx 169 sf

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Remove and replace approx. 20sf of damaged subfloor with now ¾” subflooring under cabinets. Install new Vinyl flooring.

$_______		$_________

WALLS

Approx 410 sf

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/Mildew

Paint walls. 	Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$_______		$_________

WINDOWS

Cracked/Broken/Missing/Panes/Sill/Caulking/Hardware/Paint/Water Stains/Screens

See Building Exterior						$_________ 		$_________

BEDROOMS

CEILING

2 Bedrooms approx. 156 sf each

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair cracks, water stains and paint in both Bedrooms.

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$_______		$_________

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Re-hang closet door in rear Bedroom. Adjust entrance doors to both Bedrooms to allow for closing. Paint doors. Primed once, with two-coat semi-gloss finish on all sides and faces. No VOC.

$_______	$_________

ELECTRICAL SYSTEM

Inoperative/Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Install cover plates on outlets. See Building Systems.

$_________         $_________

FLOORS

2 Bedrooms approx. 156 sf each

Bulging/Buckling/Damaged/Missing Covering/Tile/Deteriorated Subfloor/Water Stains

Install new carpet in bedrooms.

Carpet Padding: 7/16” thick, 6-lb. minimum re-bond polyurethane.

Sheet Carpet: 25 oz. minimum, 100 percent nylon. Other options include Berber type with blended fiber.

$_______		$_________

SMOKE DETECTOR

Damaged/Missing/Inoperative/Non-Compliant

Install smoke detector in both Bedrooms

$_______		$_________

WALLS

Approx 400 sf

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/ Mildew

Paint walls in front Bedroom. Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$_______	$_________

WINDOWS

Cracked/Broken/Missing/Panes/Sill/Caulking/Hardware/Paint/Water Stains/Screens

See Building Exterior						$_________ 		$_________

HALLWAY

FLOORS

Approx 28 sf

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Install new carpet.	Carpet Padding: 7/16” thick, 6-lb. minimum re-bond polyurethane.

Sheet Carpet: 25 oz. minimum, 100 percent nylon. Other options include Berber type with blended fiber.

$_______	$_________

SMOKE DETECTOR

Damaged/Missing/Inoperative/Non-Compliant

Install smoke detector in Hallway ceiling

$______	                        $______

LIVING ROOM

LIVING ROOM

## Damaged/Missing/Ceiling Fan/Closet/Entry

Install new ceiling fan with light.	Ceiling fans shall be minimum 42” 4 paddle with light kit located in all bedrooms and a 52” 5 paddle with light kit in the living room. Fans shall be Energy Star qualified. The Energy Star mark must be clearly marked on the front/top of the product, clearly displayed in product literature and listed on the manufacturer’s Internet site. Minimum 120 cfm/watt at medium speed setting.

$_______		$_________

CEILING

Approx 195 sf

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair cracks & water stains and paint.

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC.

$______		$______

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Install new hardware and paint. Primed once, with two-coat semi-gloss finish on all sides and faces. No VOC. All exterior doors shall have a lever key-lock latch and security accessories (eyelet peep hole and deadbolt).

$_______	$_________

FLOORS

Approx 195 sf

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Install carpet.	Carpet Padding: 7/16” thick, 6-lb. minimum re-bond polyurethane.

Sheet Carpet: 25 oz. minimum, 100 percent nylon. Other options include Berber type with blended fiber.

$_______	$_________

WALLS

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/Mildew

Repair damaged paneling

$_______	$_________

WINDOWS

Cracked/Broken/Missing/Panes/Sill/Caulking/Hardware/Paint/Water Stains/Screens

See Building Exterior						$_________ 		$_________

LAUNDRY AREA

Washer in Kitchen

LAUNDRY AREA

## Damaged/Missing/Dryer Vent/Sink/Plumbing/Exhaust Fan/Closet/Shelving

Add hot water line for washing machine

Potable water lines shall be copper or PEX material. Installation in exterior walls must be center in the insulation. Hose bibs must be of the freeze proof kind. All piping located in attic or crawl space shall be insulated. All hot water lines shall be insulated equal to or greater than R-4 pipe wrap.

$_______		$_________

GENERAL CONDITIONS

GENERAL CONDITIONS

Administrative/Fees/Insurance/Temporary/Rental Supplies/Mobilization

$_______		$_________

OVERHEAD & PROFIT

Reasonable and customary to base bid only

$_______	$_________

Project Total Costs:	$_______			  Total Costs:     $_______		$________

Tools/Equipment/Fuel/Maintenance/Travel Allowances/Benefits are not acceptable line item fees and will not be reimbursed or charged to the project.