HO-6B

REHABILITATION WORK WRITE-UP ESTIMATE

Owner:  Nadine Watson			 Date: 1-22-2016

Physical Address:  1806 Sunset Ave

City: Columbia	 Zip Code: 38401        Grant Year: 2012

Administrator: Maggie Fincher                              Grantee: City of Columbia

Year Built: 	1955 – Lead Levels Not Elevated                If pre-1978, attach LBP Assessment Report

Formatted in conjunction with the Uniform Physical Condition Standards Checklist

Rehabilitation     Lead Based Paint

BUILDING EXTERIOR

DOORS

Damaged/Frames/Thresholds/Hardware/Surface/Weather Stripping/Caulking/Storm Door

Add door and hardware at exterior storage area.

Exterior Doors shall be 1 3⁄4” thick 6 panel, steel material, insulated and be appropriately finished as recommended by the manufacturer. All opaque exterior doors shall have a U-factor equal to or less than 0.40

All exterior doors shall have a lever key-lock latch

$_________	$_________

FOUNDATIONS

Damaged/Cracking/ Crawl Space Access/Vapor Barrier/Insulation/Debris/Vermin

Remove and replace approx. 100 sf of floor insulation

All crawl spaces shall contain cavity floor insulation equal to or greater than R-19

Make new crawlspace access door panel and paint.

- Wood Exposed to Weathering Elements:  All exterior wood shall have a minimum preservative retention rate of 25 percent for above ground applications and a minimum preservative retention rate of 40 percent for all wood in contact with the ground.

$_________		$_________

PATIO/ PORCH/ BALCONY

Damaged/Missing/Baluster/Railing/Decking/Surface/Cover

Add approx. 12 lf of handrail and approx. 10lf of guardrail.

Wood Stairs and Handrails:  Exterior wood stairs shall be constructed with properly treated dimensional lumber. Handrails shall be constructed of composite, natural durable wood or pressure treated wood material.

Wood Exposed to Weathering Elements:  All exterior wood shall have a minimum preservative retention rate of 25 percent for above ground applications and a minimum preservative retention rate of 40 percent for all wood in contact with the ground.

$_________		$_________

WALLS

Damaged/Missing/Cracks/Gaps/Holes/Spalling/Peeling/Caulking/Chimneys

Replace approx.  1 sheet of exterior wall sheathing that is rotted. Re-install

Siding board that fell down.	Power wash and paint approx. 1300 sf of exterior siding and trim with minimum of primer and two finish coats of paint.

$_________		$_________

WINDOWS

Damaged/Missing/Broken/Frames/Panes/Sills/Lintels/Trim/Screens/Caulking/Paint

Replace 13 windows and associated jambs and sills.

All window frames must be solid vinyl. Double hung type to have both upper and lower sashes able to slide with a removable bug screen for lower section.

All glazing shall be double-paned with low E with Argon.

The vapor seal on the glazing must have a minimum ten-year  warranty.

All windows shall have a minimum one-year warranty on the operation of the window.

All windows shall have a National Fenestration Rating meeting minimum energy code requirements for Zone 4.

All windows shall have a U-Factor equal to or less than 0.40 and an SHGC rating equal to or less than 0.40.  Division 9: Finishes

$_________		$_________

BUILDING SYSTEMS

WATER HEATER

Inoperative/Leaking/Rust/Corrosion/Missing/TPRV/Vent/Chimney/Combustion Air

Repair leaking pipes and install discharge pipe

Attach water heater electrical cover.

$________	 	$_________

HVAC

Inoperative/Noisy/Vibrating/Leaking/Rust/Corrosion/Missing/Vent/Chimney

Re-attach one piece of HVAC ductwork in crawlspace

$_________ 		$_________

BATHROOM

BATHROOM

Cabinets/Top/Sink/Plumbing/Tub/Shower/Ventilation/Water Closet/Accessories

Repair dripping sink faucet. Repair loose plumbing at shower controls.

$________		$_________

ELECTRICAL SYSTEM

Inoperative/GFCI/ Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Install GFCI

$________		$_________

FLOORS

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Remove approx. 46 sf of existing flooring and install with new sheet vinyl flooring.

Sheet Vinyl: Shall be Armstrong or equivalent minimum 10 mil wear layer. Provide product adhesive and underlayment as recommended by the manufacturer. All surfaces shall be clean, dry, and appropriate temperature during installation. Vinyl sheet flooring shall conform to the requirements of ASTM F 1303, Type I.

$_________		$_________

KITCHEN

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Remove and replace 36” exterior door and associated trims. Paint door.

Primed once, with two-coat semi-gloss finish on all sides and faces. No VOC

$________		$_________

ELECTRICAL SYSTEM

Inoperative/GFCI/Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Install GFCI								$_________		$_________

WALLS

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/Mildew

Repair approx. 2 sf of water damaged drywall inside of cabinet from previous water leak. Tape, mud and sand smooth. Paint wall. Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC.

$_______		$_________

WINDOWS

Cracked/Broken/Missing/Panes/Sill/Caulking/Hardware/Paint/Water Stains/Screens

See Building exterior						  $_________ 		$_________

BEDROOMS

CEILING

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Bedroom #1 – Repair approx. 1-1/2” hole in ceiling. Tape, mud and sand smooth. Paint approx. 121 sf of ceiling. Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$_________	 		$______

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Bedroom #1 - Adjust door hinges to allow for ease of shutting.

Bedroom #2 – Adjust door so door will latch.

$_________	$_________

ELECTRICAL SYSTEM

Inoperative/Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Bedroom #1 - Install globe on light fixture.

$_________		$_________

FLOORS

Bulging/Buckling/Damaged/Missing Covering/Tile/Deteriorated Subfloor/Water Stains

Bedroom #1 - Remove and replace with new approx. 5 pieces of hardwood flooring 2’ long and approx. 3 sf of ¾” thick subflooring.

$________		$_________

SMOKE DETECTOR

Damaged/Missing/Inoperative/Non-Compliant

Bedroom #1,2 & 3 - Install smoke detector

$_________		$_________

WALLS

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/ Mildew

Bedroom #1 – Repair approx. 12lf of cracks over doors and windows.

Remove and replace approx. 6 sf of damaged sheetrock and 4lf of damaged baseboard below window. Tape, mud and sand smooth. Paint approx. 352 sf of walls.

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

Bedroom #2 – Repair approx. 6lf of cracks over windows. Tape, mud and sand smooth. Paint approx. 384 sf of walls. Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC

$_________	 	$_________

WINDOWS

Cracked/Broken/Missing/Panes/Sill/Caulking/Hardware/Paint/Water Stains/Screens

See building exterior							$_________ 		$_________

HALLWAY

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Replace inoperative lock at door.

$_________		$_________

LIVING ROOM

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Replace missing hardware at front door. Replace damaged storm door.

$_______		$_________

GENERAL CONDITIONS

GENERAL CONDITIONS

Administrative/Fees/Insurance/Temporary/Rental Supplies/Mobilization

$_________		$_________

OVERHEAD & PROFIT

Reasonable and customary to base bid only

$_________		$_________

Project Total Costs:	$_________			  Total Costs:     $_________		$________

Tools/Equipment/Fuel/Maintenance/Travel Allowances/Benefits are not acceptable line item fees and will not be reimbursed or charged to the project.