HO-6B

REHABILITATION WORK WRITE-UP ESTIMATE

Owner:     Patricia Webster                                                         Date: 1-20-2106

Physical Address: 2215 Williamsport Pike

City: Columbia                                 Zip Code: 38401       Grant Year: 2012

Administrator:  Maggie Fincher                             Grantee: City of Columbia

Year Built: 1977  -Lead Levels Not Elevated             If pre-1978, attach LBP Assessment Report

Formatted in conjunction with the Uniform Physical Condition Standards Checklist

BUILDING EXTERIOR

FOUNDATIONS

Damaged/Cracking/ Crawl Space Access/Vapor Barrier/Insulation/Debris/Vermin

Re-point approx. 15lf of cracks in foundation walls                                    $______	$_________

ROOFS

Approx. 11.5 sq

Damaged/Soffit/Fascia/Vents/Drains/Membrane/Shingles/Gutters/Downspout/Ponding

Remove existing shingles and replace with new. Replace one sheet of roof sheathing. Repair approx. 4lf of damaged soffit and fascia. Replace approx. 10 lf of gutter.

Shingles: 215 lb. seal tab type over 30 lb. felt with metal drip edge,  with minimum 25-year product warranty

$________	$_________

BUILDING SYSTEMS

ELECTRICAL SYSTEMS

Access/Damaged/Missing/Breakers/Covers/Wires/Corrosion/Evidence Water Leaks

Replace missing breaker or cover in breaker box.			  $_________	         $_________

WATER HEATER

Inoperative/Leaking/Rust/Corrosion/Missing/TPRV/Vent/Chimney/Combustion Air

Install new water heater.						$________		$_________

Water Heater shall be minimum 40 gallon electric with a minimum Energy Factor (EF) equal to or greater than .92. Gas fired water heaters shall an EF equal to or greater than .59. Instantaneous gas water heaters shall have an EF equal to or greater than .94 (96%AFUE). Prefer direct vent on gas water heaters or sealed combustion.

SANITARY SYSTEMS

Broken/Leaking/Clogged/Missing/Cleanout/Septic System

Remove and replace septic tank and lines				$_________		$_________

HVAC

Inoperative/Noisy/Vibrating/Leaking/Rust/Corrosion/Missing/Vent/Chimney

Install new HVAC unit.						$_________		$_________

All units shall have a central HVAC system comprised of a split system heat pump with emergency heat strips for heating and cooling using high- efficiency equipment. Units with gas shall be a split system air conditioner and furnace. Heat pump/ AC systems shall have a minimum SEER (Seasonal Energy Efficiency Rating) rating of 14.5 with a minimum HSPF (Heating Seasonal Performance Factor) rating of 8.5. Fuel oil, gas fired furnaces and boilers shall have an Annual Fuel Utilization Efficiency (AFUE) equal to or greater than ninety-two percent (92%). Electric- resistance-only heat systems are prohibited. Alternative HVAC systems may be approved by THDA. Prefer direct vent on gas furnace or sealed combustion.

BATHROOM

BATHROOM

Cabinets/Top/Sink/Plumbing/Tub/Shower/Ventilation/Water Closet/Accessories

Replace vent fan.						$________			$_________

CEILING

Approx. 40 sf

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair approx. 4lf of drywall tape and paint ceiling

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC. 						$_________		$_________

ELECTRICAL SYSTEM

Inoperative/GFCI/ Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Add GFCI breaker in panel box     					$_________		$_________

WALLS

Approx. 208 sf

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/ Mildew

Repair approx. 4 lf of drywall tape and paint walls

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC. 					$________	 		$_________

KITCHEN

KITCHEN

## Damaged/Missing/Cabinets/Countertop/Sink/Faucet/Plumbing/Appliances

Replace oven/cooktop.  An electric range shall be minimum 30” 4 burner with self-cleaning oven provided in all dwelling units to match manufacturer and model line with the refrigerator and dishwasher on all projects including new construction and rehabilitation projects where identified in the PCNA (physical/capital needs assessment).     			     				              $_______	 		$_________

CEILING

Approx. 192 sf

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair water stains and paint ceiling. Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC.       $_______	$_________

ELECTRICAL SYSTEM

Inoperative/GFCI/Damaged/Missing/Wire/Cover Plates/Light Fixture/Evidence Water Leaks

Add GFCI breaker in breaker box.				$_______			$_________

FLOORS

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Add one piece of 2x8x12 bracing under sub-floor

For bracing.							$_______	 		$_________

WALLS

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/Mildew

Replace one piece of damaged paneling next to stove and paint.

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC. 					$________	 		$_________

BEDROOMS

CEILING

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Bedroom #2 – Repair approx. 4lf of drywall tape then paint approx. 120 sf of ceiling.

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC. 						$_________		$_________

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Bedroom #1 – Adjust door latch to allow door to close. 	$________			$_________

SMOKE DETECTOR

Damaged/Missing/Inoperative/Non-Compliant

Bedrooms #1, 2&3. Add smoke detectors.			$________	 		$_________

WALLS

Bulging/Buckling/Damaged/Trim/Paint/Water Stains/Mold/ Mildew

Bedroom #2 – Repair 2’ crack over closet door and approx. 4lf

Of drywall tape then paint approx. 352 sf of walls.		$________	 		$_________

HALLWAY

CEILING

Approx. 40 sf.

Bulging/Buckling/Cracks/Holes/Peeling/Damaged/Missing/Tile/Panels/Water Stains

Repair approx. 6lf of drywall tape then paint ceiling

Primed once and two finish coats of flat, eggshell or satin. Use semi-gloss, or satin finish for bathrooms, laundry, and kitchens. No VOC. 					$_________			$_________

LIVING ROOM

FLOORS

Bulging/Buckling/Damaged/Missing/Covering/Tile/Deteriorated Subfloor/Water Stains

Install 3 pieces of 2x8x12 under sub-floor for bracing        $________			$_________

LAUNDRY AREA

DOORS

Damaged/Missing/Frames/Threshold/Lintel/Hardware/Surface/Paint

Replace door trim for 36” door and paint.				$_________		$_________

GENERAL CONDITIONS

GENERAL CONDITIONS

Administrative/Fees/Insurance/Temporary/Rental Supplies/Mobilization

$_______		$_________

OVERHEAD & PROFIT

Reasonable and customary to base bid only

$_________		$_________

Project Total Costs:	$_________			  Total Costs:     $_________		$________

Tools/Equipment/Fuel/Maintenance/Travel Allowances/Benefits are not acceptable line item fees and will not be reimbursed or charged to the project.