REPAIR SCOPE OF WORK and REQUEST FOR QUOTATION

Location:

Bridge Location No. 61-0A022-1.06

Henley Mill Bridge, Meigs County, Decatur, TN

TDOT Bridge #61-OA022-01.06

Current Conditions:

Due to deterioration of specific eyebar connections on the truss-style bridge built in 1884, the Tennessee Department of Transportation has closed the Henley Mill Bridge to all vehicle traffic.

Repair / replacement of specific components of the bridge will be required prior to the bridge being approved for reopening to public traffic by the TDOT.  The following and attached pages provide specifications and request for quote for the required bridge repairs.

Bridge Geometry:

- Roadway Width:	11’-1”

- Structure Length:	99’-0”

- Span Lengths:		1 @ 99’-0”

Type of Construction:	This structure consists of one steel truss and floorbeam system span with a timber deck. The substructure is masonry.

Repair Scope of Work:

- There is heavy corrosion on the eyebar at LEL connection.  Replace this eyebar

- Replace eyebars L3-U3, L4-L5, and L3-L4

- Replace other eyebars that have stretched over time (provide list of intended replacements)

- Replace diagonal round tension bars and round horizontal tension bars and provide method to prevent round rods from rubbing against each other

- Provide visual inspection of all pins, replace as required

- Tighten all loose connections and members

- Repair all poor connections

- Cut and remove vegetation from around truss

Engineering:

Prepare and provide Tennessee Registered Professional stamped drawings of all replacement components.  Prepare and submit to the TN Department of Transportation for approval a work plan for TDOT approval of all fabricated components prior to commencement of work.

Reference Materials:

- TDOT Bridge Maintenance Recommendations dated 6.23.2022

- TDOT Bridge Evaluation Report (BER) dated 6.28.2022

- Repair Material Specifications 2022.09.14.0

Request for Quotation:

Provide all required engineering, materials, equipment, labor, and supervision to perform minimum repairs to the Henley Mill Bridge per the attached reference materials.  Repair materials and procedures must be approved by TDOT prior to commencement of work.

Quote should include the following:

- Engineering Costs

- Material Costs

- Labor Costs

- Estimated time required to complete engineering and repairs ARO

** All prep work, labor, materials, and clean-up is the responsibility of the contractor.

** Proof of license and insurance required with bid

** Signed copy of Bid Terms and Agreement required with bid

**Deadline for submitting bids is December 5, 2022 at 10:00am. Late bids will not be accepted for any reason. Public opening will follow immediately in the mayor’s office on the 1st floor of the Meigs county courthouse at 17214 State Hwy 58 N Decatur, TN 37322

**Bids can be delivered by carrier or personal delivery to the Meigs county Finance office on the 1st floor of the Meigs county courthouse at 17214 State Hwy 58 N Decatur, TN 37322. Bids may also be delivered via mail at PO BOX 38 Decatur, TN 37322.

**Meigs county is not responsible for late/delayed bids due to carrier or mail issues.

**Contact the Finance office for additional information or questions (423)334-5846 or financedirector@meigscounty.org