SPECIFICATIONS

LAKE CITY FIRE DEPARTMENT

REPLACE ROOF

Overview

The City of Lake City is seeking bids to replace the roof of the Lake City Fire Department building located at 233 North Acline Street, Lake City SC  29560.

Scope of Work:

- Remove existing roofs, coping caps, and flashing membrane from parapet wall and dispose of debris.  NOTE: The City of Lake City will waive C&D landfill tipping fee for contractor to dispose of the debris.

- Install appropriate Polyisocyanurate rigid insulation per manufacture’s standard details.

- Install fully adhered .060 TPO roofing system.

- Flash all parapet walls, curbs and pipes as per manufacture’s standard details.

- Install 24-gauge drip edge and coping caps (Metal and caulk colors to match existing as closely as possible)

- Install 24-guage gutters and downspouts (Metal and caulk colors to match existing as closely as possible).

- Construct insulation crickets between roof drains to provide positive drainage.

- Provide 2 years contractor’s warranty.

- Provide a 20 year manufacturer’s warranty.