City Of Brunswick Fire Dept


THE CITY OF BRUNSWICK, GEORGIA
JANUARY 2, 2017

FIRE DEPARTMENT RESCUE VEHICLE
RFP (#E-15)
[image: image1.png]




The Brunswick Fire Department is accepting bids for the following Recue Vehicle specifications. The City reserves the right to reject any or all proposals, waive technicalities and make the award in the best interest of the City.

The City anticipates making a single award; however, it reserves the right to make multiple awards should it deem it in the best interest of the City.  Such an award, if any, is projected to be accomplished within sixty (60) days from the proposal opening.

Any questions and/or specifications that may need clarification should be submitted in writing, and forwarded to the Purchasing Agent at the below mailing address or email address.  All quotes must be submitted to the below address by 3:00 p.m. on Friday January 26, 2018, late quotes WILL NOT BE ACCEPTED.

The envelopes containing the quote must be sealed, and addressed to:

J. Aaron Dunham Jr.

Purchasing Agent 

City of Brunswick

601 Gloucester Street, City Hall

Brunswick, Georgia 31520

ADunham@cityofbrunswick-ga.gov
All proposals must be marked “Fire Department Rescue Vehicle” RFQ (#E-15). The envelope must bear on the outside the name and address of the Service provider.  No bid may be withdrawn or modified in any way after the deadline for quote openings, and no faxed or electronic submissions of quotes will be accepted.

Bids will be publically opened approximately 30 minutes after the deadline for bids, and only the name(s) of those service providers responding will be mentioned.  All service providers are invited to be present during the bid opening, however this is not mandatory.  Bids must be valid for ninety (90) days following the opening date.  

· End of This Section
SCOPE AND GENERAL REQUIREMENTS 
It is the intent of the manufacturer to provide a new fire apparatus that will withstand the continuous use encountered in the emergency fire fighting service.  The apparatus shall be of the latest type, symmetrically proportioned and constructed with due consideration of the load to be sustained. 
All parts not specifically mentioned herein, but which are necessary in order to furnish a complete fire apparatus, shall be furnished and shall conform to the best practices known to the fire apparatus industry. 
The apparatus shall comply with all Federal, State, and local requirements pertaining to vehicles used for fire fighting, and emergency vehicles at time of contract signing.  The apparatus must also comply with all requirements as specified in N.F.P.A. standards that are applicable on date of contract signing.  The unit is to be of current year manufacture, and is to be new and unused.  The bid price shall not include any local, State, or Federal taxes.  The Bidder shall not be liable for any State or Federally mandated tax or program after the sale of this apparatus. 
These specifications shall be construed as minimum.  Should the manufacturer's current published data or specifications exceed these, they shall be considered minimum and be furnished.
PRIME BIDDER, MANUFACTURER
The manufacturer shall be prime bidder and shall identify the location of their facility.
BIDDERS BACKGROUND 

Bids are requested from responsible manufacturers who are engaged in the manufacture of fire apparatus.  To insure reliable and complete acceptance of the apparatus, bidder shall have been in operation for a minimum of twenty (20) years in the manufacturing of fire apparatus.

The manufacturer of the apparatus must be fully owned and managed by a Parent Company, Corporation, or Individual(s) that is 100% held by United States of America based Company, Corporation, or United States citizen(s).

Proposals from any manufacturer that is fully or partially owned and/or operated by a foreign company, Corporation or Individual(s) under any type of ownership, partnership, or any similar type of agreement will be immediately rejected.

If the manufacturer of the apparatus, or if any owner, shareholder, or immediate relative of an owner or shareholder that has previously been involved in or held ownership in any company that has filed bankruptcy or any other type of reorganization plan, it must be clearly stated in the bid proposal.  The statement must include details and dates of all occurrences.

FAMA COMPLIANCE
The apparatus manufacturer must be a current member of the Fire Apparatus Manufacturer's Association (FAMA) and must provide certificate of membership.  

PROPRIETARY PARTS

It is the intention of the purchaser for all bidders to furnish the apparatus with major parts commonly used by the heavy-duty truck manufacturers and open market vendors where as replacement parts are more readily available and at reduced cost.  The use of proprietary parts may not be acceptable to the purchaser. 

MANUFACTURER'S DISCRETION
Materials, parts, or procedures used are subject to change at manufacturer's discretion at any time to provide equal or better products.

COOPERATIVE PURCHASING
The manufacturer shall be pleased to allow other public agencies to use the purchase agreement resulting from this invitation to bid unless the bidder expressly notes on the proposal form that prices are not available for tag-on.  The condition of such use by other agencies shall be that any such agency must make and pursue contact, purchase order/contract, and all contractual remedies with the successful bidder.  Such tag-ons shall be done so that the purchaser has no responsibility for performance by either the manufacturer or the agency using the contract. 

FAIR, ETHICAL AND LEGAL COMPETITION
In order to ensure fair, ethical, and legal competition the apparatus manufacturer shall have ever been fined or convicted of price fixing, bid rigging, or collusion in any domestic or international fire apparatus market.   

PRODUCT QUALITY AND WORKMANSHIP

The components provided and workmanship performed shall be of the highest quality available for this application.  Special consideration shall be given to the following areas:

A).  Accessibility to various components that require periodic maintenance or lubrication checks.

B).  Ease of vehicle and pump operation.

C).  Features beneficial to the intended operation of the apparatus.

Construction of the complete apparatus shall be designed to carry the loads intended to meet the road and terrain conditions and speed requirements desired when specified by the purchaser.

Welding shall not be employed in the assembly of the apparatus in a manner that will prevent the removal of any major component part for service and/or repair.

INSURANCE REQUIREMENTS

Each bidder must submit with their bid proposal a Certificate of Insurance listing the proposed manufacturer's product liability insurance coverage.  Liability insurance shall be a minimum amount of ten (10) million dollars.  Submitted certificate shall name the apparatus manufacturer, insurance company, policy number, and effective dates of the insurance policy.  Bids submitted without the required certificate will be considered non responsive and automatically rejected.  No exceptions are allowed to the minimum insurance coverage requirement. 
The manufacturer shall maintain full insurance coverage on the purchaser's cab and chassis from time of first possession by the manufacturer until the apparatus is delivered and accepted by the purchaser (No Exceptions).  Purchaser reserves the right to require proof of insurance from the manufacturer's insurance carrier prior to entering into a contract for the apparatus.

PAYMENT TERMS

Full payment for the apparatus shall be made at time of delivery of the completed vehicle.  Due to insurance liability, the apparatus will not be left at the purchaser's location without full acceptance and payment or prior agreement between the Purchaser and Bidder.

Final delivery price shall not include any Local, State or Federal taxes.  The manufacturer shall not be liable for any State or Federal mandated tax or program after sale or delivery of the apparatus.

VEHICLE ACCEPTANCE AND DELIVERY
The customer shall pickup the vehicle at the manufacturing facility and shall supply evidence of sufficient insurance coverage to transport the vehicle.

FUEL TANK FILLED AT DELIVERY

The fuel tank and DEF tank (if applicable) shall be filled upon final delivery at the factory.

ONE YEAR APPARATUS WARRANTY

The complete apparatus detailed herein shall be warranted against defects in materials and workmanship for a period of twelve (12) months, effective upon pick up or delivery of the completed apparatus to the purchaser, as detailed in the respective warranty documents.  Any unauthorized alterations or modifications to the apparatus shall void this warranty.

Other warrantees, as provided by individual component manufacturers may extend beyond this warranty.

APPARATUS BODY WARRANTY, TEN YEAR 

The apparatus body as detailed herein shall have a structural warranty against defects in materials and workmanship for a period of ten (10) years, effective upon final payment in full by the Purchaser, and pick up or delivery of the completed apparatus to the Purchaser.  Any unauthorized alterations or modifications to the body shall void this warranty.

PAINT WARRANTY, FIVE YEAR 

The finish paint as used on the proposed apparatus shall be warranted against defects in materials and workmanship for a prorated period of five (5) years, effective upon final payment in full by the Purchaser, and pick up or delivery of the completed apparatus to the Purchaser.  Any unauthorized alterations or modifications to the apparatus shall void this warranty.

APPARATUS ELECTRICAL WARRANTY, TWO YEAR 

The apparatus electrical system as detailed herein shall have a electrical warranty against defects in materials and workmanship for a period of two (2) years, effective upon final payment in full by the Purchaser, and pick up or delivery of the completed apparatus to the Purchaser.  Any unauthorized alterations or modifications to the electrical system shall void this warranty.

APPARATUS DIMENSIONS

These are standard truck dimensions.  Changes in configuration or additional options may affect these dimensions.  The contract specification shall contain the exact dimensions.

OVERALL HEIGHT

The overall height shall be less than 96".

OVERALL LENGTH

The overall length shall be no longer than 26 feet.

OVERALL WIDTH

The overall width of the body shall be 78 inches wide; chassis mirrors will extend out past this width.

ANGLE OF APPROACH

The angle of approach for the apparatus shall not be less than eight (8) degrees as specified by the current edition of NFPA 1901/ 1906.

ANGLE OF DEPARTURE

The angle of departure for the apparatus shall not be less than eight (8) degrees as specified by the current edition of NFPA 1901/ 1906.

COMMERCIAL CHASSIS SPECIFICATION

CHASSIS PROVIDER

The chassis, as detailed in these specifications, shall be ordered and supplied by the apparatus manufacturer.

CHASSIS

One (1) new 2017 FORD F-250 rear axle drive 4x2, dual rear wheels (SRW), four (4) door crew cab and chassis with XL trim. Or comparable major brand manufacturer.
Wheelbase: 

179"

Cab to Axle:

60"

GVWR: 10,000 pounds

Steering: Hydraulic power-assist re-circulating ball

Non-Driving Front Axle and Suspension: HD front package, firm ride suspension package, Twin I-Beam front suspension, anti-roll bar, HD front shocks, HD front coil springs 

Rear Axle and Suspension: wide track, rigid rear axle, HD rear suspension package, anti-roll bar, HD rear leaf springs, HD rear shocks 

Braking System: four (4) wheel disc brake system with front and rear vented discs and Anti-Lock (ABS) 
POWERTRAIN

6.2L, 2-Valve SOHC EFI NA V8

Rated Brake Horsepower: 316 hp @ 5500 rpm

Rated Torque- 397 @ 4500 rpm

Battery: 650 CCA, 72 AH

Exhaust System: horizontally mounted, discharge on right side aft of wheels 

Transmission: Six speed automatic with overdrive.
Alternator:  200 Amp extra heavy-duty

REAR AXLE RATIO 

The ratio of the rear axle shall be the standard ratio supplied by the chassis manufacturer. 3:73

CAB TYPE

Conventional, engine forward, four (4) door crew cab 

Construction: Welded steel 

Accessories: 


- Solar Tinted glass in all windows 


- Dual sun visors 


- Electric windshield washer 


- Dome light 


- Fresh air heater and defroster 


- Dual electric horns 


- Driver and passenger air bags


- Gray Vinyl Upholstery 


- Roof Clearance Lights


- Black vinyl full floor covering


- 12V Auxiliary Power Point

Headlamps:  dual beam jewel effect LED

Climate Controls: controls for heat, defroster, and air conditioning 

Mirrors: black manually telescope fold-away in/out for view adjustment. 

Instrumentation:


- Tachometer


- Speedometer


- Turbo boost (diesel only)


- Oil pressure


- Coolant temperature


- Fuel gauge 


- Transmission temperature gauge


- Indicator lights & Message Center/odometer, trip odometer, engine hour meter & warning messages.

CAB SEATING

The front seating shall consist of a heavy duty vinyl 40/20/40 split front bench seat w/center armrest, cup holder, storage and manual driver-side lumbar support

REAR CAB SEATING

The rear seating shall consist of a heavy duty vinyl 60/40 bench flip-up/fold-down rear seat.

AUDIO

AM/FM stereo, w/digital clock & 2-speakers

GRILLE

Black painted

BUMPER

Chrome Steel with grained MIC top cover.

Tire and Wheels: F250                                                           

WHEELS

Four (4) 17.00" Polished Forged Aluminum Wheels with Bright Hub Covers

TIRES

Four (4) LT245/75Rx17E black side-wall all-terrain tires shall come supplied from ford with the chassis.

FORD SUPERDUTY WARRANTY

Description 







Months/Distance
Basic...................................................................................36 month/36,000 miles

Powertrain...........................................................................60 month/60,000 miles

Corrosion Perforation..................................................60 month/unlimited mileage

Roadside Assistance .........................................................60 month/60,000 miles

Gasoline Engine ..............................................................60 month/100,000 miles

CHASSIS PAINT COLOR

The cab shall be painted a single color by the chassis manufacturer.

Color: Ford Race Red
Paint Number: PQ
STEP BARS

One (1) set of polished stainless steel step bars shall be mounted to each side of the chassis.  There shall be a slip resistant step pad located bellow each chassis door.

FRONT MOUNTED WINCH

A WARN model M12000 electric winch with 12,000 pound (5,440 kg) rated line pull shall be installed in the brush guard.  The winch shall be equipped with 125.00' of 3/8" diameter wire rope, clevis hook and a 4-way roller fairlead.  The winch shall be operated through a 12.00' pendant with a hand held control.  The winch shall include an automatic mechanical cone brake.  It shall feature an easy to use free-spooling rotating ring gear clutch.
The installation shall maintain access to the winch controls. 

GRILLE GUARD - STAINLESS STEEL

A Warn model 85060 Trans4mer Gen II grille guard shall be provided.   The grille guard kit shall be provided with a stainless steel finish.

HEADLAMP GUARD - STAINLESS STEEL

A Warn model 84980 Trans4mer Gen II headlamp guard kit shall be provided and installed.  The headlamp guard kit shall attach to the grille guard.  The headlamp guard kit shall be provided with a stainless steel finish.

WINCH CARRIER - LARGE FRAME - BLACK FINISH
A Warn model 90110 large frame winch carrier shall be installed in grille guard.  It shall be capable of carrying Warn winch models 16.5ti, M15, M12, and M8274-50.

The winch carrier shall have a powder coated black finish. 

TOWING HITCH RECEIVER
A trailer towing hitch receiver with safety chain anchors shall be installed at the rear of the apparatus.

The hitch receiver shall be constructed of heavy steel tubing and reinforced to the apparatus framework.  The hitch receiver shall have a Class V rating of 16,000 pounds towing and 1,600 pounds tongue weight when used with a weight distributing hitch assembly.  
The receiver shall accept a 2.00" hitch.

One (1) 7-prong connector with a weatherproof cover shall be supplied and mounted near the rear receiver tube.

SEAT BELT CLARIFICATION 

Red seat belts shall be provided if available from the chassis manufacturer.  The apparatus manufacturer shall not change commercial chassis seat belts. 

TIRE PRESSURE MONITORING

The apparatus shall be provided with tire pressure indicating valve stem caps.  The indicators shall be installed on each tire and be a heavy duty design manufactured specifically for trucks.  When tire is properly inflated, the indicator inside the cap shall be green, and when the tire is underinflated by 10%, the indicator inside the cap shall be red. 
CHASSIS TOW HOOKS 

The front tow hooks shall be supplied by the chassis manufacturer.
REAR TOW PLATES 
Two (2) rear tow plates with 1-1/2" I.D. holes, constructed with 1" steel plate shall be provided on the apparatus.  These shall be located below the apparatus body and fastened to the rear chassis frame rails.  
The tow plates shall be painted black.

REAR MUD FLAPS

A pair of black rubber mud flaps, with the Manufacturer's logo, shall be provided and installed behind the rear wheels.

CENTER CONSOLE
A center console fabricated from 1/8" aluminum shall be furnished and shall be located between the driver and officer's seats. 
The forward area of the console shall have a mounting surface for emergency lighting switch panels and/or electronic siren control boxes within reach of the driver or officer.  

MASTER BODY DISCONNECT SWITCH

A master body disconnect on/off switch shall be provided in the cab, near the driver’s door.  The switch shall disconnect the power to the apparatus body when the ignition switch is in the off position.

One (1) reset breaker shall be installed between the solenoid output and any electrical load.

One (1) indicator light shall be provided to indicate the apparatus 12-volt system is on.  The light shall be located in the chassis cab and be visible from the driver’s positions.  The light shall be green in color and labeled “Master Battery Disconnect”.

BATTERY CHARGER 
A PRO MARINER / ON BOARD SOLUTIONS, 1240, advanced electronic 4-step battery charger/power supply with a 40 amp output shall be installed, under the driver's seat.  
Since shoreline power is not always stable the charger shall be equipped with Auto-Ranging AC Input to automatically accept global voltages of 90 VAC to 270 VAC at 45-440 Hz.
Field Selectable - Use with lead/acid or gel batteries (AGM factory option). Select length of absorption charge cycle based on size of batteries.

In the 4-step charging system the charger will provide the following sequence.

Step 1: Fast Charge - Charger will deliver its maximum amperage rating to the connected batteries for the fastest charge (current regulation mode) until battery voltage is raised to 14.6V (lead acid factory setting).   At this time, the ProTech will shift to step 2. 

Step 2: Absorption Charge - Maximizes charge and holds voltage (voltage regulation mode) at 14.6V (lead acid factory setting) for 1 to 4 hours (selectable based on battery size), while letting the batteries determine the amount of amps they can accept.  This mode creates activity in the batteries, reducing sulfate buildup, and conditions the batteries for an extended life.  After the programmed 1 to 4 hours have elapsed, the ProTech will shift to step 3. 

Step 3: Float Mode - A precision 13.3V (lead acid factory setting) finishing voltage that maintains each battery (step-down voltage regulation mode), which is perfect for short or long storage periods and will never overcharge your batteries.  ProTech will deliver its full rated output for house loads including: lighting, electronics and pumps.

Step 4: Recycle - If there are very large loads on the battery while the charger is on, the unit will recycle to the first step, ensuring that batteries stay fully charged.
120 VOLT SHORELINE CONNECTION - "SUPER" AUTO EJECT
One (1) Kussmaul "Super" Auto Eject model  091-55-20-120, automatic, 120 volt, 20 amp shoreline disconnect shall be provided for the on board, 110 volt battery charging systems. 
The Kussmaul auto-eject connection shall be equipped with a Yellow weatherproof cover. model # 091-55YW. 

The disconnect shall be equipped with a NEMA 5-20P male receptacle, which shall automatically eject the shoreline when the vehicle starter is energized.  The mating connector shall be included with the auto eject and shall be provided as loose equipment.

SHORELINE POWER INLET PLATE
A shoreline power receptacle information plate shall be permanently affixed at or near the power inlet.  The plate shall indicate the following:

· Type of Line Voltage
· Current Rating in Amps Power Inlet Type (DC or AC).
BACK-UP ALARM

One (1) 97 DB back up alarm shall be provided and installed at the rear of the unit.  It shall be wired to activate when the transmission is placed in reverse.

APPARATUS BODY

BODY DESIGN
The body shall be modular in design, allowing it to be removed and remounted on a new chassis. 

The fabrication of the body shall be formed sheet metal.  Formed components shall allow the Purchaser to have the body repaired locally in the case where any object has struck the body and caused damage. The use of proprietary extrusions will prevent the Purchaser from such repair and shall NOT be used.   

SUBFRAME

The body shall be attached to and supported by a heavy duty, spring loaded, and aluminum sub frame u-bolted to the truck frame.  The sub frame shall be spring mounted to the chassis frame to allow for independent flexing of the body in relation to the chassis frame. 

BODY CONSTRUCTION

The body shall be fabricated using aluminum extrusions, angle, smooth aluminum sheet and aluminum tread plate providing a durable, corrosion resistant, and lightweight body.  All seams shall be welded.

The entire body shall be a welded into a one piece module, assembled and painted prior to mounting on the chassis.

COMPARTMENT CONSTRUCTION

The compartments shall be constructed of .125" 5052-H32 Aluminum.  All seams shall be completely welded.  Divider walls between compartments shall be of single wall construction with a minimum wall thickness of 0.125".

The rear compartment floor shall be constructed of .125" 3003-H22 Aluminum Tread Plate.

For adequate ventilation and air displacement, each compartment shall be properly louvered with square vents.  The rearmost wall of the rear compartments shall have full height removable panels, constructed of aluminum, to cover and protect all 12 volt electrical accessories mounted on the walls.  The panels shall be removable to provide access to those components.

FENDER PANELS

Side fender panels above the rear wheels shall be 0.125" 5052-H32 Aluminum.

BODY CONFIGURATION

The apparatus body shall be 108.00" long.

BODY WIDTH

The apparatus body shall be 76.50" wide.

COMPARTMENT DOORS
All side compartment doors shall be double-paneled, and designed to fit flush with the side of the apparatus body.
The exterior panel of the door shall be pan formed, shall be a minimum of 1.25" thick, and shall be constructed of minimum .063" smooth aluminum 5052-H32 Aluminum alloy.  Each door shall have full length aluminum hat section reinforcement between panels.

Vertically-hinged doors shall have nitrogen strut door holders with internal dampeners that shall hold the door open at 90 degrees.

Horizontally-hinged doors shall be secured in the open position with a gas cylinder. 

Each door shall have an automotive quality bubble type seal with a high bond PSA heat-sealed backing mounted on the door to keep dust and water out of the compartment and prevent metal to metal contact between the door and the door frame.
Also each door shall have die-formed v-channels that mesh with vertical and horizontal door frame baffles to direct water away from compartment.

COMPARTMENT DOOR LATCHES
Flush mounted 2-point paddle style door latches shall be installed on each door.  They shall polished stainless steel for maximum corrosion protection.

COMPARTMENT SHELF TRACKS - ALUMINUM  
Heavy-duty aluminum Uni-Strut tracks shall be provided in the compartments as specified.  The tracks shall not be welded to the apparatus body.  The Uni-strut tracks shall allow the shelving to be positioned at any location in the compartment by simply loosening a bolt on each end of the shelf, pushing inward on the bolt, and sliding the shelf to the desired location.

COMPARTMENT SHELVING - SIDE COMPARTMENTS 
Adjustable shelving shall be installed in the side compartments as identified later in this specification.  Each shelf shall be made of 0.125” smooth aluminum with a 2.00” high perimeter retaining lip with welded corners. 

BODY HEIGHT
The overall height of the apparatus body shall not exceed 41.00".

LEFT SIDE COMPARTMENTS

LEFT SIDE COMPARTMENT IN FRONT OF REAR WHEELS, L1

There shall be one (1) full height compartment ahead of the rear wheels, approximately 29.25" wide x 35.25" high x 14.00" deep.

One (1) louver vent shall be installed in the compartment.

ADJUSTABLE SHELVING TRACKS

There shall be two (2) uni-strut channels, one (1) per side, installed in the compartment.

ADJUSTABLE SHELF(S)

There shall be One (1) adjustable shelf constructed of 0.125” aluminum with a 2.00” upward bend at front and rear, and side supports.

COMPARTMENT LIGHT

One (1) surface mount LED light shall be installed in the compartment.  The light shall have a clear lens and shall a minimum effective output of 810 lumens.

The lights shall function independently of other compartments by an automatic "On-Off" switch located on each compartment door.

LEFT SIDE ABOVE WHEEL COMPARTMENT, L2

There shall be one (1) standard height compartment above of the rear wheels, approximately 45.5" wide x 14.00" high x 14.00" deep.

The compartment shall be cross vented into the full height compartments

ADJUSTABLE SHELVING TRACKS

There shall be two (2) uni-strut channels, one (1) per side, installed in the compartment.

ADJUSTABLE SHELF(S)

There shall be One (1) adjustable shelf constructed of 0.125” aluminum with a 2.00” upward bend at front and rear, and side supports.

COMPARTMENT LIGHT

One (1) surface mount LED light shall be installed in the compartment.  The light shall have a clear lens and shall a minimum effective output of 810 lumens.

The lights shall function independently of other compartments by an automatic "On-Off" switch located on each compartment door.

LEFT SIDE COMPARTMENT BEHIND REAR WHEELS, L3

There shall be one (1) full height compartment behind of the rear wheels, approximately 20.75" wide x 35.25" high x 14.00" deep.
One (1) louver vent shall be installed in the compartment.

ADJUSTABLE SHELVING TRACKS

There shall be two (2) uni-strut channels, one (1) per side, installed in the compartment.

ADJUSTABLE SHELF(S)

There shall be One (1) adjustable shelf constructed of 0.125” aluminum with a 2.00” upward bend at front and rear, and side supports.

COMPARTMENT LIGHT

One (1) surface mount LED light shall be installed in the compartment.  The light shall have a clear lens and shall a minimum effective output of 810 lumens.

The lights shall function independently of other compartments by an automatic "On-Off" switch located on each compartment door.

RIGHT SIDE COMPARTMENTS

RIGHT SIDE COMPARTMENT IN FRONT OF REAR WHEELS, R1

There shall be one (1) full height compartment ahead of the rear wheels, approximately 29.25" wide x 35.25" high x 14.00" deep.

One (1) louver vent shall be installed in the compartment.

ADJUSTABLE SHELVING TRACKS

There shall be two (2) uni-strut channels, one (1) per side, installed in the compartment.

ADJUSTABLE SHELF(S)

There shall be One (1) adjustable shelf constructed of 0.125” aluminum with a 2.00” upward bend at front and rear, and side supports.

COMPARTMENT LIGHT

One (1) surface mount LED light shall be installed in the compartment.  The light shall have a clear lens and shall a minimum effective output of 810 lumens.

The lights shall function independently of other compartments by an automatic "On-Off" switch located on each compartment door.

RIGHT SIDE ABOVE WHEEL COMPARTMENT, R2

There shall be one (1) standard height compartment above of the rear wheels, approximately 45.50" wide x 14.00" high x 14.00" deep.

The compartment shall be cross vented into the full height compartments

ADJUSTABLE SHELVING TRACKS

There shall be two (2) uni-strut channels, one (1) per side, installed in the compartment.

ADJUSTABLE SHELF(S)

There shall be One (1) adjustable shelf constructed of 0.125” aluminum with a 2.00” upward bend at front and rear, and side supports.

COMPARTMENT LIGHT

One (1) surface mount LED light shall be installed in the compartment.  The light shall have a clear lens and shall a minimum effective output of 810 lumens.

The lights shall function independently of other compartments by an automatic "On-Off" switch located on each compartment door.

RIGHT SIDE COMPARTMENT BEHIND REAR WHEELS, R3

There shall be one (1) full height compartment behind of the rear wheels, approximately 20.75" wide x 35.25" high x 14.00" deep.

One (1) louver vent shall be installed in the compartment.

ADJUSTABLE SHELVING TRACKS

There shall be two (2) uni-strut channels, one (1) per side, installed in the compartment.

ADJUSTABLE SHELF(S)

There shall be One (1) adjustable shelf constructed of 0.125” aluminum with a 2.00” upward bend at front and rear, and side supports.

HYDRAULIC HOSE REEL -  ELECTRIC REWIND 
One (1) Hannay series #EF2016-17-18 electric rewind dual hydraulic hose reel, shall be provided on the apparatus. 
All hydraulic reels shall be installed to be easily accessible for removal, hose access, maintenance, and servicing. 
The power rewind hose reel spool area shall be visible to the operator during the rewind operation, or the reel spool shall be encapsulated to prevent cable from spooling off the reel. 
Power rewind type reels shall have the control in a position where the operator can safely observe the rewinding operation. 
The rewind control shall be a Hannay #900-30 push sealed button with operational label next to button. 
 MOUNTING 
The reel shall be mounted in a customer specified compartment.  R-3 compartment
HOSE ROLLER ASSEMBLY ON REEL 
A Hannay model #C2 captive type roller assembly shall be installed on the reel assembly. The roller shall be mounted to permit the hose to feed directly off the reel and away from the compartment. 
DUAL HYDRAULIC HOSE - TNT
100 feet of twin TNT hydraulic hose with 6' leads shall be supplied. 

HOSE STOP

One (1) ball stop shall be attached to the electric cable to prevent total re-wind and to allow the hose to remain at a reachable position. The ball shall positively attach to the cable and be bright orange in color for high visibility.

COMPARTMENT LIGHT

One (1) surface mount LED light shall be installed in the compartment.  The light shall have a clear lens and shall a minimum effective output of 810 lumens.

The lights shall function independently of other compartments by an automatic "On-Off" switch located on each compartment door.

REAR BODY CONFIGURATION, OPEN

REAR CENTER CARGO COMPARTMENT, CR1
There shall be one (1) cargo compartment at the rear, approximately 50.00” wide x 102.00” deep.

The rear center compartment shall start at the top of the frame and be as high as the body permits.  

(Dimensions are generic and subject to change during the actual design process) 
The cargo area of the apparatus body shall be left uncovered for the installation of a skid unit.

FRONT BODY PROTECTION PANELS

Aluminum tread plate overlays and panels shall be installed on the front of the body compartment from the lower edge to the top of the compartment doors.

WHEEL WELL PROTECTION PANELS

Aluminum tread plate overlays and panels shall be installed on the side fender panels, above the rear wheels.

REAR LOWER BODY PROTECTION PANEL

Aluminum tread plate overlays and panels shall be installed the full width of the body, in the area below rear compartment and above the tailboard.

FUEL FILL W/ ACCESS DOOR

One (1) Cast Products aluminum fuel fill with a hinged access door shall be installed in the left hand side wheel well rear of the axle.  It shall be labeled “Ultra Low Sulfur Diesel Fuel Only”.

REAR TAILBOARD

A rear tailboard shall be provided and installed at the rear of the body.  The rear tailboard shall be a minimum of 10.00" deep and constructed of aluminum Grip Strut. The outside edges of the rear tailboard shall be trimmed with bright diamond plate aluminum.

The tailboard shall meet the NFPA 1901/1906 recommended requirements for non-slip surfaces.

The rear tailboard shall be the full width of the body.

EXTERIOR GRAB RAILS
Each grab rail shall be non-slip, 1.25” diameter extruded polished aluminum grab rails with rubber inserts designed to provide maximum gripping ability, strength, and durability.  The rails shall comply with NFPA 1901.

GRAB RAILS, REAR, VERTICAL
There shall be two (2) grab rails, located at the rear of the body.  One (1) shall be located on the left hand side and one (1) shall be located on the right hand side.
12 VOLT ELECTRICAL SYSTEM

The truck shall have a 12-Volt electrical system.  All wiring will be run in convoluted high temperature plastic loom.  The wiring shall be color-coded, numbered, and function imprinted for permanent identification.  All wiring devices shall be rated to carry 125% of the maximum ampere load for which the circuit is protected.  All added electrical equipment shall be served by circuits separate and distinct from the chassis circuits.  All solenoids, relays, and terminal blocks will be located in an easily accessible area.  All circuits provided shall have properly rated low voltage over current protective devices.  All electrical will be accordance with modern automotive wiring standards.  All under side terminal junctions shall be fully enclosed in sealed plastic weather proof boxes.

ELECTRICAL SYSTEM (CHASSIS OEM)
The commercial chassis electrical system shall be furnished and installed by the chassis manufacturer and shall not be altered in any way so as to void or diminish the manufacturer’s warranty responsibilities. Body builder wiring interface harnesses shall be specific to the chassis being utilized and the apparatus specifications with all such harnesses, circuits and connections being documented by the body builder and made part of the electrical schematics provided with the completed apparatus. 

ELECTROMAGNETIC INTERFERENCE PROTECTION
The apparatus shall incorporate modern electrical system design, installation procedures, grounding techniques and wave generating components to provide the highest level of protection against electromagnetic (EMI) and radio wave frequency (RFI) interferences. 

The apparatus shall be designed to operate and correctly function in congested municipal environments as well as industrial or concentrated commercial scenes without adverse effects from either EMI or RFI. Communications equipment installed after the apparatus is delivered shall be immediately tested by the installer for reception and transmission signal quality.

NFPA 1906 CERTIFIED 12 VOLT ELECTRICAL SYSTEM 

The 12-volt apparatus body electrical system shall be provided and shall be in compliance with NFPA 1906 testing and certification procedures as follows:

NFPA MINIMUM ELECTRICAL LOAD DEFINITION

The NFPA 1906 defined minimum electrical load shall consist of the total amperage required to simultaneously operate the following in a stationary mode:

1.
Propulsion engine and transmission.

2.
The clearance and marker lights.

3.
Communication equipment.  5 amp default.

4.
Illumination of all walking surfaces, the ground at all egress points, control and instrumentation panels and 50% of total compartment lighting.
5.
Minimum warning lights required for "blocking right of way" mode.
6.
The current to simultaneously operate and fire pump and all specified electrical devices.
7.
Anything defined by the purchaser, in the advertised specifications, to be critical to the mission of the apparatus.

RESERVE CAPACITY TEST

The first electrical test to be performed will be the Reserve Capacity Test.  All items listed in NFPA Minimum Load Definition shall be activated with the engine shut off.  After 10 minutes of operation, the items 1-7 shall be deactivated.  After deactivation, the battery system shall have ample reserve to start the engine.

ALTERNATOR PERFORMANCE TEST AT IDLE

The second electrical test to be performed shall be Alternator Performance Test at Full Load.  All electrical loads shall be activated with the engine running up to the governed rpm for two hours.  During the test, the system voltage shall not drop below 11.7 volts or have excessive battery discharge for more than 120 seconds.  Any loads not defined in the NFPA Minimum Electrical Load may be load managed to pass test.

TEST CONDITIONS

All electrical testing shall be performed with the engine compartment at approximately 200 degrees.

LOW VOLTAGE ALARM

One (1) Kussmaul 091-85-12 Low Voltage alarm system shall be supplied and installed with a Cole Hersee 4112-RC light/buzzer located in the cab.  The system shall detect when the battery voltage drops below 11.8 volts and remains low for 120 seconds.

CHASSIS GROUND LIGHTS

LED ground lights with outward facing angle brackets shall be installed, one (1) under each chassis door.

FRONT OF BODY GROUND LIGHTS

Two (2) LED ground lights with outward facing angle brackets shall be installed under the front of the body.  One (1) light shall be located on the left hand side and one (1) light located on the right hand side of the apparatus.

REAR STEP GROUND LIGHTS

Two (2) LED ground lights with outward facing angle brackets shall be installed under the rear step of the apparatus, one (1) each side.

GROUND LIGHT SWITCHING
The cab and body ground lights shall activate by engaging the parking brake.
HAZARD LIGHT

A Whelen model 0SR00FCR flashing red LED light, located in the driving compartment, shall be illuminated automatically whenever any compartment door is ajar.

The hazard light shall be marked with a sign that reads “Do Not Move Apparatus When Light is On”.

The warning light shall be interlocked to the parking brake and shall only alert the driver when the parking brake is released.  The light shall also be used to signal that other ancillary equipment such as racks light towers etc. are not in their “ready for transport” position.

LICENSE PLATE LIGHT

A license plate bracket with LED light shall be provided and installed on the rear of the body.  It shall be wired to come on with the headlights.

BRAKE, TAIL, TURN LIGHTS

One (1) pair of flush-mount round 4.00” red LED combination brake/ tail/ turn lights shall be provided at the rear of the body, one (1) each side, outboard of the reverse lights.  The lights shall be attached with a rubber grommet.  

BACK UP LIGHTS 

One (1) pair of flush-mount round 4.00” clear LED back up lights shall be provided at the rear of the body, one (1) each side, inboard of the B/T/T lights. The lights shall be attached with a rubber grommet.

MARKER LIGHTS

LED marker lights shall be installed on the vehicle in conformance to the Department of Transportation requirements.  The side and rear of the body will be provided with reflectors.  All marker lights shall be incorporated into the headlight circuit of the cab/chassis

LED TELESCOPIC BOTTOM RAISE FLOODLIGHTS 
Two (2) Akron Scene Star LED model ELSS-SLDC telescopic lights shall be installed.  The light pole shall be anodized aluminum and have a knurled twist lock mechanism to secure the extension pole in position.  The extension pole shall rotate 360 degrees.  The outer pole shall be a grooved aluminum extrusion and qualify as an NFPA compliant handrail.  The pole mounting brackets shall have a 4" offset. Wiring shall extend from the pole bottom with a 4' retractile cord.

The lamp head shall have eight (8) ultra-bright white LEDs.  It shall operate at 12 volts DC, draw 13 amps, and generate 14,000 lumens.  The lamp head shall direct 50 percent of the light onto the action area while providing 50 percent to illuminate the working area.  The light head shall tilt up and down with two heavy duty handles and shall be mounted with a swivel assembly.  The lamp head shall incorporate heat-dissipating fins and be no more than 5.6" deep by 4 1/4" high by 14" wide.  The lamp head and mounting arm shall be powder coated white. The floodlight shall be for fire service use.
An on/off switch with weatherproof boot shall be provided on the light head.

Choose mounting location for pole lights                                        

NFPA AUDIBLE AND LIGHTING WARNING PACKAGE 
The following warning light package shall include all of the minimum warning light and actuation requirements for the current revision of the NFPA 1901/ 1906.  The lighting as specified shall meet the requirements for both "Clearing Right of Way" and "Blocking Right of Way" which includes disabling all white warning lights when the apparatus is in "Blocking Right of Way" mode.
WARNING LIGHT FLASH PATTERN
All of the perimeter warning lights shall be set to the default NFPA flash pattern as provided by the warning light manufacturer.

WARNING PACKAGE ACTUATION CONTROLS
The warning light package shall be actuated with two (2) warning light switches located on the cab switch panel.  One (1) switch for the lightbar and one (1) switch for the warning lights.
The wiring for the warning light package shall engage all of the lights required for "Clearing Right of Way" mode when the vehicle parking brake is not engaged.  An automatic control system shall be provided to switch the warning lights to the "Blocking Right of Way" mode when the vehicle parking brake is engaged.

LIGHTBAR

One (1) WHELEN model JE2NFPA 56.00” LED lightbar shall be supplied and mounted.  The lightbar shall have clear lenses and contain the following modules:

Four (4) RED LIN6 LED modules, two (2) on each corner.


Four (4) RED CON3 LED modules, across the front


Two (2) WHITE CON3 LED modules, on the front

The forward facing white lights shall be automatically disabled for the "Blocking Right of Way" mode.
LIGHT BAR ACTUATION

The light bar shall be controlled by a switch in the cab.

LOWER FRONT WARNING LIGHTS

One (1) pair of Whelen model 60R02FRR Super LED warning lights shall be installed, one (1) each side on the front of the chassis cab.

The lights shall be red in color with red lens.

The light(s) shall be supplied and installed with a chrome bezel.

INTERSECTION WARNING LIGHTS

One (1) pair of Whelen model LINZ6 LED warning lights shall be installed, one (1) each side of the chassis cab front fenders.  

The lights shall be red in color with clear lens.

There shall be black bezels supplied and installed on the lights.

LOWER MID-BODY WARNING LIGHTS
One (1) pair of Whelen model 60R02FRR Super LED warning lights shall be installed , one (1) each side of the apparatus, mid-body.

The lights shall be red in color with red lens.

The light(s) shall be supplied and installed with a chrome bezel.

LOWER REAR WARNING LIGHTS

One (1) pair of  Whelen model 60R02FRR Super LED warning lights shall be installed, one (1) each side on the lower rear of the apparatus body.

The lights shall be red in color with red lens.
The light(s) shall be supplied and installed with a chrome bezel.

LOWER WARNING LIGHT ACTIVATION

The lights shall be controlled by a switch in the cab. 

ELECTRIC SIREN AND CONTROL

One (1) Whelen model #295SLSA1 electronic siren shall be mounted in the cab.  This unit shall feature an electronic air horn, wail, yelp, hi-lo and shall have a hard wired PA microphone.

ELECTRONIC SIREN SPEAKER

One (1) Whelen model SA315FIP 100 watt speaker shall be provided.  The speaker shall have a nylon composite black housing with front loaded, powder coated speaker driver.  The speaker shall produce a minimum sound output of 120 dB at 10 feet to meet current NFPA 1901/1906 requirements.  The speaker shall be wired to the electric siren located in the cab.
The speaker shall be located on the right hand side of the bumper.
PAINT, STRIPING, AND LETTERING SECTION
BODY PAINT PROCESS
All bright metal fittings, if unavailable in stainless steel shall be heavily chrome plated.  Iron fittings shall be copper plated prior to chrome plating.  
All seams shall be caulked both inside and along the exterior edges with a urethane automotive sealant to prevent moisture from entering between any body panel.
The body and all parts shall be thoroughly washed with a grease cutting solvent (PPG DX330) prior to any sanding.  After the body has been sanded and the weld marks and minor imperfections are filled and sanded, the body shall be washed again with (PPG DX330) to remove any contaminants on the surface.
The first coating to be applied is a pre-treat self etching primer (PPG DX1787) (.5 to 1.0 dry film build) for maximum adhesion to the body material.  The next two to four coats (depending on need) shall be an acrylic urethane primer surfacer (PPG K36).  The film build shall be 4-6 mils when dry.  The primer surfacer coat, after appropriate dry time, shall be sanded with 320-600 grit sandpaper to ensure maximum gloss of the paint.  The last step is the application of at least three coats of PPG Delfeet polyurethane two-component color (single stage).  The film build being 2-3 mils dry.  The single stage poluurethane, when mixed F3270 catalyst (PPG F3260) shall provide a UV barrier to prevent fading and chalking.
All products and technicians are certified by PPG every two (2) years.

CHASSIS PAINT

The chassis shall be painted by the OEM Chassis Manufacturer.

IMMERSION PRIMING

Before painting the body shall go through a computer controlled 12 stage electro-cathodic immersion prime paint system.

There shall be a 5-stage pre-treat system to ensure consistent primer adhesion.  In this process, the body is treated with phosphate to ensure the primer adheres to the metal.

The body shall then be plunged into a vat of high voltage, electrically charged primer, that shall fuse the paint to the aluminum.  This process ensures that all surfaces, even the most hidden, receive a uniform, smooth, consistent layer of protection, NO EXCEPTION TO THIS REQUIREMENT SHALL BE ALLOWED.

PAINT FINISH

The body shall be painted with a PPG Delfleet Evolution Paint System.

As part of the curing process the painted body shall go through a baking process.  The painted components shall be baked at 185 degrees for 3 hours to achieve a complete coating cure on the finished product.
After bake and ample cool down time, the coated surface shall be sanded using 3M 1000, 1200, and or 1500 grit sandpaper to remove surface defects.  In the final step,  the surface shall be buffed with 3M Super-duty compound to add extra shine to coated surface.  No more than .5 mil shall be removed in this process.

All products and technicians shall be certified by PPG every two (2) years.

UNDERCOATING

The entire understructure of the apparatus body shall be undercoated using Monsey 2.7 Low VOC Solvent Base Sound Sealer.

SIDE COMPARTMENT FINISH, ZOLATONE

The apparatus side compartment interiors are to be coated with Zolatone, a polychromatic, modified nitrocellulose coating with a flat background color with accenting fleck colors.  The compartments shall be cleaned with a grease remover, and then the surface sanded and prepared for painting.  The Zolatone finish is washed and waxed like paint, and is resistant to man solvents and wear.

PAINT COLOR

The apparatus body paint shall be "cross referenced" from the chassis paint, and shall be painted to match the main chassis color as close as possible.

REFLECTIVE STRIPING

Reflective striping shall be applied to the perimeter of the truck.  Size and design shall be determined by the department.

CHEVRON STRIPING 

At least 50% of the rear of the unit shall be covered with Red and Yellow alternating 6" stripe in an inverted Chevron pattern.

LETTERING

Reflective lettering shall be applied to the cab doors at the direction of the purchaser.

Photos or drawings of the lettering and striping layout shall be provided by the purchaser prior to construction.


