Summerville Fire – Rescue

Station 3

1701 Old Trolley Road

Summerville SC

Non-Binding Proposed

Project Specifications for

Contractor Proposal

Assistance only

Summerville Fire-Rescue – Station 3

Demolition and New Construction

1701 Old Trolley Road, Summerville, SC

INDEX

Division 1 	General Requirements, Allowances

Division 2 	Site Construction

Division 3 	Concrete

Division 4 	Masonry

Division 5 	Metals

Division 6 	Woods and Plastics

Division 7 	Thermal and Moisture Protection

Division 8 	Doors and Windows

Division 9 	Finishes

Division 10 	Specialties

Division 11 	Equipment

Division 12 	Furnishings

Division 13 	Special Construction (Pre-Engineered Building and Fire Suppression System)

Division 14 	Conveying Systems

Division 15 	Mechanical

Division 16 	Electrical

OTHER ITEMS INCLUDED WITH THIS INFORMATION PACKAGE

- DRAWINGS OF FLOOR PLAN AND ELEVATION

- CIVIL DRAWINGS

- ASBESTOS AND HAZMAT TESTING REPORT

- PROPOSAL FORMAT SHEET FOR CONTRACTOR’S USE

- PROPOSAL INFORMATION SHEET

Division 1 General Requirements –

  - Project Identification:  The project name is:  Summerville Fire & Rescue-Station 3, Demolition and New Construction of an Essential Building.  The Owner of the property is:  The Town of Summerville. The Owner Representative is:  Assistant Chief J.D. Allston and can be reached at:

Assistant Chief J.D. Allston

300 West 2nd North Street

Summerville, SC  29483

Work phone:  843-873-5107

Cell phone:  843-514-9413

Email:  jallston@summervillesc.gov

  - Project Drawings for Proposal Purposes: Civil Drawings, Building Floor Plan and Elevations, are provided with these documents to assist the proposal Contractor.  All proposals must be based on the design, size, and function as shown on the drawings.

  - Project Specifications for Proposal Purposes:  All items contained in these specifications are for assisting the preparation of a complete set of specifications for approval by the Owner. These specifications are not to be considered as complete and should not be considered permanent specifications for use in construction of the project. They should be used only as a tool to give the proposal Contractor an idea of what the Owner desires within this project. Each Contractor shall provide the Owner with additional and as much detail of specification items with their proposal.

  - Project Drawings and Specifications - Final:  Civil Drawings will be the responsibility of the Owner.  Landscaping drawings will be the responsibility of the Town of Summerville. The Contractor is responsible for Architectural, Structural, (foundation and pre-engineered building), Mechanical, Electrical, and Plumbing Drawings, all signed and sealed by a licensed professional. All Specifications, Drawings, Submittals, and Construction will meet all Federal, State and Local Codes and will be approved and inspected by the Town of Summerville Building Official and Fire Marshal, and any other required entities, for an Essential Building. The final specifications shall be a completed set agreed upon by the Owner and Contractor, and shall assist with any clarifications of the drawings. Provide for costs of drawings and specifications with proposal.

  - Project As-Built Drawings:  Drawing As-Builts will be the responsibility of the Contractor and provided to the Owner at the end of the project.  Drainage As-Builts will be the responsibility of the Owner’s Civil Engineer.

  - Project Close-Out:  Contractor shall provide to the Owner all resources, instruction manuals, warranties, etc. for their portion of the project responsibility.

  - Proposal Submission:  All proposals shall include as much detail from the Proposal Contractor to aid the Owner in understanding and deciphering their submission.  Where and when applicable, provide the Owner with the manufacturer, style, size and or other descriptions of materials proposed. When applicable, drawings and specification documents shall be provided to show electrical provisions, mechanical provisions, and plumbing. The Contractor shall make the Owner aware of any other items necessary to complete this project prior to figuring costs of the construction, and shall provide this information to the Owner with their proposal submission.

  - Pre-Bid Site Meeting with Question and Answer Period:  A mandatory pre-bid site meeting will be planned for and held by the Owner. See the Proposal Package Information for further detail.

  - Questions and Clarifications During Proposal Process:  All questions and requests for clarification or information during the proposal process may be asked during the pre-bid site meeting or submitted to the following person via email only:  Assistant Chief J.D. Allston (see email address in section 1.1).  See the Proposal Package Information for further detail.

  - Commercial Review Board of Summerville:  The Contractor will be responsible to prepare for, submit, and present the Project Drawings to the Summerville Commercial Review Board for all approval processes, and within this proposal value.  If any changes are deemed necessary which includes further review meetings or approvals, this should also be included as part of this proposal.  Assume that the proposal project drawings are accepted and approved by the Board for construction pricing purposes.

  - Zoning: The property zoning is classified as Public Lands and is approved for construction for the proposed occupancy – Fire Station.

  - Construction Permits: The Owner’s civil engineer shall submit required site approvals to the Town and any other required entities. The Contractor shall submit drawings, obtain and pay for all business license and ensure that all subcontractors do the same prior to starting work on the project. Permits will be required at no cost to the Contractor.

  - Impact Fees: The Town will not assess any impact fees for the project. Summerville CPW will require tap fees for the installation of the fire sprinkler system to be installed in the building. Their cost of the tap and the backflow preventer will be the responsibility of the Owner through a project allowance. There should be no tap fees due for domestic water or for the sewer unless upgrades are required by Summerville CPW. If the proposal Civil Drawings show that an upgrade of the existing sewer service or a larger domestic water line and meter be required, it will be installed by the Contractor and will be the responsibility of the Owner to pay the impact fees through an allowance. See Civil Drawings.

  - Insurance: Contractor shall pay for, and maintain for the duration of the project: general liability insurance in the amount of $2,000,000.00, automotive liability in the amount of $1,000,000.00, workman’s compensation in the amount of $500,000.00. Builders Risk Insurance for the duration of Contractors work, figured for the contract amount is to be included. Include all insurance costs with proposal.

  - Performance and Payment Bond:  The selected Contractor at time of contract acceptance, shall have a performance bond and payment bond, in an amount equal to 100% of the contract price. The Contractor shall be licensed and able to be bonded to perform all work under one (1) contract. A split contract will not be accepted, making all work performed under a single bonded contract. All costs for a performance and payment bond should be included with the base proposal price, and note the cost of the payment and performance separately.

  - Project Bid Bond:  Contractor should include a bid bond with proposal submission.

  - Project Contract:  Contract between Owner and selected Contractor will be an AIA format.

  - Contractor License:  All Contractors submitting a proposal for this project shall be properly licensed for this work at the time of the submission. Submit copies and or license numbers for all licensing that would apply to perform this work, with the proposal submission. Provide all numbers, classifications, and expiration dates.

  - Inspections:  All third-party inspections shall be performed by the Town of Summerville, or a third party independent company at the Owners request and paid for by the Owner through the allowance. See third-party allowance information in Division 1 for more detail on Contractors responsibility.

  - Project Phasing While Under Construction:  All construction work and scheduling by the Contractor shall be made in consideration to the Fire Department personnel, fire trucks, and other equipment that will remain active and on site for the duration of time.  Personnel will be set up in trailers provided for by Owner.  Contractor to consider their scheduling and work around an active fire department’s daily routine and work 24/7.  Driveways and parking areas for fire trucks and other vehicles will remain in operation during all construction phases.

  - Project Clean Up: The Contractor shall maintain a clean work site and remove all unwanted debris daily. Prior to the completion of the project, the Contractor shall remove all waste materials and debris arising from the performance of the work, and shall present all construction areas in a broom clean condition. Contractor will also provide a final cleanup of the building to include vacuuming, cleaning windows, mopping floors prior to turning over the building to the Owner. NO CONSTRUCTION DEBRIS SHALL BE BURNED OR BURIED ON SITE INCLUDING BACKFILLING OF DITCHES OR AROUND EXTERIOR FOUNDATION BACKFILL.

  - Payment:  Invoices are to be submitted monthly for work performed and material stored on the construction site in the preceding month. Payment will be reviewed and based on percentage of work values completed and invoiced stored materials will either be approved for payment or denied and returned to the Contractor for correction of items deemed questionable within 5 days of original submittal. Payment shall be made to the Contractor no more than 30 days after the approved submitted pay voucher is received. There will be a 10% retainage deducted from all payments which will be reduced to 5% at time of Substantial Completion. Substantial completion shall be determined and agreed upon by the Owner and the Contractor once the building is approved by the Building Official for the Town of Summerville. The remaining 5% will be paid to the Contractor upon completion of the project including punch list items, warranty information, as built plans, and close out documents. These items should be included in the contract between Owner and Contractor.

  - Materials Stored on Site:  Materials stored on site remain the responsibility of the Contractor. Replacement of stored materials due to damage or theft remains the responsibility of the Contractor.

  - Changes (Change Orders): All work shall be completed in a workmanlike manner within standard practices. Any alterations or deviations from the specifications or contract involving extra cost or credits will be executed as mutually agreed upon and be verified by a written change order signed by the Contractor and the Owner. No change orders will be allowed until discussed with the Owner, a negotiated price is agreed upon, and a signed change order is executed.  Since this is a design build project the Contractor shall endeavor to minimize any change orders by completing and submitting an all-inclusive price proposal and details of specification items with the submitted proposal.

  - Temporary Utilities/Dumpsters: The Contractor shall furnish temporary electric, needed water for construction, temporary toilet, and construction dumpster for removal of construction debris, for the duration of the job. The temporary toilet shall be serviced twice weekly.

  - Construction Signage:  The Contractor shall place signs in two separate areas showing No Trespassing, and Hard Hats Required.  Any other pertinent safety signage will also be the responsibility of the Contractor. Smoking will not be allowed within the building footprint once the exterior walls are being constructed.

  - Site Surveillance:  If the Contractor or Owner selects to have site surveillance, it would be covered at their own perspective cost.

  - Sub-Contractors, Vendors, and Other Businesses:  Contractor shall provide a listing of all subcontractors, vendors and other businesses that will be utilized on this project with their proposal.  Provide their company name, division of expertise, contact person, address and phone number.

  - Owner Allowances:  The lump sum of the total allowances described herein this document shall be considered in the proposal’s base pricing.  This allowance amount shall be used at the discretion of the Owner only and be done by the Owner’s consent.  A written direction, signed and agreed upon by the Owner and Contractor shall become record of each allowance used during the project.  All scheduling and coordination of work associated with the allowances shall be included as part of the Contractor’s base pricing, unless otherwise agreed upon by the Owner and Contractor.  All payments to third parties for the allowance work shall be paid for by the Contractor using these allowance amounts.

All excess monies not utilized during the project shall not be charged to the Owner. Unused allowance amounts will be removed from the contract cost at the completion of the project.

Schedule of Owner’s Allowances:  Include total amount in submitted base price.

      - Unsuitable Soils (muck and fill):  Removal and replacement of unsuitable soils; include the lump sum of $10,000.00 for muck and fill.  With the proposal, identify the cost of muck and fill, including grading, per CY.  All muck and fill shall be verified in the field at the time of discovery and documented before proceeding. Owner shall be notified and agree to muck and fill quantities before proceeding.

      - Landscaping, Irrigation, Irrigation Meter, Back Flow:  Include the lump sum of $10,000.00 for the installation of landscaping and or irrigation materials. Landscaping will be done by Town of Summerville. The backflow preventer and meter for the irrigation system will be the Owner’s responsibility. Backflow preventer will be installed by Contractor.

      - Communication and Data:  Include the lump sum of $15,000.00 for the purchase and installation of communication equipment and data wiring.  Conduit, hand holds and boxes shown on the drawings are not part of this allowance and will be provided by the Contractor.

      - Furniture, Fixtures, and Appliances:  Include the lump sum of $20,000.00 for the purchase and installation of furniture, fixtures and appliances.

      - General Owner’s Contingency:  Include the lump sum of $25,000.00 for discretionary use by the Owner.  This may be used for unforeseen items or necessary change orders.

      - Water and Sewer Tap Fees:  Include the sum of $15,000.00 for CPW water and sewer utility fees, water and sewer taps, meter, backflow preventers and fire sprinkler tap.

      - Chain Link Fencing:  Include the lump sum of $10,000.00 for new chain link fencing and gates.

      - Communication Pole:  Include the sum $2,500.00 for the relocation of the existing communication pole. Refer to Division 16 for responsibilities of the Contractor.

      - Vehicle Exhaust System:  Include a lump sum of $25,000.00 allowance for the removal and reinstallation of the vehicle exhaust system in the apparatus bays.  All coordination of the removal and reinstallation will be by Contractor and Owner and included in the base price proposal.  Removal will be before demolition commences.

      - Third Party Testing and Inspections:  Include a lump sum of $15,000.00 for third party testing by a third party selected by the Owner, i.e. geotechnical, Chapter 17, SWPPP, Soil Borings, Compaction Testing, Concrete Testing, Digital Cone Penetrometer, Additional Proof Rolls. Contractor shall be responsible and have in their base pricing for coordination, supervision and scheduling of the tests.

      - Self-Contained Breathing Apparatus:  Include the sum $25,000 for fill station to be supplied by Owner.  Contractor responsible for electrical costs for unit.

Division 2 Site Construction and Demolition – Refer to Civil Drawings for Site Construction

- Abatement Testing and Removal: The hazmat asbestos abatement report for the existing structures will be the responsibility of the Owner and included in proposal package for review. Any necessary removal and disposal will be the responsibility of Contractor and should be referenced and included in the proposal.

- Demolition:   All project demolition will commence once the building has been cleared of all re-usable items to include the emergency generator, switching gear, household items, vehicle exhaust system, HVAC units etc. The removal and storage of reusable items will be the responsibility of the Owner. Days needed for this will not be assessed against the completion date. All demolition and removal and disposal of demolition debris will be by the Contractor.  All items that may or may not be reusable and or salvageable is at the discretion of the Owner.

The contractor shall provide in the base price the complete demolition of the building and all other items mentioned for demolition on the civil drawings. Proper disposal of all demolition debris will be the responsibility of the Contractor. No material will be allowed to be buried or burned on site.

- Power Poles and Communication Pole (existing): Refer to Civil Drawings.  Communication pole will be relocated by Owner and paid for with an allowance. Refer to Division 16 for electrical provisions provided by the Contractor for the power and communication poles.

- Demolition Disposal:  All debris from the project shall be properly disposed of and within proper DHEC guidelines, by the Contractor. Asphalt New and Asphalt Removal:  See Civil Drawings

- Silt Fencing, Erosion Control:  See Civil Drawings

- Asphalt New and Asphalt Demolition:  Per Civil Drawings

- Soil Borings:  See third party testing allowance information in Division 1.

- Compaction Tests:  See third party testing allowance information in Division 1.

- Sub-Grade:  Site contractor to strip back up to 8” of existing material.  All areas to receive floor fill shall be accurately graded to the appropriate sub grade level and back bladed to test for any soft spots in sub-grades. Any soft spots encountered shall be brought to the attention of the Owner for a determination of any additional sub-grade work that may be required above and beyond the scope of the contract.  See Civil Drawings.

- Floor Fill: The Site Contractor will provide compacted, clean off-site suitable fill as required under the building slab to reach finished floor elevation as shown on the civil drawings.

- Back Fill:  Site Contractor will clear all debris and back fill around the building as required to prevent ponding and allow for movement of scaffolding.

- Muck and Fill Pricing:  Submitted with the proposal the Contractor shall provide a set cubic yard price for additional offsite fill dirt that may be needed for the project. If muck and fill is needed on the project, it will be the responsibility of the Owner to pay through an allowance (see schedule of allowances). All muck and fill will be discussed and agreed upon with the civil engineer and Owner before Contractor proceeds with the work.

- Parking and Driveways:  Install all parking areas, and driveways in accordance to the Civil Drawings.  Include all signage as shown on the Civil Drawings.

- Drainage and Detention:  Drainage and the need for detention will be in accordance to Civil Drawings.

- Exterior Water and Backflow Preventer: The existing water meter will remain in service for the residential section of the project and a backflow preventer (whichever is required by the sprinkler specifications) will be installed by Contractor and paid for through an allowance by the Owner. See Civil Drawings.

- Exterior Sewer:  See Civil Drawings for installation detail.

- Oil Water Separator:  The existing oil water separator will be reused for the new building purposes. Provide for any new piping and hook-up that may be needed.  See Civil Drawings.

- Natural Gas Line:  See Civil Drawings for any new gas piping.  New gas lines will connect to the emergency generator, two tank less water heaters, the 50,000 BTU heating units in the apparatus bay area, the heating units for the residential section of the building, the kitchen range, and an exterior line with shut off quick connect for a future gas grill.  Contractor will coordinate with SCE&G. See Civil Drawings for detail.  See floor plan drawing for location at rear of building.

- Dumpster Pad/Enclosure Fence:  There will be no enclosure fence or concrete pad for trash receptacles.

- Fencing (wood and metal):  Contractor will install a wooden privacy fence and gates constructed of pressure treated lumber (see Drawings for detail).  All Chain Link fence and gates will be the responsibility of the Owner and paid through an allowance included in this price proposal.

- Landscaping and Irrigation:  Any landscaping and irrigation will be the responsibility of the Owner through an allowance in this price proposal. See listing of allowances.

- Fire Hydrant:  See the Civil Drawings, existing remains. Provide for any new work.

- Termite Treatment:  The building pad and foundation will be cleared of all debris, and termite treated prior to the moisture barrier being applied. A certified letter from the licensed termite company will be furnished to the Owner.

- SWPPP Inspections:  Inspections shall be done by Owner’s civil engineer - see allowance information in Division 1.

- Digital Cone Penetrometer:  See third-party testing allowance information in Division 1.

- Proof Rolls:  Contractor shall coordinate all proof rolls that will be conducted on site with the Owner’s third party inspectors and or civil engineer.

- Geotechnical Testing:  See third party testing allowance information in Division 1.

Division 3 Concrete –

  - Footing and Foundations:  Concrete used for footing and foundations will have a 28-day minimum compressive strength of 3,000 psi. Foundations and footings will be designed in accordance with structural loads as designated by the Structural Engineer.

  - Floor Slab, with Drains and Trench: Provide a concrete slab that meets the requirements of the building.  Provide detail of the thickness, structural features, and any other pertinent detail to the Owner with the proposal submission.  Provide some detail of the apparatus bay trench drain, with a grated top cover provided. Trench drain will connect to the existing oil water separator.

The concrete floors should help water drain to a center drain trench in the apparatus bay, and to each floor drain.  See floor drain and trench locations on drawing.  Any sloping will be gradual and unnoticeable.  See Division 9 for flooring finish.

  - Concrete Ramps:  Provide and install per the Civil Drawings.

  - Concrete Entry:  Provide for front entry with footings and at least 4” slab.

  - Concrete Curb, Gutter and Sidewalks:  Provide and install per the Civil Drawings. Sidewalks should receive a light broom finish to avoid slipping. Installed as needed to comply with ADA regulations.

  - Dumpster Pad:  Refer to civil drawings for detail.

  - Control Joints:  All construction control joints will be saw cut. Contractor’s structural engineer shall determine locations and depth for the building’s slab control joints.

  - Concrete Testing:  See third party testing allowance information in Division 1.

Division 4 Masonry –

  - Brick Work Design:  Brick work will be installed as shown on the drawings; on all four sides of building. Brick ties will be attached in compliance with the International Building Code and Authority having jurisdiction.  The brick and grout selection will be the same as Fire Station # 2 on Luden Drive, and Fire Station #5 located on Butternut Road in Summerville. See drawings for soldier brick placement. All brick must be from same manufacturing run to avoid color variations.

Boral’s Full Range Wire Cut Modulars

Accent Brick: Carolina Ceramics’ Cocoa Velour Modulars

Brick Mortar: Argos’s Brown

  - Exterior Walls:  Brick work will start at the top of the building footings and terminate at the roof height on rear wall, and at the parapet height of the front and side walls. Contractor’s structural engineer shall determine locations and depth for the building’s slab control joints.

  - Weep Holes:  Weep holes will be installed every 36” and installed below the finish floor level but above surface grade. Weep holes are to be installed at window sills and at any lentils openings. The weep holes will be full brick height, mortar joint size. Mortar netting will be installed around the entire base, and at all lintels over all openings.  Include flashing, and peel and stick, at all sides of each opening. Openings include all windows, doors, and overhead doors, and other affected penetrations.

  - Mortar Joints:  All mortar joints will be uniform in size and all expansion joints will be installed at time of the work and NOT saw cut later.  Backer Rod will be installed in all expansion joints and sealed with elastomeric caulking that matches the color of the brick work. Contractor’s structural engineer shall determine locations and final type of control joints on the exterior walls.

  - Brick Work General:  All brick work will be washed and excess mortar and debris will be cleaned prior to Owner taking possession of the building.  All broken and discarded brick along with excess mixed mortar and empty mortar bags will be cleared from the project daily and placed in dumpster for proper disposal.

Division 5 Metals –

  - Exterior Metal Studs:  The perimeter curtain walls are to be framed with 6” CSJ 18 gauge metal studs at 16” on center with densglass sheathing screwed on and covered with Tyvek or equal as a moisture barrier. Refer to the Drawings for parapet curtain wall height and location details.  The exterior wall will have a Mel Roll stick and seal flashing installed completely from the bottom up 12” around the building for moisture protection. Include appropriate cold roll on all exterior walls.  The specifications for studs, bracing, strapping for all walls, clips, and anchors will be determined by structural engineer and should be provided for under this proposal. Refer to Division 7 for insulation detail.  See drawing for rated wall locations.

  - Entrance Canopy Framing:  Provide for the framing of the gabled entrance with appropriate structural components for a brick façade and roof.

  - Interior Metal Studs:  The apparatus bay interior wall separating the residential section from the apparatus bay will be 6” 18 gauge metal studs framed 16” on center, and go to roof deck.  This separation wall be at least 1hour rated wall.  Other interior walls within the residential section of the project will be framed with 3-5/8” 20 gauge metal studs, 16” on center. Ceiling height shall be 9’ AFF.  Refer to Division 7 for insulation detail. The specifications for studs, bracing, and strapping for all walls will be determined by structural engineer.

  - Additional Interior Metal Studs:  Interior wall in the men’s restroom behind the urinal and the interior kitchen wall behind the stove out to the end of the cabinets will be 6” metal studs framed 16” on center. The specifications for studs, bracing, and strapping will be determined by structural engineer.

  - Metal Cap and Liner:  Provide and install 26 ga cap flashing and 25 ga flat sheet liner panels on and behind all parapet walls.  Standard color selection.

  - Awnings:  All single metal passage doors shall have an awning, at least 4’ across.  Provide a standing seam metal panel, 40” with a downward projection of 32”.

  - Schedule 40 Steel Clad Pipe Bollard:  Although there are no plans for pipe bollards on this project, provide a price for one (1) 6”, schedule 40 steel clad pipe bollard, concrete filled, and painted safety yellow.  Any bollards that may be installed would be at the time of construction, and would be set in concrete.

  - Flag Pole:  See Division 10 for detail

Division 6 Woods and Plastics –

  - Wall Sheathing:  Contractor shall install 5/8” dens glass sheathing over the entire exterior walls of the building then covered with Tyvek or equal vapor barrier.

  - Cabinetry:  Refer to the drawings for locations, sizes, and upper and lower cabinetry areas – in kitchen areas, training room, dispatch office, and three bathrooms.   All counters will be solid surface. (No laminate will be allowed).  Cabinets will be custom built and be solid birch plywood with painted or clear lacquer covering.  No compressed particle board, or mass manufactured cabinetry shall be used.  All cabinetry equipped with heavy duty hinges, hardware, and slides. Provide at least one (1) drawer stack, and pencil drawer in front dispatch office. There are no cabinetry needs in the actual sleeping rooms.  Provide any additional detail needed with contractor’s submission to aid Owner.

  - Chair Rail:  Install minimum 5” wide chair rail throughout recreational/kitchen area and training room, at 36” AFF.  Chair rail shall be pine.  Rail will be painted.

  - Attic Access:  Attic access will be accomplished through the acoustical ceiling.

  - Wood Blocking:  Install treated wood blocking in all areas required for ADA wall mounted accessories, wall mounted water fountains, wall mounted toilet and urinals, shower heads and valves, all bathroom partitions between stalls, between urinals, and any additional places needed for support to avoid damage to sheetrock walls.  Provide blocking for three additional areas for TV mounting, etc.

  - Framing Lumber:  Provide full detail on any treated lumber proposed to be used on this project, other than for wood blocking and exterior fencing.  Provide detail of where and how wood lumber would be used.

Division 7 Thermal and Moisture Protection –

  - Insulation – Thermal Envelop of Building:  Furnish and install insulation in all required areas in accordance with current building codes, and as outlined in these specifications.  Ensure white vinyl faced in apparatus bay.  Identify type and R value of all insulated areas of the building with proposal submission.

  - Exterior and Interior Wall Insulation:  All 6” metal stud perimeter walls will receive insulation to full height of walls to roof deck. Interior walls shall receive appropriate insulation.  Building’s center separation wall will have insulation to roof desk. Identify type and R value of all insulated areas with proposal submission.

  - Roof and Ceiling Insulation:  Identify type and R value for all insulations proposed for these areas in proposal submission.

  - Back Flow Preventer Insulation:  Provide for the back-flow preventer insulated blanket to accommodate appropriate size for unit.

Division 8 Doors and Windows –

  - Doors and Windows – General Note:  Refer to drawings for locations, quantity, and type of doors.  Refer to these specifications for more detail.

  - Interior Doors:  All interior doors will be 3070 flush panel, 1-7/8” solid core birch veneer doors, with hollow metal knock-down frames.  Doors shall be finished onsite, with a clear coat finish (two coats with sanding in between coats).  Frames shall be painted.  All hardware shall be commercial, heavy duty grade.  All doors shall have adjustable closures. Push and pull plates will be installed on doors not required to have passage or privacy hardware.   At least three of the interior doors shall be fire rated – ensure proper fire rating areas.

  - Closet Doors:  Provide for closet doors in training room area to match other wood doors and frames.

  - Interior Door Glazing:  Provide and install three (3) 10”x10” view panels of tempered glass.  Provide and install six (6) 3”x33” side view panels in interior doors (at least three (3) need to be fire rated. Ensure proper fire rating for glazing.

  - Exterior Doors:  Provide and install 3070 insulated metal doors, and hollow metal frames.  Doors and frames shall be primed and painted. Provide commercial, heavy duty grade hardware, adjustable closures and locks for usage.

  - Exterior Door Glazing:  Provide and install three (3) 4”x33” side view panels for exterior doors. Impact rated. Tinted low E insulated glass.

  - Kick Plates: Provide and install at least 12” high stainless steel kick plates on the directional push side of all wood and metal doors to match the hardware.

  - Storefront Door:  Provide and install an impact rated, 3070 full vision storefront door with side lite.  Provide with commercial, heavy duty grade hardware, panic bar, and adjustable closure. Dark Bronze aluminum frame, tinted low E insulated glass. Provide for any access windows that may be required per code – specify on proposal submission.

  - Storefront Windows:  Provide and install all storefront windows and glazing, impact rated, tinted low E insulated. Note that one window in the front of the building will need ‘spandrel’ opaque accommodations from the exterior side.  The interior side of the window will be covered over as a solid wall. Note that two storefront windows will be used as emergency egress.  (See drawings)

  - Apparatus Bay Doors:  Provide and install two (2) 14’ x 16’ and two (2) 14’ x 14’ impact, required wind rated for Risk Category IV, and prefinished aluminum overhead sectional doors.  Doors and window panes must be impact rated.  All doors shall be electrically controlled.  All doors shall have manual operation capability in case of power failure. Doors shall meet all current local and state wind requirements for an essential building.  Doors shall have all proper safety features and sensors.  Motor operator for each door will also come with one remote control per door and a photo safety sensor to prevent operation of door when an obstruction is present.  Ensure sensor is accommodating for oversized trucks and positioned correctly on wall, approximately 24”-36” AFF. Each door shall have at least one section row of vision panels – impact rated. Doors shall not interfere with the heaters or exhaust system.

  - ADA Features:  All doors and hardware shall be ADA compliant.

Division 9 Finishes –

  - Acoustical Ceiling:   All interior ceilings, except Apparatus Bay – Shall be 9’ AFF.  Furnish and install 2’ X 2’ acoustical ceiling system, standard white grid and standard white tile, meeting all seismic requirements. Ceiling Contractor to install support wires for the light fixtures.  Provide moisture resistant ceiling tiles in the officer/men/women’s bathrooms.  (Two support wires per fluorescent fixture at opposite corners.  Canned light fixtures shall be braced between ceiling grids.)  Installment and materials of all ceiling components shall be to current codes and regulations.

  - Open Deck Ceiling:  Apparatus Bay will have an open exposed deck.  Paint decking and components with two coats white dryfall paint.  Insulation should be white vinyl faced.

  - Ceramic Tile Flooring:  Officer/men/women’s complete restroom areas shall have ceramic tile flooring, with ceramic tile base.  Floor tile shall be 12”x 12” porcelain floor tile, with polymer modified grout.  All final colors and selections to be made by the Owner. Provide type of ceramic flooring proposed with this submission.

  - Ceramic Tile Walls:  Officer/men/women’s complete restroom walls shall have 4” x 4” wall tile, from floor to 6” AFF.  Moisture resistant drywall shall be installed at all wall tile in all areas. Thin set method with polymer modified grout. Provide type of ceramic tile proposed with this submission.

  - Rubber Floor Covering:  The weight room floor will be covered with rubber tile flooring with sufficient thickness to avoid injury to persons and to handle free style lifting weights from being damaged due to being dropped. The weight room floor shall be by Nora; Norament Satura rubber tile.  Base shall be 4” vinyl base by Johnsonite. Color shall be selected by Owner. Specify exact rubber flooring in proposal if different than specified rubber tile.

  - Apparatus Bay Concrete Surface:  Apparatus Bay remains raw concrete finish, with a hard trowel finish.  Surface shall be cleaned and clear of any debris.

  - Other Flooring (exposed concrete):  All other interior areas shall have an exposed finished concrete with smooth, hard trowel finish. Concrete surface shall be cleaned and prepped for finish.  Stain color shall be applied, and followed by an Endurable clear sealer or approved equal.  All interior concrete needs to be a consistent color throughout facility.  Stain color to be selected by Owner, and stain manufacturer shall have multiple options available to select from.  All clear sealer should be non-yellowing in nature.  Provide detail with proposal submission.

  - Vinyl Base:  Provide a vinyl base in all floor areas with concrete or rubber flooring.  Apparatus Bay shall have a 6” vinyl base.  All other areas shall have 4” vinyl base.  Color to be selected by Owner.

  - Abuse Resistant and Fire Rated Drywall:  Apparatus bay walls will have 5/8” abuse resistant drywall from slab to 8’ AFF on the three exterior wall sides, with regular 5/8” drywall from 8’ AFF to roof deck.  The residential separation wall should be 5/8” fire rated drywall on all sides to the roof deck. The apparatus bay side of the separation wall should still have abuse resistant drywall from slab to 8’ AFF.  Whether this is accomplished with abuse resistant fire rated drywall or an additional layer of abuse resistant drywall.  See provided drawing for fire rated wall locations.

  - Moisture Resistant Drywall:  Bathroom walls shall be moisture resistant 5/8” drywall from floor to at least 3” above suspended ceiling, and appropriate type for installation of ceramic tile.

  - Drywall:  The remaining walls throughout the building will be 5/8” drywall from the finish floor level to above suspended ceiling height.

  - Interior Painting:  All walls shall be primed and receive two coats of finish egg shell paint.  See note below regarding epoxy paint.  All wood doors shall be finished with two coats clear coat on site, sanding between coats.  All exterior metal doors shall be primed and painted two coats.  All frames shall be primed and receive two coats of finish paint.  Chair rail shall be stained or primed and painted on site.  All paints shall be by Sherwin Williams, and colors selected by Owner.  All surfaces shall be sanded between coats.  Allow for at least two colors on walls, one color on door frames, one color for metal doors.

Epoxy Paint:  All four bathrooms, and Apparatus Bay walls shall be primed, and painted two coats of finish epoxy paint.

Division 10 Specialties –

  - Restroom Accessories:  Furnish and install one set of three ADA grab bars in each designated ADA commode, and ADA shower areas. Furnish and install surface mounted double roll toilet paper holders at each commode, 24” x 36” mirror, three (3) larger mirrors – each to cover sink counters’ width.

  - Bathroom Partitions:  Provide and install floor mounted, and headrail braced, partitions in the bathrooms. Stainless steel, heavy duty hardware. Standard color; color to be selected by Owner.  Specify type of partitions in proposed submission.

  - Fire Extinguishers and Cabinets: Provide and install the quantity and type/size of fire extinguishers for the building per current fire code and determined by the Town of Summerville. Specify in proposal types and cabinetry quantity for extinguishers.

  - Specialty Signage:  Provide and install black metal lettering for the fire station exterior (see elevation drawing for correct verbiage).  Specify the size (at least 12” hgt), type and lettering in proposal. Provide for custom printed signs for all rooms, and to be installed at each room door(s).  Signs shall have braille feature. Final color and font selection by Owner.

  - Other Signage:  Provide for ADA required signage for parking lot.  Provide all required exit and emergency signage, fire extinguisher signage, no parking signage at two locations.  The fire department stand pipe connection on the exterior of the building will have a permanent hard plastic sign with FDC permanently mounted to the brick. Owner should be made aware of any additional necessary signage prior to signing of contract. Provide all signage as shown on the civil drawings.

  - Striping:  Provide for parking lot striping as required in parking lot, per Civil Drawings.  Provide for ADA parking striping. See Civil Drawings for material detail.

  - Metal Personnel Lockers:  Provide metal lockers for all fire personnel, as per provided drawings.  Lockers shall be heavy duty, all-welded, single tier, non-vented, wardrobe lockers.  Contractor shall provide construction details, material description, dimensions and installation to the Owner for approval and with this proposal.  A minimum of six (6) for the Officers’ Quarters, twenty-two (22) in the men’s quarters/weight room area, and three (3) in the women’s quarters.  Standard color to select from.

  - Shower Curtain:  Provide one set of heavy duty hooks, one commercial strength curtain rod, and a shower curtain with a separate liner, for each shower enclosure.  To be approved by Owner.

  - Racks and Hooks:  Provide and install towel racks and robe hooks adjacent to each shower opening. Provide a combination holder, at least 18” long, for mops, brooms and accessories in utility room near utility sink.

  - Wall and Door Protection:  External corner guards 4’ AFF are to be installed on all exposed drywall corners to avoid damage; at least 2” wide each side.  Kick plates (at least 12” high)  will be installed on all directional sides of all solid doors to match the hardware. Push and pull plates will be installed on doors not required to have passage or privacy hardware.

  - Window Blinds: Provide and install no less than 2” wide, horizontal louvered polymer blinds on all windows, and the front storefront door and sidelite.

  - Flagpole:  Furnish and install a 40’ exposed height flagpole in the front of the building. The pole will have an external lanyard and capable of flying two flags. The flagpole will be one piece aluminum 3/16” thick, cone shaped and have an aluminum ball on top to match the pole. There will be a concrete walkway out toward the pole location that ties into a 72” diameter circular walk completely around the base of the pole. Overall 44’, 40’ exposed x 8” butt x 4” top x .188” wall thickness minimum.  Flags by Owner.  See Civil Drawings.

Division 11 Equipment –

  - Equipment:  Building equipment as described below to be provided and installed by Contractor, unless otherwise noted in these specifications.  Contractor to provide all necessary electrical provisions for items listed below. Review other pertinent Divisions of these specifications for model numbers and more detail that may be included.

  - One wall mount eye/face wash, with bowl – installed in the apparatus bay, piped with water. No drain needed at this location. Specify model number proposed.

  - Ice Machine, Ice Container Bin, and Components installed in the apparatus bay. Manitowoc Model Number IR-0906A Indigo Series Ice Maker.  AR-20000 Arctic Pure Primary Water Filter Assembly.  Ice Container Bin C-7305, and Adapter K-00023 for mounting.

  - One three-compartment sink with deep compartments and hose/sprayer nozzle to be installed in the apparatus bay. Saturn Equipment Sink MKS3-2424-SD-24 (24”x24” bowls, and two sides of drain board).  Faucet assembly T&C Brass B-0133-12CRBCF1.

  - Washer and dryer installation, electrical, plumbing, and venting for the washer and dryer furnished by Contractor. The actual washer and dryer to be Owner’s responsibility purchased through an allowance included in the contract.

  - At least two natural gas tank-less water heaters, with a maximum input rating of 199,000 BTU and a minimum input rating of 15,200 BTU. Specify model. Ensure proper location to best support the building’s hot water needs, with install on the rear exterior of the building.

  - Kitchen appliances and accessories in the kitchen will be the responsibility of the Owner unless otherwise listed. Some of the items may be purchased through the allowances included in the contract, and per the discretion of the Owner. Provide for an automatic gas shut-off tied to the fire suppression system and manual gas shut-off for the kitchen range. See Division 13. Provide for adequate amounts of outlets, GFI outlets, and dedicated outlets, needed for kitchen appliances.

  - Kitchen Heavy Duty Range:  Owner will provide for a heavy-duty range unit.  Contractor shall provide for the electrical, gas, and hood provisions, and installation.  Range will be Saturn model number SHDR-60-4-36G.  See other pertinent Divisions for more detail.

  - Kitchen Merchandiser Refrigerator:  Owner will provide for a merchandise refrigerator unit.  Contractor shall provide for electrical, and any water provisions to accommodate the unit. Unit will be Saturn – FB Series, model number FBGM48.

  - All furniture and entertainment items, all beds, bedside tables, end tables, all exercise equipment, gas range, refrigerator, kitchen appliances, kitchen accessories, furniture, etc. will be the responsibility of the Owner.  Some items may be purchased through the allowance included in the contract, and per the discretion of the Owner.

  - Apparatus Exhaust System: The Owner will be responsible for the removal and installation of the existing Plymovent Apparatus Bay Truck Exhaust System, paid through an allowance included in the contract.  The installation will be coordinated by both the Contractor and the Owner during the construction period.  All electrical needs for the system will be provided for by the Contractor before installation.

  - General Contractor will bring to the attention of the Owner, any equipment found to be in question, or seeming to be omitted from these specifications, prior to submitting a building cost.

Division 12 Furnishings:  All furnishings by Owner – (See Division 11 for further details)

Division 13 Special Construction – (Pre-Engineered Building and Fire Suppression System)

  - General:  Work under this section includes furnishing all materials, equipment and labor to complete the erection of the pre-engineered steel building. The materials furnished will include the structural framing, roofing, siding, anchor bolts, fasteners, caulking and any other component parts of the building as needed and specified.  Provide to the Owner the pre-engineered building manufacturer proposed to be used.

  - Seismic and Wind Coefficients: All wind and seismic coefficients will meet the current code requirements and approval of the Town of Summerville Building Official, for the restrictions, requirements, and needs of an Essential Building. This location is a Risk Category IV.

  - Building Size:  The building overall footprint is 8,000sf, with a front eave height of 19’-2”, and a rear eave height of 17’-8¼”.  Refer to provided Drawings for other measurements.

  - Roof:  The roof of the building will be a 24 ga standing seam, Galvalume trapezoidal metal roof, with architectural roof panels, bronze Kynar finish.  Front entrance gable roof will be 24 ga standing seam, with architectural panels, bronze Kynar finish, and 3 on 12 slope.  The building roof will be single slope ¼ on 12.  Any crimping or oil canning of the metal roofing will not be accepted. Screw down roof will not be accepted.

  - Framing System:  Framing to be rigid frame clear span.

  - Loading:  The loading will meet requirements as specified in the manufacturer recommended installation and will meet the requirements for the wind load and seismic load as required by the Local Building Official, and for Essential Building code requirements. Specify wind and roof loads with proposal submission.

  - Siding:  Refer to Division 4, 5, and 6 for detail.

  - Gutters and Down Spouts:  Provide and install metal building gutters and down spouts on the building.  Gutters shall have a leaf guard protection feature.  Downspouts will go to ground level and splash blocks at the bottom of each downspout (*unless shown with piping per Civil Drawings.)   See Civil Drawings, and elevation drawings for quantity and length of down spouts.

  - Sky Lights and Roof Vents:  Non-Applicable

  - Bay Spacing:  Bay spacing will be as shown in the furnished drawings. The bracing will be portal frame per furnished drawings and use rod bracing for the roof bracing.

  - Roof Penetrations:  The roofing contractor shall be the only one making penetrations in the roof deck. Any roof penetrations for plumbing vents, hood exhaust system vents, hot water vents etc. will be cut by the roofing contractor.  Once the vents are installed, it will be the responsibility of the roofing contractor to seal around all penetrations.

  - Fire Alarm System and Smoke Detection:  Contractor will submit to the Owner a listing of all system provisions, and include all costs for this complete building system with their proposal.  Systems will meet all current national, state and local codes. The system and components’ design shall be submitted to the Town of Summerville Fire Marshal for final approval.

  - Fire Sprinkler System:  Contractor to include all costs for a total system design, drawings and necessary provisions for a full working system to support this building.  System to be ultimately designed and stamped by a fully licensed subcontractor for this line of work. Systems will meet all current national, state and local codes.  System design and stamped plans will be submitted to the Town of Summerville Fire Marshal prior to being accepted.

  - Fire Suppression – Stove Hood:  Provide and install a hood with fire suppression over the 4-burner range/with additional 36” griddle.  Include all supply and exhaust fans, and curbs. Ensure large enough to cover entire surface, and vent out the roof of the building.  All items shall meet required codes and be approved by The Town of Summerville Fire Marshal. Actual Range and Oven provided by Owner – Saturn Heavy Duty Range SHDR-60-4-36G.

Division 14 Conveying Systems – Not Applicable

Division 15 Mechanical –

  - HVAC Heating and Air Provisions:  Provide and install a complete heating and cooling system for all interior areas, using split systems with gas furnaces.  All rooms will be heated and cooled except the apparatus bay. The units will be able to cool the building to 68 degrees in the summer and warm the building to 74 in the winter consistently throughout. Unit duct work will be all metal duct insulated with 3” thick insulation with all joints and seams sealed with mastic. Other than the flex duct, no other duct shall be longer than 8’ in length and used to tie into 2’X 2’ diffusers and returns. Each diffuser will have the capability of being manually adjusted to balance the system. The heating system for the building will be natural gas.  Provide all details, such as, but not limited to, tonnage, quantity of units, type of units, SEER, etc. to the Owner with the proposal submission.

  - Apparatus Bay Heaters:  Provide and install two (2) 50,000 BTU natural gas heaters hung from the building structure, installed on opposite ends of the bay.  The installation of the heaters will not interfere with the installation or operations of the Apparatus Bay Overhead doors or the vehicle exhaust system. The units and their insulation will meet all requirements of the Mechanical Code as well as the National Gas Code and others as adopted by the State of South Carolina.

  - Exhaust Fans:  Provide and install industrial sized exhaust fans for all bathrooms with showers, tied to ductwork and vented to the outside eaves.  Do not vent through the front of the building except for dryer vent, below 18” AFF.

  - Commercial Stove and Hood:  Owner will provide for a heavy-duty range unit.  Contractor shall provide for the electrical, gas, and hood provisions.  Range will be Saturn model number SHDR-60-4-36G.  See Division 13 – Fire Suppression, for hood detail.

  - Gas Lines:  See Division 1 for details.

Furnish and install new gas lines to the apparatus bay heaters, gas range, Rinnai tank-less hot water heaters, gas units for the building heat, the emergency power generator, and an exterior shut off quick connect for a future gas grill (see drawing for location).  Contractor will coordinate with SCE&G for the size of the main gas piping and meter to be installed to the amount of gas load.

  - Dryer Vent:  Provide venting system for standard dryer.  Vent out the front of the building, below 18” AFF, with attractive aluminum painted cap and seal in brick.

  - Plumbing:  Scope of work shall include all labor and material to install a complete sanitary waste and vent system and a water supply system.  Furnish and install water and sewer from the building to the city’s sewer and water systems.  All new water and sewer lines should be provided for, and no use of existing piping will be allowed.  All new lines should be adequate size and function for this building. See Civil Drawings. All work would consist of supplying, installing, setting, and trim out of all fixtures.  All plumbing will be in accordance to and tested as required by the current International Plumbing Code.

Project should include, but may not be limited to: (Ensure ADA applicability)

- Reuse, Connect to existing meter

- back check valves

- plumbing lines for all water and sewer connections

- water faucets

- sinks

- toilets

- urinals

- floor drains and trench

- showers

- hot water heaters

- water cooler

- mop sink

- valves

- exterior hose bibs (at least 3)

- future washer in apparatus bay

- ice machine in apparatus bay

- three-compartment sink in apparatus bay

- eye/face wash in app bay (water only)

- standard washing machine connection in utility and in apparatus bay

- bar sink in training room, 15”

- Clean outs

- Any other water or sewer connections that may be needed

- Backflows

- No garbage disposal

- Connection of existing oil/water separator

Plumbing lines will be securely installed in hangers and brackets every four feet to prevent noise from pressure differentials. All plumbing fixtures, appliances, drains, fixture supports, and appurtenances will be new, without blemishes or defects, and by Kohler or approved equal. Floor drains and cleanouts by Zurn or approved equal.  Any substitutions shall be equal in quality and size – specify any substitutions.

Proposal submission will provide a description of all plumbing fixtures. When applicable, pictures of proposed plumbing fixtures will be provided.

  - Shower Enclosure Stalls:  Shower stalls shall be single unit fiberglass enclosures with ADA accessibility where required, including valves and hand-held spray wash capability.  The units will be installed in compliance with manufactured installation requirements and the International Building Code.  There should be at least two (2) ADA shower units, with a total amount of four (4). See drawings for size.

Division 16 Electrical –

  - Electrical Provisions:  Provide complete provisions for all necessary electrical functions for a fully operational Essential Building.  Provide to the Owner a set of preliminary drawings showing all necessary disconnects and breaker boxes for an electrical service for the building. Provide for all electrical components starting from SCE&G to the underground service to the building.  Include, but not limited to:

- Disconnects, Services, Panels, Meter Bases

- Automatic Switching Gear

- Emergency Generator installation with electrical requirements

- Lighting circuits

- Recessed Canned Lighting – with dimmable feature, in all sleeping quarters

- Energy Efficient 2’X4’ lay-in fluorescent lights in residential section of building

- HVAC and exhaust needs

- High-Bay fluorescent lights in the apparatus bay

- See exterior lighting section for lighting and general electrical detail

- Vanity Lighting in all bathrooms

- All Egress and Emergency Lights, including in bathrooms and sleeping areas

- All electrical requirements for equipment and appliances

- 220v receptacles where needed

- Provide for ceiling receptacle(s) in training room – two (2)

- GFI receptacles

- 110v receptacles interior and exterior, Quads and Duplex

- No less than 58 single receptacles total, no less than 15 quads, 5 exterior

- Dedicated Circuits where needed

- Circuits to Disconnects

- Electrical provisions for under the hood

- Install 1 run of 2” electrical conduit no less than 100’ – in two different exterior trench locations, for a total of 200’ length of underground conduit.

- Install underground electrical conduit and service wire to SCE&G service point – see Civil Drawings for detail.

- Data Conduit Drops for Low Voltage Wiring; no less than 24 drops, up to 2’ above ceiling, and with pull string

- Building Door Chime/Doorbell

- All other components needed to complete the project and meet all national, state and local codes. After final drawings have been reviewed and approved by the Owner, they will be submitted to the Building Official for final approval.

Sleeping Quarters:  Provide for canned lighting with dimmers at all sleeping areas, between bunks, and walking areas.  All lighting in sleeping areas should be dimmable.  Reading lights should be separate to allow for individual reading spots. Data drops and outlets should be at each bed area for individual use as well. Provide for 2-head emergency lighting in sleeping quarters.

  - Equipment Hook-Up and Electrical Provisions:  Check Division 11 for information on equipment and appliances that will need electrical provisions and hook-up.

  - Generator:  The Owner shall be responsible for the removal and storage of the existing generator for later reuse.  The Owner will deliver the generator back on site when ready for install.  Contractor shall provide and install all electrical provisions for the generator, support, anchor and secure the generator in place, and provide concrete pad for base.

  - Doorbell:  Provide and install a push button at the front and rear passage doors which will activate chimes/buzzers (doorbell) in the living quarters, the men’s bunkroom, and the apparatus bay. The chimes/buzzers (doorbell) should be loud enough to be heard throughout the building.

  - Lighting Timers:  Provide for lights that remain on when in use, and when no movement is being detected; specifically, in the main living area.

  - Overhead Doors:  Provide for the electrical provisions for all overhead doors. Refer to Division 8 for door detail.

  - Power Poles:  All power poles will remain as is, and new underground electrical conduit will be run to the building. Contractor shall provide for two runs of 2” conduit up to 80lf each.  Contractor will coordinate this with SCE&G.

  - Exterior Electrical for Building:  Provide and install all wiring and switches for exterior lighting.  Provide and install the following electrical features on the exterior of the building:

- No less than six (6) 400 watt wall pack lighting fixtures mounted to the exterior walls.  Provide time clock feature(s) to work with all six wall packs.

- Two (2) decorative moisture resistant exterior light fixtures on the entrance columns.  (Include allowance of $400 per each decorative fixture)

- One – two (1-2) moisture resistant light fixture(s) underneath the front entrance

- One (1) moisture resistant light at each of the other three single passage doors.

- At least five (5) outside GFI quad outlets.

  - Telephone:  The telephone system will be the responsibility of the Owner by allowance included in the contract. Provide for a telephone board, at least 4’ x 5’, painted.

  - Computer:  The computer system will be the responsibility of the Owner by allowance in the contract.

  - Fire Alarm and Smoke Detection System:  See Division 13.

  - Low Voltage Cabling and Wiring:  The cabling and low voltage wiring will be the responsibility of the Owner and paid for by allowance in the contract. The electrician will supply and install conduit and data drops.  Provide up to at least twenty-four (24) conduit drops though-out the building at locations designated by the Owner. Provide pull strings, and conduits at least 2’ above ceiling height.

  - Communication Pole:  The communication pole on site will be relocated by the Owner through an allowance included in this contract. Provide and install one run of 4” schedule 40 conduit, under slab from the pole to inside the building to the storage room, and up through the ceiling (see location on drawings). Contractor will coordinate this with the Town of Summerville IT Department.

END OF THE NON-BINDING PROPOSED SPECIFICATIONS TO AID CONTRACTORS

PROPOSAL FORM FOLLOWS THIS PAGE – MUST USE FOR PROPOSAL SUBMISSION

PROPOSAL FORM			    	     SUMMERVILLE FIRE RESCUE – STATION 3

Contractors shall submit pricing with this format

CONTRACTOR COMPANY NAME: 	_________________________________________   DATE:  ________________

FULL STREET ADDRESS: 	________________________________________________________

CONTACT INFORMATION:	_________________________________________

TELEPHONE NUMBERS:  	_________________________________________

EMAIL ADDRESS:	_________________________________________

SIGNATURE OF SUBMITTOR:  	_________________________________________

Printed Name of Submitter:	_________________________________________

GENERAL CONTRACTOR’S

LICENSE NUMBER:		_________________________________________

ADDENDUM ACKNOWLEDGEMENT:  	_____________________________________________

Contractor acknowledges the receipt of	List all addenda received and acknowledged

and has incorporated the said

Addenda into this proposal.	_____________________________________________

LUMP SUM BASE PRICE:  	_________________________________________

Including payment & performance bond cost, and including all allowances (Owner’s and others)

PAYMENT AND PERFORMANCE BOND COST:  	_________________________________________

Included in base price above

OWNER ALLOWANCES:	_________________________________________

It is recognized that the above amount of allowances, per specifications, is included in base price

MUCK AND FILL/Per CY:	_________________________________________

STEEL CLAD BOLLARD, CONCRETE FILLED, 6”, PAINTED	________________________________

WITH THIS PROPOSAL INCLUDE, BUT NOT LIMIT TO:

- FULL LISTING OF SUBCONTRACTORS, VENDORS AND SUPPLIERS THAT WILL BE USED ON THIS PROJECT

- BID BOND

- COST BREAKDOWN – BY DIVISION; OR FURTHER BREAKDOWN IS PREFERRED

- PROPOSAL FORM

- ANY ALTERNATIVE OPTIONS AND OR SUBSTITUTIONS

- ALL DETAIL TO HELP ASSIST THE OWNER IN FULLY UNDERSTANDING THE PROPOSAL

- DETAIL ON ALL AREAS AS REQUESTED IN THESE SPECIFICATIONS

- CERTIFICATE OF INSURANCE