PAGE  

Hearne ISD
REQUEST FOR BIDS
Hearne ISD will be receiving bids for the following:

78 Passenger 2017 or newer Year Model School Bus
Specifications may be obtained at the Hearne Administration Building
900 Wheelock, Hearne, TX 77859 or by calling 979 279-3200.
Bids will be received until 4:00 p.m. on Monday; October 5, 2018 at the:

Hearne Administration Building
900 Wheelock
Hearne, TX 77859
Bids received either by mail or delivered after 4:00 p.m. will not be opened and will be returned to the respective bidder.

Hearne ISD assumes no responsibility or liability for the failure of potential bids not received by 4:00 p.m. on the due date. 

Bids will be opened, commencing at 5:00 p.m. at the administration building at 900 Wheelock, Hearne Texas
Should your firm be interested in submitting a bid, the outer envelope must contain the following information:

  78 Passenger School Bus



Date of Bid

Company Name




Time of Bid

Company Address 




Business License Number

Failure to include the requisite information on the outside of the envelope will potentially make the bid subject to rejection. 
Hearne ISD reserves the right to accept and/or reject any or all bids.

Hearne ISD  does not discriminate based on race, color or national origin in federal or state sponsored programs, pursuant to Title VI of the Civil Rights Act of 1964 (42 U.S.C. 2000d).

NOTICE TO BID

Hearne ISD will be accepting bids for a latest current production model, 78-Passenger Conventional School Bus for the Hearne Board of Education with the following specifications:













    Please Check One













  YES

  NO

1. Air Compressor-13.2 CFM minimum







_______

_______

2. Air Dryer-AD-S with heater








_______

_______

3. Air Lines-Quick connect air fittings in lieu of compression type




_______

_______

4. Air Tanks- FMVSS required cu. in. capacity w/ shraeder valves 

(Bendix DV-2 or equal) installed with air tanks






_______

_______


5. All Wheel Antilock-self diagnostic lights






_______

_______

6. Altenator-175 amp









_______

_______

7. Axles – Front:  10,000 pound,    Rear:  19,800  pound

8. Axle, Rear Ratio-to attain 70 mph







________
_______


9. Batteries- 3 Batteries/minimum 1950 Total Amp

10. Brakes – Outboard Drum type w/auto slacks – Front: 16.5x 5 thick, Rear: 

16.5 X 7 thick/Rockwell Q-Plus or equal.






_______

_______

11. Bumper – front: ¼ inch thick, 12 inches wide, bolted to frame-Rear: 12” bolted to frame

_______

_______

12. Dash Mounted-cruise control: high idle switch






_______

_______

13. Color – the top of the buses shall be painted white





_______

_______

14. Crossing Control Device –crossing gate, electric operated with magnet: 

stoparm, with strobe lights, include wind guard: per State of Tennessee School Bus Regulations
_______

_______

15. Exhaust System – Aluminized steel-exit left side.

 Must meet State of Tennessee School Bus Regulations





_______

_______

16. Frame – Must meet State of Tennessee School Bus Regulations: 50,000 lbs.

17. Fuel Tank – 65 gallon, right hand mounted on frame rails with spring loaded door and access plate

meeting FMVSS and State of Tennessee Standards





_______

_______

18. Fuel Filter/Separator: Separator 1: Spin on type at the engine Separator 2: Racer w/see through bowl
_______

_______

19. Heating System –90,000 BTU front: 50,000 bus step wall heater: 84,500 BTU rear mount heater: 

Ball type water valve cut off: Dual heater cut off valves with circulating pump


_______

_______

20. Block Heater- 1250 Watt minimum







_______

_______

21. Starting Aid-Grid Heater








_______

_______

22. Horn – Rear MTD back up alarm







_______

_______

23. Instruments and Instrument panel – Speedometer: Coolant Temp.: Transmission Oil Temperature: 

Amp Meter: Voltmeter: Fuel Gauge: Oil Pressure: Dual Air Pressure Gauge: Tachometer; Hour Meter: 

Trip Odometer: Air Filter Restriction Indicator






_______

_______

24. Shocks – Front and rear: Front Springs rated 10,000 lbs: Rear Springs rated 19,800 lbs.

and radius leaf for rear adjustment– Must meet State of Tennessee Regulations


_______

_______

25. Steering Wheel-Tilt Steering Wheel

26. Tires and Rims – Tubeless radial 10R22.5 14 ply Goodyear G-149 or equal: Tubeless radial 10R22.5    

14 ply Goodyear G-124 M/S or equal







_______

_______

27. Tow Hooks- front and rear 








_______

_______

28. Engine – DT466E, 225 Horsepower minimum, 6 cylinder must be of wet sleeve design

_______

_______

29. Engine Fan: Horton Drive Master







_______

_______

30. Engine Rating- Minimum 620 torque at 1400 rpm





_______

_______

31. Engine Emission- comply with Federal EPA standards for 2005




_______

_______

32. Engine Warranty-5 yr unlimited mileage 100% parts & labor




_______

_______

33. Transmission – Allison 3000 Series or equal: with external cooler




_______

_______

34. Wheels-Disc hub piloted, 22.5x8.25 painted black with 254 inch wheel base



_______

_______

35. Wiring-must be colored and number coded: protected with ATO Blade type fuses: 

spare acc. wiring: power plug-in for telephone






_______

_______

36. Heater Hoses-covered with metal







_______

_______

37. Defroster Channel-run the full length of windshield





_______

_______

38. Warranty – Shall be manufacturers standard warranty





_______

_______


Any warranty extension greater than standard, Warranty shall be made available to purchaser

_______

_______
39. Certification – Chassis manufacturer will, upon request, certify to the purchaser that their 

product meets minimum standards on items not covered by certification issued under requirements 

of National Traffic and Motor Vehicle Safety Act.





_______

_______

  Please Check One













  YES

   NO BODY


1.   Access Panels-3 located below windshield: 1 located under driver’s window



_______

_______ 2.   Driver’s Storage-glovebox and map pocket






_______

_______

3.   Dash-right side, driver’s cockpit with wraparound console





_______

_______

4.   Entrance Door-air operated: outward opening 36” wide





_______

_______

5.   Emergency Door- 36”x51” upper and lower glass






_______

_______

6.   Flooring-gray rubber: 5/8 “ thick exterior grace that is secured with screws: aluminum aisle trim: 

      molded rubber over wheel housings







_______

_______

7.   Headroom-78” minimum








_______

_______

8.   Defroster Fans-2 dual 6” mounted fans







_______

_______

9.   Insulation 1 ½” thick fiberglass in roof, front and rear: 1 ½” thick mineral wool blanksted

_______

_______

10.  Insulation/Sound- all body sections to have perforated ceiling




_______

_______

11.  Identification-8” Black letters on both sides with 8” numbers on sides, front and rear


_______

_______

12.  Mirrors-Interior 6”x30” clear with rubber edge: Heated Avle style




_______

_______

13.  Mud Flaps-Front and Rear








_______

_______

14. Emergency Exits-4 push out windows with buzzers: 2 roof hatches with buzzers 

      (Specialty model 9245 or equal)








_______

_______

15.  Acc two way wiring - wiring harness, antenna, coax, key locking bracket



_______

_______

16.  Reflective/Safety-3-M scotch lite “SCHOOL BUS” frt & rr cape: 3-M scotch lite 2” yellow tape  

       around exits, rear and sides: side reflectors attached with screws




_______

_______

17.  Safety-first aid, body fluid, fire ext., safety triangle





_______

_______

18.  Driver Seat-Air Ride: National High Back with right arm rest or equal: Gray fabric and high back 

       with tilt










_______

_______

19.  Passenger Seats-39” DOT heavy duty Gray upholstery with 26” minimum seat space…30’ body
_______

_______

20.  Side Panels-Interior aluminized steal sidewalls






_______

_______

21.  Solenoid-disconnect all body circuits







_______

_______

22.  Stepwell-slant three step entrance







_______

_______

23.  Vents-2 driver’s fresh air and static roof vent






_______

_______

24.  Drivers Window-double sliding







_______

_______

25.  Windows-aluminum frame 12” slide and rear dark tinted





_______

_______

26.  Windshield-three piece flat tinted and shaded safety with kick out: 2 grip handles above windshield
_______

_______

27.  Wipers-single switch with variable speeds






_______

_______

28.  Student Safety-Child reminder system







_______

_______

HOOD

1.  Fiberglass










_______

_______

2.  Forward tilting with quick release pins for easy removal of the entire hood



_______

_______

3.  There shall be a handhold molded into the hood for ease of tilting




_______

_______

INSTRUMENTS GAUGES & SWITCHES

Body Switches-located left and in view of the seated driver





_______

_______

Chassis Instruments and Gauges-located within easy view of the seated driver



_______

_______

The chassis instrument panel shall be equipped with:

1. Speedometer










_______

_______

2. Odometer










_______

_______

3. Voltmeter










_______

_______

4. Oil pressure gauge









_______

_______

5. Water temperature gauge








_______

_______

6. Water temperature Light/Buzzer







_______

_______

7. Fuel gauge










_______

_______

8. Upper beam headlight indicator







_______

_______

9. Turn signal indicator lights








_______

_______

10. Air gauge (Chassis with straight air brakes)






_______

_______

11. Red light to indicate low air pressure







_______

_______

12. FMVSS 105 indicator lights








_______

_______

13. Tachometer (manufacturer’s standard, located in the instrument panel)



_______

_______

14. Hour meter (located in the instrument panel and switched to operate only when engine is running
_______

_______

   Please Check One













  YES

  NO

INTERIOR

1.  Free of all unnecessary projections likely to cause injury





_______

_______ 2.  All interior panels shall have lapped edges






_______

_______

3.  Interior side panels to be mar-proof aluminized steel from window bottom to the seat- mounting rail
_______

_______  

4.  The interior body height from the floor to the center of the roof shall be a minimum of 73”

_______

_______

LAMPS & SIGNALS

1.  All lamps shall conform to all FMVSS and state laws in effect at time of manufacture


_______

_______

2.  Lights- Clearance and cluster








_______

_______

3.  Cluster- (3) amber front w/protective shields, (3) red rear





_______

_______

4.  Marker- (2) front corners, (2) real corners, and (2) side roof on bodies longer than 24”


_______

_______

5.  Stop/Tail (1) right rear, (1) left rear – 7” minimum (1) right rear, (1) left rear – 4” minimum

_______

_______

6.  Back-up Lamps (2) 4” with white lens







_______

_______

7.  Stepwell - (1) operating with dome light switch






_______

_______

8.  Dome - To be located over seat area, minimum (4)






_______

_______

9.  Directional- Side mounted: Front and Rear amber directionals: Side mounted directional

_______

_______

10. Strobe Light – Roof mounted with guard with dash switch





_______

_______


11. “Headlights On”- reminder of when lights have been left on




_______

_______

12. Warning Lights-Hooded








_______

_______

13. Parking Lights-4” amber parking







_______

_______

14. Headlights-Halogen









_______

_______

QUOTE FOLLOWING AS OPTIONAL ADDITIONS

VCR Bus watch Camera System-8MM color-2 camera heads with Quad Processor or equal     $___________________________

Air Condintioning-110,000 BTU System Dual Compressors: Skirt Mounted Condensers
$___________________________

Engine – DT466E, 275 Horsepower minimum, 6 cylinder must be of wet sleeve design
$___________________________
Total Price of Single Bus without Options




$___________________________
(The Hearne ISD and/or the Board of Education reserves the right to purchase one or more buses)
Vendors shall state the delivery date as how many days _________ from the date of issuance of a purchase order. 
Vendors shall label the sealed bid “78 Passenger School Bus Bid” on the outside of the envelope along with the Company Name.

Bids will be received until 4:00 p.m., Friday, October 5, 2018 at the:

Hearne ISD
900 Wheelock

Hearne TX 77859

Bids will be opened, commencing at 5:00 p.m. at the Hearne Administration Building, 900 Wheelock, Hearne Texas.
The Hearne ISD reserves the right to accept and/or reject any and all bids.  Bids shall remain valid for 90 days from the date of the bid opening. 
Bids will be referred to the Hearne ISD Board of Education for consideration and then placed on the agenda for the October regular meeting of the Hearne ISD Board of Education. 

Business Name ______________________________________               Business License________________________________

Address
_____________________________________________________________________________________________
Phone Number _______________________________________
Fax Number ___________________________________

E-mail _____________________________________________

Hearne ISD does not discriminate based on race, color or national origin in federal or state sponsored programs, pursuant to Title VI of the Civil Rights Act of 1964 (42 U.S.C. 2000d).
If awarded, the bus or buses shall be delivered to:  







Hearne ISD Bus Garage









1210 D Hackberry







Hearne, TX 77859
Payment will be available the day of delivery of the vehicle, however, Hearne ISD Finance must be notified a minimum of seven (7) days prior to delivery in order to have payment available.  
In case of error or discrepancy in the mathematics of the bid price, the unit prices shall prevail.
OPTIONAL EQUIPMENT. Optional Equipment selection will be at the discretion of the Hearne ISD Board of Education. 
NOTICE TO BID
The bidder acknowledges that all business with Scott County Government through the Scott County Finance Department requires a purchase order number BEFORE the goods or services are sold to Scott County Government.  The vendor assumes the risk of NOT being paid by not adhering to the procurement procedures of Scott County Government in securing a purchase order number before a transaction with Scott County Government.
 _____________ Vendor Initials
The proposed bidder must use the bid form as developed by the Scott County Finance Department.  The bidder shall not modify the bid form in any form or fashion as to alter the contents of the form. Modification of the original bid form or submission of an alternate bid form shall immediately render the proposed bid null and void and the Finance Committee will not consider the proposed bid any further including if the bid has already been opened.   










_____________ Vendor Initials
NOTICE TO BID

      Please attach this form to your bid proposal, 78 Passenger Bus 10/05/2018.  

It is the policy of Scott County Government not to discriminate on the bases of race, color, national origin, age, sex or disability in its hiring and employment practices, or in admission to, access to, or operation of its programs, services and activities. With regard to all aspects of this contract, Contractor certifies and warrants it will comply with this policy. 

For Title VI and IX compliance, we ask for voluntary disclosure of the following information:

GENDER:
___ Male

___ Female

RACE:

___ Caucasian

___ African American



___ Other (please specify)

