Molino Community Center Hardwood Floor Replacement

6450 Highway 95A, Molino Florida, 32577

General Information:

The Room size is approximately 657 square feet.

All existing flooring will be removed and replaced with new 2 ½” by ¾” New Heart Pine flooring.

The work will be done during regular work hours.

The shoe molding will be removed prior to sanding & refinishing by Facilities Management.

Scope of Work:

- SUPPLY AND INSTALL APPROX. 657 SF OF FLOORING.

- INSTALL DUST BARRIERS AT ALL CRITICAL AREAS.

- SAND AND FINISH APPROX. 657 SF OF FLOORING.

- SAND ENTIRE FLOOR TO BARE WOOD.

- HAND SAND/SCRAPE ALL WALL LINES, DOOR JAMBS & CORNERS.

- FILL ALL HOLES, GAPS, BUTT-JOINTS WITH MATCHING FILLER.

- ADDITIONAL SANDING TO PREP WOOD FOR 1 COAT SEALER & 2 COATS MATTE POLYURETHANE

Walk Through: A site visit can be arranged by using the below Point of Contact.

Start date: This work will be awarded as soon as possible.

Point of Contact:   J. David Bardin

Interim Division Manager (Maintenance)

Escambia County Facilities Management

Phone: (850) 595-4631

Email: jdbardin@myescambia.com