As part of property acquisition All Appropriate Inquiry due diligence, Escambia County (the County) is seeking a Scope of Work (SOW) and associated pricing schedule for environmental assessment activities on the western half of the vacant parcel identified as street address 401 W. Government St., Pensacola Fl 32502, Escambia County Property Appraiser Reference Number 000S00970010033, Account Number 152004000 (please see aerial).

A Phase I Environmental Assessment was conducted February 12, 2019.  The Phase I identified Recognized Environmental Conditions (REC) and recommended Phase II investigation of the soil and groundwater on site and of the vapor encroachment.  A copy of the Phase I recommendation is included for your reference.

The County is seeking to assess an approximate six (6) acre section of fill material that has been imported from an unknown source onto the western half of the subject parcel.  This material appears to have been engineered into a mounded square approximately 530’ by 530’ to an approximate depth of two feet. The County is asking for your SOW and pricing schedule to include the following:

- Eleven (11) soil sample locations as per located on aerial map provided. Each location sampled at depths of one foot (1’), three feet (3’) and five feet (5’).  (The soil sampling should include the imported fill soils and the existing underlying parent soil beneath the imported soils, to a depth representative of existing conditions.  The underlying parent soils are classified as Arent/Corolla-Urban Land Complex as per Escambia Soil Survey).

- Please also provide SOW and pricing schedule for installation and sampling of five (5) temporary groundwater sampling wells to be installed as per applicable industry specifications and standards in a location to be determined.

Groundwater and soil samples should be tested for standard petroleum and hazardous material based- pollutants to include, but not limited to, petroleum-based contaminants, volatile and semi-volatile organics (VOA, VOH, VOC) pesticides, herbicides, PCBs, Chlorinated Solvents (TCE, PCE) and RCRA Metals.

In addition, a geotechnical investigation is needed to report the following:

- Soil bearing capacity with soil borings

- Recommendations and requirements for the utilization of shallow spread foundations

- Determination of what will be encountered under the site surface and recommendations

Vendor Registry Version

As part of property acquisition due diligence, Escambia County (the County) is seeking a Scope of Work (SOW) and associated pricing schedule for environmental assessment activities on the vacant parcel identified by street address 401 W. Government St., Pensacola Fl 32502, Escambia County Property Appraiser Reference Number 000S00970010033, Account Number 152004000, former location of the Government St. Sewer Treatment Plant (please see aerial).

A Phase I Environmental Assessment was conducted February 12, 2019.  The Phase I identified Recognized Environmental Conditions (REC) and recommended Phase II investigation of the soil and groundwater on site and for vapor encroachment.  A copy of the Phase I recommendation is included for your reference.

The County is seeking to assess the entire approximately 18 acre vacant parcel to include the approximate six (6) acre section of fill material that has been imported from an unknown source onto the western half of the subject parcel (please see attached aerial).  This material appears to have been engineered into a mounded square approximately 530’ by 530’ to an approximate depth of two feet. The County is asking for your SOW and pricing schedule to include the following:

         Eleven (11) soil sample on approximate locations as per attached aerial map provided. Each location sampled at depths of one foot (1’), three feet (3’) and five feet (5’).  (The soil sampling should include the imported fill soils and the existing underlying parent soil beneath the imported soils, to a depth representative of existing conditions.  The underlying parent soils are classified as Arent/Corolla-Urban Land Complex as per Escambia Soil Survey).

         Please also provide SOW and pricing schedule for installation and sampling of five (5) temporary groundwater sampling wells to be installed as per applicable industry specifications and standards in approximate locations depicted on the attached aerial .

Groundwater and soil samples should be tested for standard petroleum and hazardous material based- pollutants to include, but not limited to, petroleum-based contaminants, volatile and semi-volatile organics (e.g. VOA, VOH, VOC), pesticides, herbicides, PCBs, Chlorinated Solvents (e.g. TCE, PCE) and RCRA Metals.

In addition, a geotechnical investigation is needed to report the following:

A.      Soil bearing capacity with soil borings

B.      Recommendations and requirements for the utilization of shallow spread foundations

C.      Determination of what will be encountered under the site surface and recommendations