Air Conditioner Service and Maintenance

Provide inspection, testing and preventative maintenance of Remote Radio Site Air Conditioning/Heater Units.  Servicer will provide all labor and provide priority service.  Includes replacement of failed or worn components, including: compressors, fan motors, switches, capacitors, including (running and starting capacitors), thermostats (digital and non-digital), blower motors and controls, circuit boards and all other ancillary components to include wiring. This agreement does NOT include evaporator/condenser/interior coils or unit housings.

Quarterly Preventative Maintenance Inspections (PMI) will include at least the minimum standards listed below:

- Check air handler filters, clean or replace as needed.

- Check and add adequate refrigerant charge.

- Check and clean condenser and evaporator.

- Lubricate fan motors and other components, such as bearings as required.

- Check all electrical connections, tighten or replace connectors if necessary.

- Check fan motor and blade fasteners and tighten if necessary.

- Check voltage under load.

- Check all drains for blockage, and clear/clean as required.

- Check thermostats, clean and adjust if necessary.

- Overall visual inspection and check for vibration and noise.

- Document any recommendations of any needed repairs to systems.

All other services provided by Servicer will require prior approval at established and approved quoted time and materials.  Servicer is available 24x7x365 and will provide a full on-call schedule with contact names and phone numbers, as well as a backup on-call person in the event the primary responder cannot arrive within predetermined response times.