“Scope of Work”

Preventive Maintenance on 14 hydronic boilers

Preventive maintenance includes:

- Remove and clean burners.

- Remove and clean pilot assembly and flame rectifier.

- Clean heat exchanger.

- Re-install boiler components.

- Start-up boiler.

- Check and adjust manifold gas pressure.

- Check limit and safety controls.  Adjust for safe and proper operation, according to the boiler

manufacturer’s specifications.

- Check make-up water system for proper operation.

- Check and repair all indicator lights to insure they are fully operational.

- Check/clean blowers/fans if applicable.

- Conduct combustion test at full fire.  Adjust for proper operation and efficiency.

- Check the operation of the safety pressure relief valve.

- Remove all maintenance debris and tools from site.  Clean work area.

Ordin-Orvis	201 South Palafox Street				Proposal amount  $__________

(5)  Boilers	Make: RBI	Model # DB1950

MC Blanchard	   190 Governmental Center				Proposal amount  $__________

(3)  Boilers	Make: Raypak	     Model # H9-2342B

Sheriff Administration Office	1700 West Leonard Street		Proposal amount  $__________

(2) Boilers	Make: Raypak	     Model # H92072

Road Prison Barracks	 601 Highway 297A 				Proposal amount  $__________

(1) Boiler	Make: Raypak	     Model # 1125C

County Probation      2251 North Palafox Street				Proposal amount  $__________

(1) Boiler	Make: Raypak	     Model # H3-1336

Animal Shelter	   200 West Fairfield Drive				Proposal amount  $__________

(1) Boiler	Make: Weil-McClain	   Model # PFG-65PN

Juvenile Justice Center	   1800 Saint Marys Avenue			Proposal amount  $__________

(1) Boiler	Make: Raypak	     Model # H3-1631

Note:  Contractor personnel working in the M C Blanchard Building will require a background check, before being allowed to work.  Background Check forms must be filled out at least 3 weeks in advance of scheduled work.